GigaDevice Semiconductor Inc.

GD32F470xx

Arm® Cortex®-M4 32-bit MCU

Datasheet

Revision 2.1 (Jan. 2025)

GD32F470xx Datasheet

Table of Contents

Table of Contents ........................................................................................................... 1 List of Figures ................................................................................................................ 4 List of Tables .................................................................................................................. 5

1.   General description ................................................................................................. 7

2.   Device overview ....................................................................................................... 8

2.1.	Device information ................................................................................................... 8

2.2.	Block diagram ........................................................................................................ 10

2.3.	Pinouts and pin assignment.................................................................................. 11

2.4.	Memory map ........................................................................................................... 14

2.5.	Clock tree................................................................................................................ 17

2.6.	Pin definitions ........................................................................................................ 18

2.6.1.	GD32F470Ix BGA176 pin definitions.................................................................................... 18

2.6.2.	GD32F470Zx LQFP144 pin definitions................................................................................. 29

2.6.3.	GD32F470Vx BGA100 pin definitions .................................................................................. 38

2.6.4.	GD32F470Vx LQFP100 pin definitions ................................................................................ 45

2.6.5.	GD32F470xx pin alternate functions .................................................................................... 52

3.   Functional description .......................................................................................... 62

3.1.	Arm® Cortex®-M4 core............................................................................................ 62

3.2.	On-chip memory..................................................................................................... 62

3.3.	Clock, reset and supply management .................................................................. 63

3.4.	Boot modes ............................................................................................................ 64

3.5.	Power saving modes.............................................................................................. 64

3.6.	Analog to digital converter (ADC) ......................................................................... 64

3.7.	Digital to analog converter (DAC) ......................................................................... 65

3.8.	DMA......................................................................................................................... 65

3.9.	General-purpose inputs/outputs (GPIOs) ............................................................. 66

3.10.	Timers and PWM generation ................................................................................. 66

3.11.	Real time clock (RTC) and backup registers ........................................................ 67

3.12.	Inter-integrated circuit (I2C) .................................................................................. 68

3.13.	Serial peripheral interface (SPI) ............................................................................ 68

3.14.	Universal synchronous/asynchronous receiver transmitter (USART/UART) .... 68

1

GD32F470xx Datasheet

3.15.	Inter-IC sound (I2S) ................................................................................................ 69

3.16.	Universal serial bus full-speed interface (USBFS)............................................... 69

3.17.	Universal serial bus high-speed interface (USBHS) ............................................ 69

3.18.	Controller area network (CAN) .............................................................................. 70

3.19.	Ethernet (ENET)...................................................................................................... 70

3.20.	External memory controller (EXMC) ..................................................................... 70

3.21.	Secure digital input and output card interface (SDIO)......................................... 71

3.22.	TFT LCD interface (TLI).......................................................................................... 71

3.23.	Image processing accelerator (IPA)...................................................................... 71

3.24.	Digital camera interface (DCI)................................................................................ 72

3.25.	Debug mode ........................................................................................................... 72

3.26.	Package and operation temperature..................................................................... 72

4.   Electrical characteristics....................................................................................... 73

4.1.	Absolute maximum ratings ................................................................................... 73

4.2.	Recommended DC characteristics ....................................................................... 73

4.3.	Power consumption ............................................................................................... 75

4.4.	EMC characteristics ............................................................................................... 81

4.5.	Power supply supervisor characteristics............................................................. 82

4.6.	Electrical sensitivity............................................................................................... 84

4.7.	External clock characteristics ............................................................................... 84

4.8.	Internal clock characteristics ................................................................................ 87

4.9.	PLL characteristics ................................................................................................ 88

4.10.	Memory characteristics ......................................................................................... 90

4.11.	NRST pin characteristics ....................................................................................... 90

4.12.	GPIO characteristics .............................................................................................. 91

4.13.	ADC characteristics ............................................................................................... 94

4.14.	Temperature sensor characteristics ..................................................................... 95

4.15.	DAC characteristics ............................................................................................... 95

4.16.	I2C characteristics ................................................................................................. 96

4.17.	SPI characteristics ................................................................................................. 98

4.18.	I2S characteristics................................................................................................ 100

4.19.	USART characteristics......................................................................................... 102

2

GD32F470xx Datasheet

4.20.	SDIO characteristics ............................................................................................ 102

4.21.	CAN characteristics ............................................................................................. 102

4.22.	USBFS characteristics......................................................................................... 103

4.23.	USBHS characteristics ........................................................................................ 104

4.24.	Ethernet (ENET) characteristics.......................................................................... 104

4.25.	EXMC characteristics........................................................................................... 105

4.26.	TIMER characteristics.......................................................................................... 109

4.27.	DCI characteristics............................................................................................... 109

4.28.	WDGT characteristics ...........................................................................................110

4.29.	Parameter conditions............................................................................................110

5.   Package information............................................................................................ 111

5.1.	BGA176 package outline dimensions..................................................................111

5.2.	LQFP144 package outline dimensions ................................................................113

5.3.	BGA100 package outline dimensions..................................................................115

5.4.	LQFP100 package outline dimensions ................................................................117

5.5.	Thermal characteristics ........................................................................................119

6.   Ordering information ........................................................................................... 121

7.   Revision history ................................................................................................... 122

3

GD32F470xx Datasheet

List of Figures

Figure 2-1. GD32F470xx block diagram .................................................................................................. 10 Figure 2-2. GD32F470Ix BGA176 pinouts.................................................................................................11 Figure 2-3. GD32F470Zx LQFP144 pinouts ............................................................................................. 12 Figure 2-4. GD32F470Vx BGA100 pinouts .............................................................................................. 13 Figure 2-5. GD32F470Vx LQFP100 pinouts............................................................................................. 13 Figure 2-6. GD32F470xx clock tree .......................................................................................................... 17 Figure 4-1. Recommended power supply decoupling capacitors(1)(2) .................................................. 74 Figure 4-2. Typical supply current consumption in Run mode ............................................................ 81 Figure 4-3. Typical supply current consumption in Sleep mode.......................................................... 81 Figure 4-4. Recommended PDR_ON pin circuit ..................................................................................... 83 Figure 4-5. Recommended external NRST pin circuit(1)......................................................................... 91 Figure 4-6. I2C bus timing diagram.......................................................................................................... 97 Figure 4-7. SPI timing diagram - master mode ....................................................................................... 98 Figure 4-8. SPI timing diagram - slave mode.......................................................................................... 99 Figure 4-9. I2S timing diagram - master mode ..................................................................................... 101 Figure 4-10. I2S timing diagram - slave mode ...................................................................................... 101 Figure 4-11. USBFS timings: definition of data signal rise and fall time ........................................... 103 Figure 4-12. Ethernet SMI timing diagram ............................................................................................ 105 Figure 5-1. BGA176 package outline ...................................................................................................... 111 Figure 5-2. BGA176 recommended footprint .........................................................................................112 Figure 5-3. LQFP144 package outline ....................................................................................................113 Figure 5-4. LQFP144 recommended footprint .......................................................................................114 Figure 5-5. BGA100 package outline ......................................................................................................115 Figure 5-6. BGA100 recommended footprint .........................................................................................116 Figure 5-7. LQFP100 package outline ....................................................................................................117 Figure 5-8. LQFP100 recommended footprint .......................................................................................118

4

GD32F470xx Datasheet

List of Tables

Table 2-1. GD32F470xx devices features and peripheral list .................................................................. 8 Table 2-2. GD32F470xx memory map ...................................................................................................... 14 Table 2-3. GD32F470Ix BGA176 pin definitions...................................................................................... 18 Table 2-4. GD32F470Zx LQFP144 pin definitions................................................................................... 29 Table 2-5. GD32F470Vx BGA100 pin definitions .................................................................................... 38 Table 2-6. GD32F470Vx LQFP100 pin definitions................................................................................... 45 Table 2-7. Port A alternate functions summary...................................................................................... 52 Table 2-8. Port B alternate functions summary...................................................................................... 53 Table 2-9. Port C alternate functions summary...................................................................................... 54 Table 2-10. Port D alternate functions summary.................................................................................... 55 Table 2-11. Port E alternate functions summary .................................................................................... 56 Table 2-12. Port F alternate functions summary .................................................................................... 57 Table 2-13. Port G alternate functions summary.................................................................................... 58 Table 2-14. Port H alternate functions summary.................................................................................... 59 Table 2-15. Port I alternate functions summary ..................................................................................... 60 Table 4-1. Absolute maximum ratings(1)(4) ............................................................................................... 73 Table 4-2. DC operating conditions ......................................................................................................... 73 Table 4-3. Clock frequency(1) .................................................................................................................... 74 Table 4-4. Operating conditions at Power up / Power down(1) .............................................................. 74 Table 4-5. Start-up timings of Operating conditions(1)(2)(3) ..................................................................... 74 Table 4-6. Power saving mode wakeup timings characteristics(1)(2)..................................................... 75 Table 4-7. Power consumption characteristics(2)(3)(4)(5)(6)........................................................................ 75 Table 4-8. EMS characteristics(1) .............................................................................................................. 82 Table 4-9. EMI characteristics(1) ............................................................................................................... 82 Table 4-10. Power supply supervisor characteristics............................................................................ 82 Table 4-11. ESD characteristics(1) ............................................................................................................ 84 Table 4-12. Static latch-up characteristics(1) ........................................................................................... 84 Table 4-13. High speed external clock (HXTAL) generated from a crystal/ceramic characteristics. 84 Table 4-14. High speed external clock characteristics (HXTAL in bypass mode) .............................. 85 Table 4-15. Low speed external clock (LXTAL) generated from a crystal/ceramic characteristics .. 86 Table 4-16. Low speed external user clock characteristics (LXTAL in bypass mode)....................... 86 Table 4-17. High speed internal clock (IRC16M) characteristics .......................................................... 87 Table 4-18. High speed internal clock (IRC48M) characteristics .......................................................... 88 Table 4-19. Low speed internal clock (IRC32K) characteristics ........................................................... 88 Table 4-20. PLL characteristics................................................................................................................ 88 Table 4-21. PLLI2S characteristics .......................................................................................................... 89 Table 4-22. PLLSAI characteristics.......................................................................................................... 89 Table 4-23. PLL spread spectrum clock generation (SSCG) characteristics ...................................... 90 Table 4-24. Flash memory characteristics(1) ........................................................................................... 90 Table 4-25. NRST pin characteristics ...................................................................................................... 90

5

GD32F470xx Datasheet Table 4-26. I/O port DC characteristics(1)(3).............................................................................................. 91 Table 4-27. I/O port AC characteristics(1)(2)(4) ........................................................................................... 93 Table 4-28. ADC characteristics............................................................................................................... 94 Table 4-29. ADC RAIN max for fADC = 40 MHz(2) ....................................................................................... 94 Table 4-30. ADC dynamic accuracy at fADC = 40 MHz(1) .......................................................................... 95 Table 4-31. ADC static accuracy at fADC = 40 MHz(1) ............................................................................... 95 Table 4-32. Temperature sensor characteristics(1) ................................................................................. 95 Table 4-33. DAC characteristics............................................................................................................... 95 Table 4-34. I2C characteristics(1)(2) ........................................................................................................... 96 Table 4-35. Standard SPI characteristics(1) ............................................................................................. 98 Table 4-36. I2S characteristics(1)(2) ......................................................................................................... 100 Table 4-37. USART characteristics(1) ..................................................................................................... 102 Table 4-38. SDIO characteristics(1)(2) ...................................................................................................... 102 Table 4-39. USBFS start up time ............................................................................................................ 103 Table 4-40. USBFS DC electrical characteristics ................................................................................. 103 Table 4-41. USBFS full speed-electrical characteristics(1)................................................................... 103 Table 4-42. USBHS clock timing parameters(1) ......................................................................................... 104 Table 4-43. USB-ULPI Dynammic characteristics..................................................................................... 104 Table 4-44. Dynamics characteristics: Ethernet MAC signals for SMI(1)............................................ 104 Table 4-45. Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings(1)(2)(3) ....................... 105 Table 4-46. Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings(1)(2)(3) ...................... 106 Table 4-47. Asynchronous multiplexed PSRAM/NOR read timings(1)(2)(3) .......................................... 106 Table 4-48. Asynchronous multiplexed PSRAM/NOR write timings(1)(2)(3).......................................... 106 Table 4-49. Synchronous multiplexed PSRAM/NOR read timings(1)(2)(3)............................................. 107 Table 4-50. Synchronous multiplexed PSRAM write timings(1)(2)(3) ..................................................... 107 Table 4-51. Synchronous non-multiplexed PSRAM/NOR read timings(1)(2)(3)..................................... 108 Table 4-52. Synchronous non-multiplexed PSRAM write timings(1)(2)(3) ............................................. 108 Table 4-53. TIMER characteristics(1) ...................................................................................................... 109 Table 4-54. DCI characteristics(1) ........................................................................................................... 109 Table 4-55. FWDGT min/max timeout period at 32 kHz (IRC32K)(1) .....................................................110 Table 4-56. WWDGT min-max timeout value at 60 MHz (fPCLK1)(1).........................................................110 Table 5-1. BGA176 package dimensions ............................................................................................... 111 Table 5-2. LQFP144 package dimensions ..............................................................................................113 Table 5-3. BGA100 package dimensions ...............................................................................................115 Table 5-4. LQFP100 package dimensions ..............................................................................................117 Table 5-5. Package thermal characteristics(1) ........................................................................................119 Table 6-1. Part ordering code for GD32F470xx devices ...................................................................... 121 Table 7-1. Revision history ..................................................................................................................... 122

6

GD32F470xx Datasheet

1.

General description

The GD32F470xx device belongs to the stretch performance line of GD32 MCU family. It is a new 32-bit general-purpose microcontroller based on the Arm® Cortex®-M4 RISC core with best cost-performance  ratio in terms  of  enhanced  processing capacity, reduced power consumption and peripheral set. The Cortex®-M4 core features a Floating Point Unit (FPU) that accelerates single precision floating point math operations and supports all Arm® single precision instructions and data types. It implements a full set of DSP instructions to address digital signal control markets that demand an efficient, easy-to-use blend of control and signal processing capabilities. It also provides a Memory Protection Unit (MPU) and powerful trace technology for enhanced application security and advanced debug support.

The GD32F470xx device incorporates the Arm® Cortex®-M4 32-bit processor core operating at 240 MHz frequency with Flash accesses zero wait states to obtain maximum efficiency. It provides up to 3072 KB on-chip Flash memory and 768 KB SRAM memory. An extensive range of enhanced I/Os and peripherals connected to two APB buses. The devices offer up to three 12-bit 2.6 MSPS ADCs, two 12-bit DACs, up to eight general 16-bit timers, two 16- bit PWM advanced timers, two 32-bit general timers, and two 16-bit basic timers, as well as standard and advanced communication interfaces: up to six SPIs, three I2Cs, four USARTs and four UARTs, two I2Ss, two CANs, a SDIO, USBFS and USBHS, and an ENET. Additional peripherals as Digital camera interface (DCI), EXMC interface with SDRAM extension support, TFT-LCD Interface (TLI) and Image Processing Accelerator (IPA) are included.

The device operates from a 2.6 to 3.6V power supply and available in -40 to +85 °C temperature range for grade 6 devices, and -40 to 105 °C temperature range for grade 7 devices. Three power saving modes provide the flexibility for maximum optimization of power consumption, an especially important consideration in low power applications.

The above features make GD32F470xx devices suitable for a wide range of interconnection and advanced applications, especially in areas such as industrial control, consumer and handheld equipment, embedded modules, human machine interface, security and alarm systems, graphic display, automotive navigation, drone, IoT and so on.

7

Flash

Timers

Connectivity

GD32F470xx Datasheet

2.	Device overview

2.1.

Device information

Table 2-1. GD32F470xx devices features and peripheral list

| Part Number |  | GD32F470xx |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | VE | VG | VI | VK | VG | VI | VK | ZE | ZG | ZI | ZK | IG | II | IK |
|  | Code area (KB) | 512 | 768 | 512 | 1024 | 768 | 512 | 1024 | 512 | 768 | 512 | 1024 | 768 | 512 | 1024 |
|  | Data area (KB) | 0 | 256 | 1536 | 2048 | 256 | 1536 | 2048 | 0 | 256 | 1536 | 2048 | 256 | 1536 | 2048 |
|  | Total (KB) | 512 | 1024 | 2048 | 3072 | 1024 | 2048 | 3072 | 512 | 1024 | 2048 | 3072 | 1024 | 2048 | 3072 |
| SRAM (KB) |  | 256 | 512 | 768 | 256 | 512 | 768 | 256 | 256 | 512 | 768 | 256 | 512 | 768 | 256 |
|  | General timer(16 -bit) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) | 8 (2-3,8-13) |
|  | General timer(32 -bit) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) | 2 (1,4) |
|  | Advanc ed timer(16 -bit) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) | 2 (0,7) |
|  | Basic timer(16 -bit) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) | 2 (5,6) |
|  | SysTick | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | Watchd og | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | RTC | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | USART | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | UART | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 | 4 |
|  | I2C | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 | 3 |
|  | SPI/I2S | 5/2 (0-4)/(1-2) | 5/2 (0-4)/(1-2) | 5/2 (0-4)/(1-2) | 5/2 (0-4)/(1-2) | 5/2 (0-4)/(1-2) | 5/2 (0-4)/(1-2) | 5/2 (0-4)/(1-2) | 6/2 (0-5)/(1-2) | 6/2 (0-5)/(1-2) | 6/2 (0-5)/(1-2) | 6/2 (0-5)/(1-2) | 6/2 (0-5)/(1-2) | 6/2 (0-5)/(1-2) | 6/2 (0-5)/(1-2) |
|  | SDIO | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |

8

DAC

|  |  | Part Number | GD32F470xx |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VE | VG | VI | VK | VG | VI | VK | ZE | ZG | ZI | ZK | IG | II | IK |
|  |  | CAN | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  |  | USB | FS+ HS | FS+ HS | FS+ HS | FS+ HS | FS+H S | FS+H S | FS+H S | FS+ HS | FS+ HS | FS+ HS | FS+ HS | FS+ HS | FS+ HS | FS+ HS |
|  |  | ENET | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | TLI | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | DCI | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  | GPIO |  | 82 | 82 | 82 | 82 | 82 | 82 | 82 | 114 | 114 | 114 | 114 | 140 | 140 | 140 |
|  | EXMC/SDR AM |  | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/0 | 1/1 | 1/1 | 1/1 | 1/1 | 1/1 | 1/1 | 1/1 |
|  | ADC(CHs) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
|  |  | Units | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 | 1 |
|  |  | Channel | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 | 2 |
|  | Package |  | LQFP100 |  |  |  | BGA100 |  |  | LQFP144 |  |  |  | BGA176 |  |  |

GD32F470xx Datasheet

3(16) 3(16) 3(16) 3(16) 3(16) 3(16) 3(16) 3(24) 3(24) 3(24) 3(24) 3(24) 3(24) 3(24)

9

IBUS

DBUS

SBUS

AHB Interconnect Matrix (Fmax=240MHz)

APB2 (Fmax=120MHz)

slave

APB1 (Fmax=60MHz)

slave

GD32F470xx Datasheet

2.2.

Block diagram

Figure 2-1. GD32F470xx block diagram

Powered By LDO (1.2V)

Flash Memory

TPIU	SW/JTAG	master

slave

FMC

ARM Cortex-M4	master	slave	Powered By VDDA

Processor

Fmax: 240MHz

master

slave

slave

TCMSRAM

SRAM0

DAC

LVD	PLLs

DMA0

DMA1

ENET

M

P

M

P

master

master

master

master

master

slave slave slave slave

SRAM1

SRAM2

ADDSRAM

EXMC

IRC16M

IRC32K

BKPSRAM	CRC	GPIO	RCU

TLI	master	slave

AHB1 Peripherals

USBHS	master

TRNG	DCI	USBFS

slave

IPA	master	AHB2 Peripherals

AHB Interconnect Matrix (Fmax=240MHz)

EXTI SDIO SPI5 SPI4 SPI3 SPI0

ADC0~2

SYSCFG TIMER10 TIMER9 TIMER8 TIMER7 TIMER0 USART5 USART0

CTC

IREF TIMER13 TIMER12 TIMER11 TIMER6 TIMER5 TIMER4 TIMER3 TIMER2 TIMER1 WWDGT

DAC0 CAN1 CAN0 UART7 UART6 UART4 UART3 USART2 USART1

I2C2 I2C1 I2C0

SAR

ADC

Powered By VDDA

POR/ PDR

LDO

I2S2_add SPI2/I2S2 SPI1/I2S1

FWDGT	I2S1_add

HXTAL	PMU

LXTAL

Powered By VDD

RTC

Powered By VBAT

10

GD32F470xx Datasheet

2.3.

Pinouts and pin assignment

Figure 2-2. GD32F470Ix BGA176 pinouts

1	2	3	4	5	6	7	8	9	10	11	12	13	14	15

A	PE3	PE2	PE1	PE0	PB8	PB5	PG14  PG13   PB4	PB3	PD7   PC12   PA15   PA14   PA13

B	PE4	PE5	PE6	PB9	PB7	PB6	PG15  PG12  PG11  PG10	PD6	PD0	PC11  PC10   PA12

C	VBAT   PI7	PI6	PI5	VDD   PDR_ON	VDD	VDD	VDD	PG9	PD5	PD1	PI3	PI2	PA11

D	PC13	PI8	PI9	PI4	VSS   BOOT0	VSS	VSS	VSS	PD4	PD3	PD2	PH15	PI1	PA10

E	PC14	PF0	PI10	PI11	PH13   PH14	PI0	PA9

F

G

PC15

PH0/O SCIN

VSS

VSS

VDD

VDD

PH2

PH3

VSS	VSS	VSS	VSS	VSS

VSS	VSS	VSS	VSS	VSS

VSS

VSS

NC

VDD

PC9	PA8

PC8	PC7

H

J

PH1/O

SCOU

T

NRST

PF2	PF1	PH4

PF3	PF4	PH5

VSS	VSS	VSS	VSS	VSS

VSS	VSS	VSS	VSS	VSS

VSS

VDD

VDD

VDD

PG8	PC6

PG7	PG6

K	PF7	PF6	PF5	VDD	VSS	VSS	VSS	VSS	VSS	PH12	PG5	PG4	PG3

L

PF10

PF9	PF8	NC

GigaDevice GD32F470Ix

BGA176

PH11   PH10  PD15

PG2

M	VSSA   PC0	PC1	PC2	PC3	PB2	PG1	VSS	VSS	NC	PH6	PH8	PH9   PD14   PD13

N

VREFN

PA1	PA0-

WKUP

PA4	PC4

PF13

PG0

VDD	VDD	VDD   PE13

PH7

PD12

PD11   PD10

P	VREFP	PA2	PA6	PA5	PC5	PF12   PF15   PE8	PE9	PE11   PE14   PB12   PB13	PD9	PD8

R	VDDA   PA3	PA7	PB1	PB0	PF11   PF14	PE7	PE10   PE12   PE15   PB10	PB11  PB14   PB15

11

PA3

VSS

VDD

PA4

PA5

PA6

PA7

PC4

PC5

PB0

PB1

PB2

VDD

PDR_ON

PE1

PE0

PB9

PB8

BOOT0

PB7

PB6

PB5

PB4

PB3

PF11

PF12

PF13

PF14

PF15

PG0

PG1

PE7

PE9

PG14

PG13

PG12

PG11

PG10

PG9

PD7

PD6

VSS

PG15

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

PA14

VSS

VDD

PE8

VSS

VDD

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

NC

VDD

VDD

VSS

VDD

GD32F470xx Datasheet

Figure 2-3. GD32F470Zx LQFP144 pinouts

144143142141140139138137136135134133132131130129128127126125124123122121120119118117116115114113112111110109

PE2 PE3 PE4 PE5 PE6

VBAT

PC13-TAMPER-RTC

PC14-OSC32IN

PC15-OSC32OUT PF0 PF1 PF2 PF3 PF4 PF5 VSS VDD PF6 PF7 PF8 PF9 PF10

PH0/OSCIN PH1/OSCOUT

NRST PC0 PC1 PC2 PC3 VDD VSSA VREFP

1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16

17 18 19 20 21 22 23 24 25 26 27 28 29 30 31 32

GigaDevice GD32F470Zx

LQFP144

108 107 106 105 104 103 102 101 100 99 98 97 96 95 94 93 92 91 90 89 88 87 86 85 84 83 82 81 80 79 78 77

VDD VSS NC PA13 PA12 PA11 PA10 PA9 PA8 PC9 PC8 PC7 PC6 VDD VSS PG8 PG7 PG6 PG5 PG4 PG3 PG2 PD15 PD14 VDD VSS PD13 PD12 PD11 PD10 PD9 PD8

VDDA	33	76	PB15

PA0-WKUP PA1 PA2

34 35 36

75 74 73

PB14 PB13 PB12

37  38  39  40  41  42  43  44  45  46  47  48  49  50  51  52  53  54  55  56  57  58  59  60  61  62  63  64  65  66  67  68  69  70  71  72

12

PA3

VSS

VDD

PA4

PA5

PA6

PA7

PC4

PC5

PB0

PB1

PB2

VDD

VSS

PE1

PE0

PB9

PB8

BOOT0

PB7

PB6

PB5

PB4

PB3

PE7

PE8

PE9

PE10

PE11

PE12

PE13

PE14

PE15

PB10

PB11

NC

PD5

PD4

PD3

PD2

PD1

PD0

PC12

PC11

PC10

PA15

VDD

PA14

PD6

PD7

GD32F470xx Datasheet

Figure 2-4. GD32F470Vx BGA100 pinouts

1	2	3	4	5	6	7	8	9	10	11	12

A	PE3	PE1	PB8   BOOT0	PD7	PD5	PB4	PB3	PA15   PA14   PA13   PA12

B	PE4	PE2	PB9	PB7	PB6	PD6	PD4	PD3	PD1   PC12   PC10   PA11

C	PC13	PE5	PE0	VDD	PB5	PD2	PD0	PC11	NC	PA10

D	PC14	PE6	VSS	PA9	PA8	PC9

E	PC15   VBAT	NC	PC8	PC7	PC6

F

PH0

VSS

GigaDevice GD32F470Vx

BGA100

VSS

VSS

G

H

PH1

PC0

VDD

NRST

PDR_

ON

VDD	VDD

PD15	PD14  PD13

J	VSSA   PC1	PC2	PD12   PD11  PD10

K	VREFN   PC3	PA2	PA5	PC4	PD9   PB11   PB15	PB14   PB13

L	VREFP	PA0	PA3	PA6	PC5	PB2	PE8	PE10   PE12   PB10	NC	PB12

M	VDDA   PA1	PA4	PA7	PB0	PB1	PE7	PE9	PE11   PE13   PE14   PE15

Figure 2-5. GD32F470Vx LQFP100 pinouts

100 99  98  97  96  95  94  93  92  91  90  89  88  87  86  85  84  83  82  81  80  79  78  77  76

PE2 PE3 PE4 PE5 PE6

VBAT

PC13-TAMPER-RTC

PC14-OSC32IN PC15-OSC32OUT

VSS

VDD

PH0/OSCIN

PH1/OSCOUT

NRST PC0 PC1 PC2 PC3 VDD

VSSA

VREFP

VDDA

PA0-WKUP PA1 PA2

1 2 3 4 5 6 7 8 9 10 11 12 13 14 15 16 17 18 19 20

21 22 23 24 25

GigaDevice GD32F470Vx

LQFP100

75 74 73 72 71 70 69 68 67 66 65 64 63 62 61 60 59 58 57 56 55 54 53 52 51

VDD VSS NC PA13 PA12 PA11 PA10 PA9 PA8 PC9 PC8 PC7 PC6 PD15 PD14 PD13 PD12 PD11 PD10 PD9 PD8 PB15 PB14 PB13 PB12

26  27  28  29  30  31  32  33  34  35  36  37  38  39  40  41  42  43  44  45  46  47  48  49  50

13

GD32F470xx Datasheet

2.4.

Memory map

Table 2-2. GD32F470xx memory map

| Pre-defined Regions | Bus | Address | Peripherals |
| --- | --- | --- | --- |
| External Device | AHB | 0xC000 0000 - 0xDFFF FFFF | EXMC - SDRAM |
|  |  | 0xA000 1000 - 0xBFFF FFFF | Reserved |
|  |  | 0xA000 0000 - 0xA000 0FFF | EXMC - SWREG |
| External RAM |  | 0x9000 0000 - 0x9FFF FFFF | EXMC - PC CARD |
|  |  | 0x7000 0000 - 0x8FFF FFFF | EXMC - NAND |
|  |  | 0x6000 0000 - 0x6FFF FFFF | EXMC - NOR/PSRAM/SRAM |
| Peripheral | AHB2 | 0x5006 0C00 - 0x5FFF FFFF | Reserved |
|  |  | 0x5006 0800 - 0x5006 0BFF | TRNG |
|  |  | 0x5005 0400 - 0x5006 07FF | Reserved |
|  |  | 0x5005 0000 - 0x5005 03FF | DCI |
|  |  | 0x5004 0000 - 0x5004 FFFF | Reserved |
|  |  | 0x5000 0000 - 0x5003 FFFF | USBFS |
|  | AHB1 | 0x4008 0000 - 0x4FFF FFFF | Reserved |
|  |  | 0x4004 0000 - 0x4007 FFFF | USBHS |
|  |  | 0x4002 BC00 - 0x4003 FFFF | Reserved |
|  |  | 0x4002 B000 - 0x4002 BBFF | IPA |
|  |  | 0x4002 A000 - 0x4002 AFFF | Reserved |
|  |  | 0x4002 8000 - 0x4002 9FFF | ENET |
|  |  | 0x4002 6800 - 0x4002 7FFF | Reserved |
|  |  | 0x4002 6400 - 0x4002 67FF | DMA1 |
|  |  | 0x4002 6000 - 0x4002 63FF | DMA0 |
|  |  | 0x4002 5000 - 0x4002 5FFF | Reserved |
|  |  | 0x4002 4000 - 0x4002 4FFF | BKP SRAM |
|  |  | 0x4002 3C00 - 0x4002 3FFF | FMC |
|  |  | 0x4002 3800 - 0x4002 3BFF | RCU |
|  |  | 0x4002 3400 - 0x4002 37FF | Reserved |
|  |  | 0x4002 3000 - 0x4002 33FF | CRC |
|  |  | 0x4002 2400 - 0x4002 2FFF | Reserved |
|  |  | 0x4002 2000 - 0x4002 23FF | GPIOI |
|  |  | 0x4002 1C00 - 0x4002 1FFF | GPIOH |
|  |  | 0x4002 1800 - 0x4002 1BFF | GPIOG |
|  |  | 0x4002 1400 - 0x4002 17FF | GPIOF |
|  |  | 0x4002 1000 - 0x4002 13FF | GPIOE |
|  |  | 0x4002 0C00 - 0x4002 0FFF | GPIOD |
|  |  | 0x4002 0800 - 0x4002 0BFF | GPIOC |
|  |  | 0x4002 0400 - 0x4002 07FF | GPIOB |
|  |  | 0x4002 0000 - 0x4002 03FF | GPIOA |

14

GD32F470xx Datasheet

|  | Pre-defined Regions | Bus | Address | Peripherals |
| --- | --- | --- | --- | --- |
|  |  | APB2 | 0x4001 6C00 - 0x4001 FFFF | Reserved |
|  |  |  | 0x4001 6800 - 0x4001 6BFF | TLI |
|  |  |  | 0x4001 5800 - 0x4001 67FF | Reserved |
|  |  |  | 0x4001 5400 - 0x4001 57FF | SPI5 |
|  |  |  | 0x4001 5000 - 0x4001 53FF | SPI4 |
|  |  |  | 0x4001 4C00 - 0x4001 4FFF | Reserved |
|  |  |  | 0x4001 4800 - 0x4001 4BFF | TIMER10 |
|  |  |  | 0x4001 4400 - 0x4001 47FF | TIMER9 |
|  |  |  | 0x4001 4000 - 0x4001 43FF | TIMER8 |
|  |  |  | 0x4001 3C00 - 0x4001 3FFF | EXTI |
|  |  |  | 0x4001 3800 - 0x4001 3BFF | SYSCFG |
|  |  |  | 0x4001 3400 - 0x4001 37FF | SPI3 |
|  |  |  | 0x4001 3000 - 0x4001 33FF | SPI0 |
|  |  |  | 0x4001 2C00 - 0x4001 2FFF | SDIO |
|  |  |  | 0x4001 2400 - 0x4001 2BFF | Reserved |
|  |  |  | 0x4001 2300 - 0x4001 23FF | ADC0(1) |
|  |  |  | 0x4001 2200 - 0x4001 22FF | ADC2 |
|  |  |  | 0x4001 2100 - 0x4001 21FF | ADC1 |
|  |  |  | 0x4001 2000 - 0x4001 20FF | ADC0 |
|  |  |  | 0x4001 1800 - 0x4001 1FFF | Reserved |
|  |  |  | 0x4001 1400 - 0x4001 17FF | USART5 |
|  |  |  | 0x4001 1000 - 0x4001 13FF | USART0 |
|  |  |  | 0x4001 0800 - 0x4001 0FFF | Reserved |
|  |  |  | 0x4001 0400 - 0x4001 07FF | TIMER7 |
|  |  |  | 0x4001 0000 - 0x4001 03FF | TIMER0 |
|  |  | APB1 | 0x4000 C800 - 0x4000 FFFF | Reserved |
|  |  |  | 0x4000 C400 - 0x4000 C7FF | IREF |
|  |  |  | 0x4000 8000 - 0x4000 C3FF | Reserved |
|  |  |  | 0x4000 7C00 - 0x4000 7FFF | UART7 |
|  |  |  | 0x4000 7800 - 0x4000 7BFF | UART6 |
|  |  |  | 0x4000 7400 - 0x4000 77FF | DAC0 |
|  |  |  | 0x4000 7000 - 0x4000 73FF | PMU |
|  |  |  | 0x4000 6C00 - 0x4000 6FFF | CTC |
|  |  |  | 0x4000 6800 - 0x4000 6BFF | CAN1 |
|  |  |  | 0x4000 6400 - 0x4000 67FF | CAN0 |
|  |  |  | 0x4000 6000 - 0x4000 63FF | Reserved |
|  |  |  | 0x4000 5C00 - 0x4000 5FFF | I2C2 |
|  |  |  | 0x4000 5800 - 0x4000 5BFF | I2C1 |
|  |  |  | 0x4000 5400 - 0x4000 57FF | I2C0 |
|  |  |  | 0x4000 5000 - 0x4000 53FF | UART4 |

15

GD32F470xx Datasheet

|  | Pre-defined Regions | Bus | Address | Peripherals |
| --- | --- | --- | --- | --- |
|  |  |  | 0x4000 4C00 - 0x4000 4FFF | UART3 |
|  |  |  | 0x4000 4800 - 0x4000 4BFF | USART2 |
|  |  |  | 0x4000 4400 - 0x4000 47FF | USART1 |
|  |  |  | 0x4000 4000 - 0x4000 43FF | I2S2_add |
|  |  |  | 0x4000 3C00 - 0x4000 3FFF | SPI2/I2S2 |
|  |  |  | 0x4000 3800 - 0x4000 3BFF | SPI1/I2S1 |
|  |  |  | 0x4000 3400 - 0x4000 37FF | I2S1_add |
|  |  |  | 0x4000 3000 - 0x4000 33FF | FWDGT |
|  |  |  | 0x4000 2C00 - 0x4000 2FFF | WWDGT |
|  |  |  | 0x4000 2800 - 0x4000 2BFF | RTC |
|  |  |  | 0x4000 2400 - 0x4000 27FF | Reserved |
|  |  |  | 0x4000 2000 - 0x4000 23FF | TIMER13 |
|  |  |  | 0x4000 1C00 - 0x4000 1FFF | TIMER12 |
|  |  |  | 0x4000 1800 - 0x4000 1BFF | TIMER11 |
|  |  |  | 0x4000 1400 - 0x4000 17FF | TIMER6 |
|  |  |  | 0x4000 1000 - 0x4000 13FF | TIMER5 |
|  |  |  | 0x4000 0C00 - 0x4000 0FFF | TIMER4 |
|  |  |  | 0x4000 0800 - 0x4000 0BFF | TIMER3 |
|  |  |  | 0x4000 0400 - 0x4000 07FF | TIMER2 |
|  |  |  | 0x4000 0000 - 0x4000 03FF | TIMER1 |
|  | SRAM | AHB | 0x200B 0000 - 0x3FFF FFFF | Reserved |
|  |  |  | 0x2003 0000 - 0x200A FFFF | ADDSRAM(512KB) |
|  |  |  | 0x2002 0000 - 0x2002 FFFF | SRAM2(64KB) |
|  |  |  | 0x2001 C000 - 0x2001 FFFF | SRAM1(16KB) |
|  |  |  | 0x2000 0000 - 0x2001 BFFF | SRAM0(112KB) |
|  | Code | AHB | 0x1FFF C010 - 0x1FFF FFFF | Reserved |
|  |  |  | 0x1FFF C000 - 0x1FFF C00F | Option bytes(Bank 0) |
|  |  |  | 0x1FFF 7A10 - 0x1FFF BFFF | Reserved |
|  |  |  | 0x1FFF 7800 - 0x1FFF 7A0F | OTP(512B) |
|  |  |  | 0x1FFF 0000 - 0x1FFF 77FF | Boot loader(30KB) |
|  |  |  | 0x1FFE C010 - 0x1FFE FFFF | Reserved |
|  |  |  | 0x1FFE C000 - 0x1FFE C00F | Option bytes(Bank 1) |
|  |  |  | 0x1001 0000 - 0x1FFE BFFF | Reserved |
|  |  |  | 0x1000 0000 - 0x1000 FFFF | TCMSRAM(64KB) |
|  |  |  | 0x0830 0000 - 0x0FFF FFFF | Reserved |
|  |  |  | 0x0800 0000 - 0x082F FFFF | Main Flash(3072KB) |
|  |  |  | 0x0000 0000 - 0x07FF FFFF | Aliased to the boot device |

Note:

(1)  ADC_SSTAT, ADC_SYNCCTL, ADC_SYNCDATA based on base address of ADC0.

16

GD32F470xx Datasheet

2.5.

Clock tree

Figure 2-6. GD32F470xx clock tree

CK_HXTAL	/2 to /31	11

32.768 KHz LXTAL OSC

32 KHz IRC32K

01

10

RTCSRC[1:0]

CK_RTC (to RTC)

CK_FWDGT (to FWDGT)

CK_OUT1

CKOUT1DIV ÷1,2,3,4,5

00 01 10 11

CKOUT1SEL[1:0]

CK_SYS CK_PLLI2SR CK_HXTAL CK_PLLP

AHB enable

HCLK

(to AHB bus,Cortex-

CK_OUT0

CKOUT0DIV ÷1,2,3,4,5

00 01 10 11

CKOUT0SEL[1:0]

CK_IRC16M CK_LXTAL CK_HXTAL CK_PLLP

SCS[1:0]

APB1 Prescaler ÷1,2,4,8,16

M4,SRAM,DMA,peripherals)

CK_CST ÷8

(to Cortex-M4 SysTick)

FCLK

(free running clock)

CK_APB1

PCLK1

60 MHz max	to APB1 peripherals

16 MHz IRC16M

4-32 MHz HXTAL

PLLSEL

CK_IRC16M

CK_HXTAL

CK_PLLP

00

CK_SYS

01	240 MHz max

10

Clock

Monitor

AHB Prescaler ÷1,2...512

CK_AHB

240 MHz max

TIMER1,2,3,4,5,6,

11,12,13

CK_APB1 x1 x2 or x4

APB2 Prescaler ÷1,2,4,8,16

TIMER0,7,8, 9,10

CK_APB2 x1

Peripheral enable

240 MHz max TIMERx enable

CK_APB2

120 MHz max

Peripheral enable

240 MHz max

CK_TIMERx

to TIMER1,2,3,4, 5,6,11,12,13

PCLK2

to APB2 peripherals

CK_TIMERx

/PSC

0

1

CTC

x2 or x4

ADC

TIMERx enable

ADCCK[2]

to TIMER0,7, 8,9,10

VCO

xN

PLL

/P /Q /R

PLL48MSEL

48 MHz IRC48M

CK_CTC CK48MSEL

Prescaler

÷2,4,6,8

ADC Prescaler ÷5,6,10,20

0

1

CK_ADCX to ADC0,1,2

40 MHz max

VCO

/P

0

1

I2SSEL

1

0

CK48M

Peripheral enable

to USBFS USBHS TRNG

/Q

xN	/R

PLLI2S

/P

VCO

/Q

xN	/R

PLLSAI

ENET_TX_CLK

/2 or /20

0

1

I2S_CKIN

/DIV

ENET_PHY_SEL

1

0

Peripheral enable

Peripheral enable

Peripheral enable

SDIO

CK_I2Sx

to I2S

CK_TLI

to TLI CK_ENETTX to ENET TX

ENET_RX_CLK

1

0

EMBPHY

Peripheral enable

CK_ENETRX

to ENET RX

USB HS PHY clock 24Mhz to 60Mhz

CK48M

0

1

Peripheral enable

CK_USBHS_ULPI to USBHS ULPI

Legend:

HXTAL: High speed crystal oscillator LXTAL: Low speed crystal oscillator IRC16M: Internal 16M RC oscillators IRC32K: Internal 32K RC oscillator IRC48M: Internal 48M RC oscillators

17

GD32F470xx Datasheet

2.6.	Pin definitions

2.6.1.

GD32F470Ix BGA176 pin definitions

Table 2-3. GD32F470Ix BGA176 pin definitions

| GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
| PE2 | A2 | I/O | 5VT | Default: PE2 Alternate: SPI3_SCK, ENET_MII_TXD3, EXMC_A23, EVENTOUT |
| PE3 | A1 | I/O | 5VT | Default: PE3 Alternate: EXMC_A19, EVENTOUT |
| PE4 | B1 | I/O | 5VT | Default: PE4 Alternate: SPI3_NSS, EXMC_A20, DCI_D4, TLI_B0, EVENTOUT |
| PE5 | B2 | I/O | 5VT | Default: PE5 Alternate: TIMER8_CH0, SPI3_MISO, EXMC_A21, DCI_D6, TLI_G0, EVENTOUT |
| PE6 | B3 | I/O | 5VT | Default: PE6 Alternate: TIMER8_CH1, SPI3_MOSI, EXMC_A22, DCI_D7, TLI_G1, EVENTOUT |
| VBAT | C1 | P | - | Default: VBAT |
| PI8 | D2 | I/O | 5VT | Default: PI8 Alternate: EVENTOUT Additional: RTC_TAMP1, RTC_TAMP0, RTC_TS |
| PC13- TAMPER- RTC | D1 | I/O | 5VT | Default: PC13 Alternate: EVENTOUT Additional: RTC_TAMP0, RTC_OUT, RTC_TS |
| PC14- OSC32IN | E1 | I/O | 5VT | Default: PC14 Alternate: EVENTOUT Additional: OSC32IN |
| PC15- OSC32OU T | F1 | I/O | 5VT | Default: PC15 Alternate: EVENTOUT Additional: OSC32OUT |
| PI9 | D3 | I/O | 5VT | Default: PI9 Alternate: CAN0_RX, EXMC_D30, TLI_VSYNC, EVENTOUT |
| PI10 | E3 | I/O | 5VT | Default: PI10 Alternate: ENET_MII_RX_ER, EXMC_D31, TLI_HSYNC, EVENTOUT |
| PI11 | E4 | I/O | 5VT | Default: PI11 Alternate: USBHS_ULPI_DIR, EVENTOUT |
| VSS | F2 | P | - | Default: VSS |
| VDD | F3 | P | - | Default: VDD |
| PF0 | E2 | I/O | 5VT | Default: PF0 |

18

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Alternate: I2C1_SDA, EXMC_A0, EVENTOUT, CTC_SYNC |
|  | PF1 | H3 | I/O | 5VT | Default: PF1 Alternate: I2C1_SCL, EXMC_A1, EVENTOUT |
|  | PF2 | H2 | I/O | 5VT | Default: PF2 Alternate: I2C1_SMBA, EXMC_A2, EVENTOUT |
|  | PF3 | J2 | I/O | 5VT | Default: PF3 Alternate: EXMC_A3, EVENTOUT, I2C1_TXFRAME Additional: ADC2_IN9 |
|  | PF4 | J3 | I/O | 5VT | Default: PF4 Alternate: EXMC_A4, EVENTOUT Additional: ADC2_IN14 |
|  | PF5 | K3 | I/O | 5VT | Default: PF5 Alternate: EXMC_A5, EVENTOUT Additional: ADC2_IN15 |
|  | VSS | G2 | P | - | Default: VSS |
|  | VDD | G3 | P | - | Default: VDD |
|  | PF6 | K2 | I/O | 5VT | Default: PF6 Alternate: TIMER9_CH0, SPI4_NSS, UART6_RX, EXMC_NIORD, EVENTOUT Additional: ADC2_IN4 |
|  | PF7 | K1 | I/O | 5VT | Default: PF7 Alternate: TIMER10_CH0, SPI4_SCK, UART6_TX, EXMC_NREG, EVENTOUT Additional: ADC2_IN5 |
|  | PF8 | L3 | I/O | 5VT | Default: PF8 Alternate: SPI4_MISO, TIMER12_CH0, EXMC_NIOWR, EVENTOUT Additional: ADC2_IN6 |
|  | PF9 | L2 | I/O | 5VT | Default: PF9 Alternate: SPI4_MOSI, TIMER13_CH0, EXMC_CD, EVENTOUT Additional: ADC2_IN7 |
|  | PF10 | L1 | I/O | 5VT | Default: PF10 Alternate: EXMC_INTR, DCI_D11, TLI_DE, EVENTOUT Additional: ADC2_IN8 |
|  | PH0/OSCI N | G1 | I/O | 5VT | Default: PH0, OSCIN Alternate: EVENTOUT Additional: OSCIN |
|  | PH1/OSCO UT | H1 | I/O | 5VT | Default: PH1, OSCOUT Alternate: EVENTOUT Additional: OSCOUT |
|  | NRST | J1 | - | - | Default: NRST |
|  | PC0 | M2 | I/O | 5VT | Default: PC0 Alternate: USBHS_ULPI_STP, EXMC_SDNWE, EVENTOUT |

19

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Additional: ADC012_IN10 |
|  | PC1 | M3 | I/O | 5VT | Default: PC1 Alternate: SPI2_MOSI, I2S2_SD, SPI1_MOSI, I2S1_SD, ENET_MDC, EVENTOUT Additional: ADC012_IN11 |
|  | PC2 | M4 | I/O | 5VT | Default: PC2 Alternate: SPI1_MISO, I2S1_ADD_SD, USBHS_ULPI_DIR, ENET_MII_TXD2, EXMC_SDNE0, EVENTOUT Additional: ADC012_IN12 |
|  | PC3 | M5 | I/O | 5VT | Default: PC3 Alternate: SPI1_MOSI, I2S1_SD, USBHS_ULPI_NXT, ENET_MII_TX_CLK, EXMC_SDCKE0, EVENTOUT Additional: ADC012_IN13 |
|  | VSSA | M1 | P | - | Default: VSSA |
|  | VREFN | N1 | P | - | Default: VREFN |
|  | VREFP | P1 | P | - | Default: VREFP |
|  | VDDA | R1 | P | - | Default: VDDA |
|  | PA0-WKUP | N3 | I/O | 5VT | Default: PA0 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER4_CH0, TIMER7_ETI, USART1_CTS, UART3_TX, ENET_MII_CRS, EVENTOUT Additional: ADC012_IN0, WKUP |
|  | PA1 | N2 | I/O | 5VT | Default: PA1 Alternate: TIMER1_CH1, TIMER4_CH1, SPI3_MOSI, USART1_RTS, UART3_RX, ENET_MII_RX_CLK, ENET_RMII_REF_CLK, EVENTOUT Additional: ADC012_IN1 |
|  | PA2 | P2 | I/O | 5VT | Default: PA2 Alternate: TIMER1_CH2, TIMER4_CH2, TIMER8_CH0, I2S_CKIN, USART1_TX, ENET_MDIO, EVENTOUT Additional: ADC012_IN2 |
|  | PH2 | F4 | I/O | 5VT | Default: PH2 Alternate: ENET_MII_CRS, EXMC_SDCKE0, TLI_R0, EVENTOUT |
|  | PH3 | G4 | I/O | 5VT | Default: PH3 Alternate: ENET_MII_COL, EXMC_SDNE0, TLI_R1, EVENTOUT, I2C1_TXFRAME |
|  | PH4 | H4 | I/O | 5VT | Default: PH4 Alternate: I2C1_SCL, USBHS_ULPI_NXT, EVENTOUT |
|  | PH5 | J4 | I/O | 5VT | Default: PH5 Alternate: I2C1_SDA, SPI4_NSS, EXMC_SDNWE, EVENTOUT |
|  | PA3 | R2 | I/O | 5VT | Default: PA3 Alternate: TIMER1_CH3, TIMER4_CH3, TIMER8_CH1, |

20

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | I2S1_MCK, USART1_RX, USBHS_ULPI_D0, ENET_MII_COL, TLI_B5, EVENTOUT Additional: ADC012_IN3 |
|  | NC | L4 | - | - | - |
|  | VDD | K4 | P | - | Default: VDD |
|  | PA4 | N4 | I/O |  | Default: PA4 Alternate: SPI0_NSS, SPI2_NSS, I2S2_WS, USART1_CK, USBHS_SOF, DCI_HSYNC, TLI_VSYNC, EVENTOUT Additional: ADC01_IN4, DAC0_OUT0 |
|  | PA5 | P4 | I/O |  | Default: PA5 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER7_CH0_ON, SPI0_SCK, USBHS_ULPI_CK, EVENTOUT Additional: ADC01_IN5, DAC0_OUT1 |
|  | PA6 | P3 | I/O | 5VT | Default: PA6 Alternate: TIMER0_BRKIN, TIMER2_CH0, TIMER7_BRKIN, SPI0_MISO, I2S1_MCK, TIMER12_CH0, SDIO_CMD, DCI_PIXCLK, TLI_G2, EVENTOUT Additional: ADC01_IN6 |
|  | PA7 | R3 | I/O | 5VT | Default: PA7 Alternate: TIMER0_CH0_ON, TIMER2_CH1, TIMER7_CH0_ON, SPI0_MOSI, TIMER13_CH0, ENET_MII_RX_DV, ENET_RMII_CRS_DV, EXMC_SDNWE, EVENTOUT Additional: ADC01_IN7 |
|  | PC4 | N5 | I/O | 5VT | Default: PC4 Alternate: ENET_MII_RXD0, ENET_RMII_RXD0, EXMC_SDNE0, EVENTOUT Additional: ADC01_IN14 |
|  | PC5 | P5 | I/O | 5VT | Default: PC5 Alternate: USART2_RX, ENET_MII_RXD1, ENET_RMII_RXD1, EXMC_SDCKE0, EVENTOUT Additional: ADC01_IN15 |
|  | PB0 | R5 | I/O | 5VT | Default: PB0 Alternate: TIMER0_CH1_ON, TIMER2_CH2, TIMER7_CH1_ON, SPI4_SCK, SPI2_MOSI, I2S2_SD, TLI_R3, USBHS_ULPI_D1, ENET_MII_RXD2, SDIO_D1, EVENTOUT Additional: ADC01_IN8, IREF |
|  | PB1 | R4 | I/O | 5VT | Default: PB1 Alternate: TIMER0_CH2_ON, TIMER2_CH3, TIMER7_CH2_ON, SPI4_NSS, TLI_R6, USBHS_ULPI_D2, ENET_MII_RXD3, SDIO_D2, EVENTOUT Additional: ADC01_IN9 |
|  | PB2 | M6 | I/O | 5VT | Default: PB2, BOOT1 Alternate: TIMER1_CH3, SPI2_MOSI, I2S2_SD, |

21

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | USBHS_ULPI_D4, SDIO_CK, EVENTOUT |
|  | PF11 | R6 | I/O | 5VT | Default: PF11 Alternate: SPI4_MOSI, EXMC_SDNRAS, DCI_D12, EVENTOUT |
|  | PF12 | P6 | I/O | 5VT | Default: PF12 Alternate: EXMC_A6, EVENTOUT |
|  | VSS | M8 | P | - | Default: VSS |
|  | VDD | N8 | P | - | Default: VDD |
|  | PF13 | N6 | I/O | 5VT | Default: PF13 Alternate: EXMC_A7, EVENTOUT |
|  | PF14 | R7 | I/O | 5VT | Default: PF14 Alternate: EXMC_A8, EVENTOUT |
|  | PF15 | P7 | I/O | 5VT | Default: PF15 Alternate: EXMC_A9, EVENTOUT |
|  | PG0 | N7 | I/O | 5VT | Default: PG0 Alternate: EXMC_A10, EVENTOUT |
|  | PG1 | M7 | I/O | 5VT | Default: PG1 Alternate: EXMC_A11, EVENTOUT |
|  | PE7 | R8 | I/O | 5VT | Default: PE7 Alternate: TIMER0_ETI, UART6_RX, EXMC_D4, EVENTOUT |
|  | PE8 | P8 | I/O | 5VT | Default: PE8 Alternate: TIMER0_CH0_ON, UART6_TX, EXMC_D5, EVENTOUT |
|  | PE9 | P9 | I/O | 5VT | Default: PE9 Alternate: TIMER0_CH0, EXMC_D6, EVENTOUT |
|  | VSS | M9 | P | - | Default: VSS |
|  | VDD | N9 | P | - | Default: VDD |
|  | PE10 | R9 | I/O | 5VT | Default: PE10 Alternate: TIMER0_CH1_ON, EXMC_D7, EVENTOUT |
|  | PE11 | P10 | I/O | 5VT | Default: PE11 Alternate: TIMER0_CH1, SPI3_NSS, SPI4_NSS, EXMC_D8, TLI_G3, EVENTOUT |
|  | PE12 | R10 | I/O | 5VT | Default: PE12 Alternate: TIMER0_CH2_ON, SPI3_SCK, SPI4_SCK, EXMC_D9, TLI_B4, EVENTOUT |
|  | PE13 | N11 | I/O | 5VT | Default: PE13 Alternate: TIMER0_CH2, SPI3_MISO, SPI4_MISO, EXMC_D10, TLI_DE, EVENTOUT |
|  | PE14 | P11 | I/O | 5VT | Default: PE14 Alternate: TIMER0_CH3, SPI3_MOSI, SPI4_MOSI, EXMC_D11, TLI_PIXCLK, EVENTOUT |
|  | PE15 | R11 | I/O | 5VT | Default: PE15 Alternate: TIMER0_BRKIN, EXMC_D12, TLI_R7, |

22

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | EVENTOUT |
|  | PB10 | R12 | I/O | 5VT | Default: PB10 Alternate: TIMER1_CH2, I2C1_SCL, SPI1_SCK, I2S1_CK, I2S2_MCK, USART2_TX, USBHS_ULPI_D3, ENET_MII_RX_ER, SDIO_D7, TLI_G4, EVENTOUT |
|  | PB11 | R13 | I/O | 5VT | Default: PB11 Alternate: TIMER1_CH3, I2C1_SDA, I2S_CKIN, USART2_RX, USBHS_ULPI_D4, ENET_MII_TX_EN, ENET_RMII_TX_EN, TLI_G5, EVENTOUT |
|  | NC | M10 | - | - | - |
|  | VDD | N10 | P | - | Default: VDD |
|  | PH6 | M11 | I/O | 5VT | Default: PH6 Alternate: I2C1_SMBA, SPI4_SCK, TIMER11_CH0, ENET_MII_RXD2, EXMC_SDNE1, DCI_D8, EVENTOUT |
|  | PH7 | N12 | I/O | 5VT | Default: PH7 Alternate: I2C2_SCL, SPI4_MISO, ENET_MII_RXD3, EXMC_SDCKE1, DCI_D9, EVENTOUT |
|  | PH8 | M12 | I/O | 5VT | Default: PH8 Alternate: I2C2_SDA, EXMC_D16, DCI_HSYNC, TLI_R2, EVENTOUT |
|  | PH9 | M13 | I/O | 5VT | Default: PH9 Alternate: I2C2_SMBA, TIMER11_CH1, EXMC_D17, DCI_D0, TLI_R3, EVENTOUT |
|  | PH10 | L13 | I/O | 5VT | Default: PH10 Alternate: TIMER4_CH0, EXMC_D18, DCI_D1, TLI_R4, EVENTOUT, I2C2_TXFRAME |
|  | PH11 | L12 | I/O | 5VT | Default: PH11 Alternate: TIMER4_CH1, EXMC_D19, DCI_D2, TLI_R5, EVENTOUT |
|  | PH12 | K12 | I/O | 5VT | Default: PH12 Alternate: TIMER4_CH2, EXMC_D20, DCI_D3, TLI_R6, EVENTOUT |
|  | VSS | H12 | P | - | Default: VSS |
|  | VDD | J12 | P | - | Default: VDD |
|  | PB12 | P12 | I/O | 5VT | Default: PB12 Alternate: TIMER0_BRKIN, I2C1_SMBA, SPI1_NSS, I2S1_WS, SPI3_NSS, USART2_CK, CAN1_RX, USBHS_ULPI_D5, ENET_MII_TXD0, ENET_RMII_TXD0, USBHS_ID, EVENTOUT |
|  | PB13 | P13 | I/O | 5VT | Default: PB13 Alternate: TIMER0_CH0_ON, SPI1_SCK, I2S1_CK, SPI3_SCK, USART2_CTS, CAN1_TX, USBHS_ULPI_D6, ENET_MII_TXD1, ENET_RMII_TXD1, EVENTOUT, I2C1_TXFRAME |

23

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Additional: USBHS_VBUS |
|  | PB14 | R14 | I/O | 5VT | Default: PB14 Alternate: TIMER0_CH1_ON, TIMER7_CH1_ON, SPI1_MISO, I2S1_ADD_SD, USART2_RTS, TIMER11_CH0, USBHS_DM, EVENTOUT |
|  | PB15 | R15 | I/O | 5VT | Default: PB15 Alternate: RTC_REFIN, TIMER0_CH2_ON, TIMER7_CH2_ON, SPI1_MOSI, I2S1_SD, TIMER11_CH1, USBHS_DP, EVENTOUT |
|  | PD8 | P15 | I/O | 5VT | Default: PD8 Alternate: USART2_TX, EXMC_D13, EVENTOUT |
|  | PD9 | P14 | I/O | 5VT | Default: PD9 Alternate: USART2_RX, EXMC_D14, EVENTOUT |
|  | PD10 | N15 | I/O | 5VT | Default: PD10 Alternate: USART2_CK, EXMC_D15, TLI_B3, EVENTOUT |
|  | PD11 | N14 | I/O | 5VT | Default: PD11 Alternate: USART2_CTS, EXMC_A16/EXMC_CLE, EVENTOUT |
|  | PD12 | N13 | I/O | 5VT | Default: PD12 Alternate: TIMER3_CH0, USART2_RTS, EXMC_A17/EXMC_ALE, EVENTOUT |
|  | PD13 | M15 | I/O | 5VT | Default: PD13 Alternate: TIMER3_CH1, EXMC_A18, EVENTOUT |
|  | VDD | J13 | P | - | Default: VDD |
|  | PD14 | M14 | I/O | 5VT | Default: PD14 Alternate: TIMER3_CH2, EXMC_D0, EVENTOUT |
|  | PD15 | L14 | I/O | 5VT | Default: PD15 Alternate: TIMER3_CH3, EXMC_D1, EVENTOUT, CTC_SYNC |
|  | PG2 | L15 | I/O | 5VT | Default: PG2 Alternate: EXMC_A12, EVENTOUT |
|  | PG3 | K15 | I/O | 5VT | Default: PG3 Alternate: EXMC_A13, EVENTOUT |
|  | PG4 | K14 | I/O | 5VT | Default: PG4 Alternate: EXMC_A14, EVENTOUT |
|  | PG5 | K13 | I/O | 5VT | Default: PG5 Alternate: EXMC_A15, EVENTOUT |
|  | PG6 | J15 | I/O | 5VT | Default: PG6 Alternate: EXMC_INT1, DCI_D12, TLI_R7, EVENTOUT |
|  | PG7 | J14 | I/O | 5VT | Default: PG7 Alternate: USART5_CK, EXMC_INT2, DCI_D13, TLI_PIXCLK, EVENTOUT |
|  | PG8 | H14 | I/O | 5VT | Default: PG8 Alternate: SPI5_NSS, USART5_RTS, ENET_PPS_OUT, EXMC_SDCLK, EVENTOUT |

24

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | VSS | G12 | P | - | Default: VSS |
|  | VDD | H13 | P | - | Default: VDD |
|  | PC6 | H15 | I/O | 5VT | Default: PC6 Alternate: TIMER2_CH0, TIMER7_CH0, I2S1_MCK, USART5_TX, SDIO_D6, DCI_D0, TLI_HSYNC, EVENTOUT |
|  | PC7 | G15 | I/O | 5VT | Default: PC7 Alternate: TIMER2_CH1, TIMER7_CH1, SPI1_SCK, I2S1_CK, I2S2_MCK, USART5_RX, SDIO_D7, DCI_D1, TLI_G6, EVENTOUT |
|  | PC8 | G14 | I/O | 5VT | Default: PC8 Alternate: TIMER2_CH2, TIMER7_CH2, USART5_CK, SDIO_D0, DCI_D2, EVENTOUT |
|  | PC9 | F14 | I/O | 5VT | Default: PC9 Alternate: CK_OUT1, TIMER2_CH3, TIMER7_CH3, I2C2_SDA, I2S_CKIN, SDIO_D1, DCI_D3, EVENTOUT |
|  | PA8 | F15 | I/O | 5VT | Default: PA8 Alternate: CK_OUT0, TIMER0_CH0, I2C2_SCL, USART0_CK, USBFS_SOF, SDIO_D1, TLI_R6, EVENTOUT, CTC_SYNC |
|  | PA9 | E15 | I/O | 5VT | Default: PA9 Alternate: TIMER0_CH1, I2C2_SMBA, SPI1_SCK, I2S1_CK, USART0_TX, SDIO_D2, DCI_D0, EVENTOUT Additional: USBFS_VBUS |
|  | PA10 | D15 | I/O | 5VT | Default: PA10 Alternate: TIMER0_CH2, SPI4_MOSI, USART0_RX, USBFS_ID, DCI_D1, EVENTOUT, I2C2_TXFRAME |
|  | PA11 | C15 | I/O | 5VT | Default: PA11 Alternate: TIMER0_CH3, SPI3_MISO, USART0_CTS, USART5_TX, CAN0_RX, USBFS_DM, TLI_R4, EVENTOUT |
|  | PA12 | B15 | I/O | 5VT | Default: PA12 Alternate: TIMER0_ETI, SPI4_MISO, USART0_RTS, USART5_RX, CAN0_TX, USBFS_DP, TLI_R5, EVENTOUT |
|  | PA13 | A15 | I/O | 5VT | Default: JTMS, SWDIO, PA13 Alternate: EVENTOUT |
|  | NC | F13 | - | - | - |
|  | VSS | F12 | P | - | Default: VSS |
|  | VDD | G13 | P | - | Default: VDD |
|  | PH13 | E12 | I/O | 5VT | Default: PH13 Alternate: TIMER7_CH0_ON, CAN0_TX, EXMC_D21, TLI_G2, EVENTOUT |
|  | PH14 | E13 | I/O | 5VT | Default: PH14 Alternate: TIMER7_CH1_ON, EXMC_D22, DCI_D4, TLI_G3, EVENTOUT |
|  | PH15 | D13 | I/O | 5VT | Default: PH15 |

25

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Alternate: TIMER7_CH2_ON, EXMC_D23, DCI_D11, TLI_G4, EVENTOUT |
|  | PI0 | E14 | I/O | 5VT | Default: PI0 Alternate: TIMER4_CH3, SPI1_NSS, I2S1_WS, EXMC_D24, DCI_D13, TLI_G5, EVENTOUT |
|  | PI1 | D14 | I/O | 5VT | Default: PI1 Alternate: SPI1_SCK, I2S1_CK, EXMC_D25, DCI_D8, TLI_G6, EVENTOUT |
|  | PI2 | C14 | I/O | 5VT | Default: PI2 Alternate: TIMER7_CH3, SPI1_MISO, I2S1_ADD_SD, EXMC_D26, DCI_D9, TLI_G7, EVENTOUT |
|  | PI3 | C13 | I/O | 5VT | Default: PI3 Alternate: TIMER7_ETI, SPI1_MOSI, I2S1_SD, EXMC_D27, DCI_D10, EVENTOUT |
|  | VSS | D9 | P | - | Default: VSS |
|  | VDD | C9 | P | - | Default: VDD |
|  | PA14 | A14 | I/O | 5VT | Default: JTCK, SWCLK, PA14 Alternate: EVENTOUT |
|  | PA15 | A13 | I/O | 5VT | Default: JTDI, PA15 Alternate: TIMER1_CH0, TIMER1_ETI, SPI0_NSS, SPI2_NSS, I2S2_WS, USART0_TX, EVENTOUT |
|  | PC10 | B14 | I/O | 5VT | Default: PC10 Alternate: SPI2_SCK, I2S2_CK, USART2_TX, UART3_TX, SDIO_D2, DCI_D8, TLI_R2, EVENTOUT |
|  | PC11 | B13 | I/O | 5VT | Default: PC11 Alternate: I2S2_ADD_SD, SPI2_MISO, USART2_RX, UART3_RX, SDIO_D3, DCI_D4, EVENTOUT |
|  | PC12 | A12 | I/O | 5VT | Default: PC12 Alternate: I2C1_SDA, SPI2_MOSI, I2S2_SD, USART2_CK, UART4_TX, SDIO_CK, DCI_D9, EVENTOUT |
|  | PD0 | B12 | I/O | 5VT | Default: PD0 Alternate: SPI3_MISO, SPI2_MOSI, I2S2_SD, CAN0_RX, EXMC_D2, EVENTOUT |
|  | PD1 | C12 | I/O | 5VT | Default: PD1 Alternate: SPI1_NSS, I2S1_WS, CAN0_TX, EXMC_D3, EVENTOUT |
|  | PD2 | D12 | I/O | 5VT | Default: PD2 Alternate: TIMER2_ETI, UART4_RX, SDIO_CMD, DCI_D11, EVENTOUT |
|  | PD3 | D11 | I/O | 5VT | Default: PD3 Alternate: SPI1_SCK, I2S1_CK, USART1_CTS, EXMC_CLK, DCI_D5, TLI_G7, EVENTOUT |
|  | PD4 | D10 | I/O | 5VT | Default: PD4 Alternate: USART1_RTS, EXMC_NOE, EVENTOUT |

26

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | PD5 | C11 | I/O | 5VT | Default: PD5 Alternate: USART1_TX, EXMC_NWE, EVENTOUT |
|  | VSS | D8 | P | - | Default: VSS |
|  | VDD | C8 | P | - | Default: VDD |
|  | PD6 | B11 | I/O | 5VT | Default: PD6 Alternate: SPI2_MOSI, I2S2_SD, USART1_RX, EXMC_NWAIT, DCI_D10, TLI_B2, EVENTOUT |
|  | PD7 | A11 | I/O | 5VT | Default: PD7 Alternate: USART1_CK, EXMC_NE0, EXMC_NCE1, EVENTOUT |
|  | PG9 | C10 | I/O | 5VT | Default: PG9 Alternate: USART5_RX, EXMC_NE1, EXMC_NCE2, DCI_VSYNC, EVENTOUT |
|  | PG10 | B10 | I/O | 5VT | Default: PG10 Alternate: SPI5_IO2, TLI_G3, EXMC_NCE3_0, EXMC_NE2, DCI_D2, TLI_B2, EVENTOUT |
|  | PG11 | B9 | I/O | 5VT | Default: PG11 Alternate: SPI5_IO3, SPI3_SCK, ENET_MII_TX_EN, ETH_RMII_TX_EN, EXMC_NCE3_1, DCI_D3, TLI_B3, EVENTOUT |
|  | PG12 | B8 | I/O | 5VT | Default: PG12 Alternate: SPI5_MISO, SPI3_MISO, USART5_RTS, TLI_B4, EXMC_NE3, TLI_B1, EVENTOUT |
|  | PG13 | A8 | I/O | 5VT | Default: PG13 Alternate: SPI5_SCK, SPI3_MOSI, USART5_CTS, ENET_MII_TXD0, ENET_RMII_TXD0, EXMC_A24, EVENTOUT |
|  | PG14 | A7 | I/O | 5VT | Default: PG14 Alternate: SPI5_MOSI, SPI3_NSS, USART5_TX, ENET_MII_TXD1, ENET_RMII_TXD1, EXMC_A25, EVENTOUT |
|  | VSS | D7 | P | - | Default: VSS |
|  | VDD | C7 | P | - | Default: VDD |
|  | PG15 | B7 | I/O | 5VT | Default: PG15 Alternate: USART5_CTS, EXMC_SDNCAS, DCI_D13, EVENTOUT |
|  | PB3 | A10 | I/O | 5VT | Default: JTDO, PB3 Alternate: TRACESWO, TIMER1_CH1, SPI0_SCK, SPI2_SCK, I2S2_CK, USART0_RX, I2C1_SDA, EVENTOUT |
|  | PB4 | A9 | I/O | 5VT | Default: JNTRST, PB4 Alternate: TIMER2_CH0, SPI0_MISO, SPI2_MISO, I2S2_ADD_SD, I2C2_SDA, SDIO_D0, EVENTOUT, I2C0_TXFRAME |
|  | PB5 | A6 | I/O | 5VT | Default: PB5 |

27

GD32F470xx Datasheet

|  | GD32F470Ix BGA176 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Alternate: TIMER2_CH1, I2C0_SMBA, SPI0_MOSI, SPI2_MOSI, I2S2_SD, CAN1_RX, USBHS_ULPI_D7, ENET_PPS_OUT, EXMC_SDCKE1, DCI_D10, EVENTOUT |
|  | PB6 | B6 | I/O | 5VT | Default: PB6 Alternate: TIMER3_CH0, I2C0_SCL, USART0_TX, CAN1_TX, EXMC_SDNE1, DCI_D5, EVENTOUT |
|  | PB7 | B5 | I/O | 5VT | Default: PB7 Alternate: TIMER3_CH1, I2C0_SDA, USART0_RX, EXMC_NL/EXMC_NADV, DCI_VSYNC, EVENTOUT |
|  | BOOT0 | D6 | I/O | 5VT | Default: BOOT0 |
|  | PB8 | A5 | I/O | 5VT | Default: PB8 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER3_CH2, TIMER9_CH0, I2C0_SCL, SPI4_MOSI, CAN0_RX, ENET_MII_TXD3, SDIO_D4, DCI_D6, TLI_B6, EVENTOUT |
|  | PB9 | B4 | I/O | 5VT | Default: PB9 Alternate: TIMER1_CH1, TIMER3_CH3, TIMER10_CH0, I2C0_SDA, SPI1_NSS, I2S1_WS, CAN0_TX, SDIO_D5, DCI_D7, TLI_B7, EVENTOUT |
|  | PE0 | A4 | I/O | 5VT | Default: PE0 Alternate: TIMER3_ETI, UART7_RX, EXMC_NBL0, DCI_D2, EVENTOUT |
|  | PE1 | A3 | I/O | 5VT | Default: PE1 Alternate: TIMER0_CH1_ON, UART7_TX, EXMC_NBL1, DCI_D3, EVENTOUT |
|  | VSS | D5 | P | - | Default: VSS |
|  | PDR_ON | C6 | P | - | Default: PDR_ON(3) |
|  | VDD | C5 | P | - | Default: VDD |
|  | PI4 | D4 | I/O | 5VT | Default: PI4 Alternate: TIMER7_BRKIN, EXMC_NBL2, DCI_D5, TLI_B4, EVENTOUT |
|  | PI5 | C4 | I/O | 5VT | Default: PI5 Alternate: TIMER7_CH0, EXMC_NBL3, DCI_VSYNC, TLI_B5, EVENTOUT |
|  | PI6 | C3 | I/O | 5VT | Default: PI6 Alternate: TIMER7_CH1, EXMC_D28, DCI_D6, TLI_B6, EVENTOUT |
|  | PI7 | C2 | I/O | 5VT | Default: PI7 Alternate: TIMER7_CH2, EXMC_D29, DCI_D7, TLI_B7, EVENTOUT |

Notes:

(1)  Type: I = input, O = output, P = power.

(2)  I/O Level: 5VT = 5 V tolerant.

(3)  PDR_ON pin·should·be·pulled-up·to·VDD, refer to Figure 4-4. Recommended PDR_ON

pin circuit.

28

GD32F470xx Datasheet

2.6.2.

GD32F470Zx LQFP144 pin definitions

Table 2-4. GD32F470Zx LQFP144 pin definitions

| GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
| PE2 | 1 | I/O | 5VT | Default: PE2 Alternate: SPI3_SCK, ENET_MII_TXD3, EXMC_A23, EVENTOUT |
| PE3 | 2 | I/O | 5VT | Default: PE3 Alternate: EXMC_A19, EVENTOUT |
| PE4 | 3 | I/O | 5VT | Default: PE4 Alternate: SPI3_NSS, EXMC_A20, DCI_D4, TLI_B0, EVENTOUT |
| PE5 | 4 | I/O | 5VT | Default: PE5 Alternate: TIMER8_CH0, SPI3_MISO, EXMC_A21, DCI_D6, TLI_G0, EVENTOUT |
| PE6 | 5 | I/O | 5VT | Default: PE6 Alternate: TIMER8_CH1, SPI3_MOSI, EXMC_A22, DCI_D7, TLI_G1, EVENTOUT |
| VBAT | 6 | P | - | Default: VBAT |
| PC13- TAMPER- RTC | 7 | I/O | 5VT | Default: PC13 Alternate: EVENTOUT Additional: RTC_TAMP0, RTC_OUT, RTC_TS |
| PC14- OSC32IN | 8 | I/O | 5VT | Default: PC14 Alternate: EVENTOUT Additional: OSC32IN |
| PC15- OSC32OU T | 9 | I/O | 5VT | Default: PC15 Alternate: EVENTOUT Additional: OSC32OUT |
| PF0 | 10 | I/O | 5VT | Default: PF0 Alternate: I2C1_SDA, EXMC_A0, EVENTOUT, CTC_SYNC |
| PF1 | 11 | I/O | 5VT | Default: PF1 Alternate: I2C1_SCL, EXMC_A1, EVENTOUT |
| PF2 | 12 | I/O | 5VT | Default: PF2 Alternate: I2C1_SMBA, EXMC_A2, EVENTOUT |
| PF3 | 13 | I/O | 5VT | Default: PF3 Alternate: EXMC_A3, EVENTOUT, I2C1_TXFRAME Additional: ADC2_IN9 |
| PF4 | 14 | I/O | 5VT | Default: PF4 Alternate: EXMC_A4, EVENTOUT Additional: ADC2_IN14 |
| PF5 | 15 | I/O | 5VT | Default: PF5 Alternate: EXMC_A5, EVENTOUT Additional: ADC2_IN15 |
| VSS | 16 | P | - | Default: VSS |

29

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | VDD | 17 | P | - | Default: VDD |
|  | PF6 | 18 | I/O | 5VT | Default: PF6 Alternate: TIMER9_CH0, SPI4_NSS, UART6_RX, EXMC_NIORD, EVENTOUT Additional: ADC2_IN4 |
|  | PF7 | 19 | I/O | 5VT | Default: PF7 Alternate: TIMER10_CH0, SPI4_SCK, UART6_TX, EXMC_NREG, EVENTOUT Additional: ADC2_IN5 |
|  | PF8 | 20 | I/O | 5VT | Default: PF8 Alternate: SPI4_MISO, TIMER12_CH0, EXMC_NIOWR, EVENTOUT Additional: ADC2_IN6 |
|  | PF9 | 21 | I/O | 5VT | Default: PF9 Alternate: SPI4_MOSI, TIMER13_CH0, EXMC_CD, EVENTOUT Additional: ADC2_IN7 |
|  | PF10 | 22 | I/O | 5VT | Default: PF10 Alternate: EXMC_INTR, DCI_D11, TLI_DE, EVENTOUT Additional: ADC2_IN8 |
|  | PH0/OSCI N | 23 | I/O | 5VT | Default: PH0, OSCIN Alternate: EVENTOUT Additional: OSCIN |
|  | PH1/OSCO UT | 24 | I/O | 5VT | Default: PH1, OSCOUT Alternate: EVENTOUT Additional: OSCOUT |
|  | NRST | 25 | - | - | Default: NRST |
|  | PC0 | 26 | I/O | 5VT | Default: PC0 Alternate: USBHS_ULPI_STP, EXMC_SDNWE, EVENTOUT Additional: ADC012_IN10 |
|  | PC1 | 27 | I/O | 5VT | Default: PC1 Alternate: SPI2_MOSI, I2S2_SD, SPI1_MOSI, I2S1_SD, ENET_MDC, EVENTOUT Additional: ADC012_IN11 |
|  | PC2 | 28 | I/O | 5VT | Default: PC2 Alternate: SPI1_MISO, I2S1_ADD_SD, USBHS_ULPI_DIR, ENET_MII_TXD2, EXMC_SDNE0, EVENTOUT Additional: ADC012_IN12 |
|  | PC3 | 29 | I/O | 5VT | Default: PC3 Alternate: SPI1_MOSI, I2S1_SD, USBHS_ULPI_NXT, ENET_MII_TX_CLK, EXMC_SDCKE0, EVENTOUT Additional: ADC012_IN13 |
|  | VDD | 30 | P | - | Default: VDD |

30

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | VSSA | 31 | P | - | Default: VSSA |
|  | VREFP | 32 | P | - | Default: VREFP |
|  | VDDA | 33 | P | - | Default: VDDA |
|  | PA0-WKUP | 34 | I/O | 5VT | Default: PA0 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER4_CH0, TIMER7_ETI, USART1_CTS, UART3_TX, ENET_MII_CRS, EVENTOUT Additional: ADC012_IN0, WKUP |
|  | PA1 | 35 | I/O | 5VT | Default: PA1 Alternate: TIMER1_CH1, TIMER4_CH1, SPI3_MOSI, USART1_RTS, UART3_RX, ENET_MII_RX_CLK, ENET_RMII_REF_CLK, EVENTOUT Additional: ADC012_IN1 |
|  | PA2 | 36 | I/O | 5VT | Default: PA2 Alternate: TIMER1_CH2, TIMER4_CH2, TIMER8_CH0, I2S_CKIN, USART1_TX, ENET_MDIO, EVENTOUT Additional: ADC012_IN2 |
|  | PA3 | 37 | I/O | 5VT | Default: PA3 Alternate: TIMER1_CH3, TIMER4_CH3, TIMER8_CH1, I2S1_MCK, USART1_RX, USBHS_ULPI_D0, ENET_MII_COL, TLI_B5, EVENTOUT Additional: ADC012_IN3 |
|  | VSS | 38 | P | - | Default: VSS |
|  | VDD | 39 | P | - | Default: VDD |
|  | PA4 | 40 | I/O |  | Default: PA4 Alternate: SPI0_NSS, SPI2_NSS, I2S2_WS, USART1_CK, USBHS_SOF, DCI_HSYNC, TLI_VSYNC, EVENTOUT Additional: ADC01_IN4, DAC0_OUT0 |
|  | PA5 | 41 | I/O |  | Default: PA5 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER7_CH0_ON, SPI0_SCK, USBHS_ULPI_CK, EVENTOUT Additional: ADC01_IN5, DAC0_OUT1 |
|  | PA6 | 42 | I/O | 5VT | Default: PA6 Alternate: TIMER0_BRKIN, TIMER2_CH0, TIMER7_BRKIN, SPI0_MISO, I2S1_MCK, TIMER12_CH0, SDIO_CMD, DCI_PIXCLK, TLI_G2, EVENTOUT Additional: ADC01_IN6 |
|  | PA7 | 43 | I/O | 5VT | Default: PA7 Alternate: TIMER0_CH0_ON, TIMER2_CH1, TIMER7_CH0_ON, SPI0_MOSI, TIMER13_CH0, ENET_MII_RX_DV, ENET_RMII_CRS_DV, EXMC_SDNWE, EVENTOUT Additional: ADC01_IN7 |
|  | PC4 | 44 | I/O | 5VT | Default: PC4 |

31

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Alternate: ENET_MII_RXD0, ENET_RMII_RXD0, EXMC_SDNE0, EVENTOUT Additional: ADC01_IN14 |
|  | PC5 | 45 | I/O | 5VT | Default: PC5 Alternate: USART2_RX, ENET_MII_RXD1, ENET_RMII_RXD1, EXMC_SDCKE0, EVENTOUT Additional: ADC01_IN15 |
|  | PB0 | 46 | I/O | 5VT | Default: PB0 Alternate: TIMER0_CH1_ON, TIMER2_CH2, TIMER7_CH1_ON, SPI4_SCK, SPI2_MOSI, I2S2_SD, TLI_R3, USBHS_ULPI_D1, ENET_MII_RXD2, SDIO_D1, EVENTOUT Additional: ADC01_IN8, IREF |
|  | PB1 | 47 | I/O | 5VT | Default: PB1 Alternate: TIMER0_CH2_ON, TIMER2_CH3, TIMER7_CH2_ON, SPI4_NSS, TLI_R6, USBHS_ULPI_D2, ENET_MII_RXD3, SDIO_D2, EVENTOUT Additional: ADC01_IN9 |
|  | PB2 | 48 | I/O | 5VT | Default: PB2, BOOT1 Alternate: TIMER1_CH3, SPI2_MOSI, I2S2_SD, USBHS_ULPI_D4, SDIO_CK, EVENTOUT |
|  | PF11 | 49 | I/O | 5VT | Default: PF11 Alternate: SPI4_MOSI, EXMC_SDNRAS, DCI_D12, EVENTOUT |
|  | PF12 | 50 | I/O | 5VT | Default: PF12 Alternate: EXMC_A6, EVENTOUT |
|  | VSS | 51 | P | - | Default: VSS |
|  | VDD | 52 | P | - | Default: VDD |
|  | PF13 | 53 | I/O | 5VT | Default: PF13 Alternate: EXMC_A7, EVENTOUT |
|  | PF14 | 54 | I/O | 5VT | Default: PF14 Alternate: EXMC_A8, EVENTOUT |
|  | PF15 | 55 | I/O | 5VT | Default: PF15 Alternate: EXMC_A9, EVENTOUT |
|  | PG0 | 56 | I/O | 5VT | Default: PG0 Alternate: EXMC_A10, EVENTOUT |
|  | PG1 | 57 | I/O | 5VT | Default: PG1 Alternate: EXMC_A11, EVENTOUT |
|  | PE7 | 58 | I/O | 5VT | Default: PE7 Alternate: TIMER0_ETI, UART6_RX, EXMC_D4, EVENTOUT |
|  | PE8 | 59 | I/O | 5VT | Default: PE8 Alternate: TIMER0_CH0_ON, UART6_TX, EXMC_D5, EVENTOUT |

32

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | PE9 | 60 | I/O | 5VT | Default: PE9 Alternate: TIMER0_CH0, EXMC_D6, EVENTOUT |
|  | VSS | 61 | P | - | Default: VSS |
|  | VDD | 62 | P | - | Default: VDD |
|  | PE10 | 63 | I/O | 5VT | Default: PE10 Alternate: TIMER0_CH1_ON, EXMC_D7, EVENTOUT |
|  | PE11 | 64 | I/O | 5VT | Default: PE11 Alternate: TIMER0_CH1, SPI3_NSS, SPI4_NSS, EXMC_D8, TLI_G3, EVENTOUT |
|  | PE12 | 65 | I/O | 5VT | Default: PE12 Alternate: TIMER0_CH2_ON, SPI3_SCK, SPI4_SCK, EXMC_D9, TLI_B4, EVENTOUT |
|  | PE13 | 66 | I/O | 5VT | Default: PE13 Alternate: TIMER0_CH2, SPI3_MISO, SPI4_MISO, EXMC_D10, TLI_DE, EVENTOUT |
|  | PE14 | 67 | I/O | 5VT | Default: PE14 Alternate: TIMER0_CH3, SPI3_MOSI, SPI4_MOSI, EXMC_D11, TLI_PIXCLK, EVENTOUT |
|  | PE15 | 68 | I/O | 5VT | Default: PE15 Alternate: TIMER0_BRKIN, EXMC_D12, TLI_R7, EVENTOUT |
|  | PB10 | 69 | I/O | 5VT | Default: PB10 Alternate: TIMER1_CH2, I2C1_SCL, SPI1_SCK, I2S1_CK, I2S2_MCK, USART2_TX, USBHS_ULPI_D3, ENET_MII_RX_ER, SDIO_D7, TLI_G4, EVENTOUT |
|  | PB11 | 70 | I/O | 5VT | Default: PB11 Alternate: TIMER1_CH3, I2C1_SDA, I2S_CKIN, USART2_RX, USBHS_ULPI_D4, ENET_MII_TX_EN, ENET_RMII_TX_EN, TLI_G5, EVENTOUT |
|  | NC | 71 | - | - | - |
|  | VDD | 72 | P | - | Default: VDD |
|  | PB12 | 73 | I/O | 5VT | Default: PB12 Alternate: TIMER0_BRKIN, I2C1_SMBA, SPI1_NSS, I2S1_WS, SPI3_NSS, USART2_CK, CAN1_RX, USBHS_ULPI_D5, ENET_MII_TXD0, ENET_RMII_TXD0, USBHS_ID, EVENTOUT |
|  | PB13 | 74 | I/O | 5VT | Default: PB13 Alternate: TIMER0_CH0_ON, SPI1_SCK, I2S1_CK, SPI3_SCK, USART2_CTS, CAN1_TX, USBHS_ULPI_D6, ENET_MII_TXD1, ENET_RMII_TXD1, EVENTOUT, I2C1_TXFRAME Additional: USBHS_VBUS |
|  | PB14 | 75 | I/O | 5VT | Default: PB14 Alternate: TIMER0_CH1_ON, TIMER7_CH1_ON, |

33

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | SPI1_MISO, I2S1_ADD_SD, USART2_RTS, TIMER11_CH0, USBHS_DM, EVENTOUT |
|  | PB15 | 76 | I/O | 5VT | Default: PB15 Alternate: RTC_REFIN, TIMER0_CH2_ON, TIMER7_CH2_ON, SPI1_MOSI, I2S1_SD, TIMER11_CH1, USBHS_DP, EVENTOUT |
|  | PD8 | 77 | I/O | 5VT | Default: PD8 Alternate: USART2_TX, EXMC_D13, EVENTOUT |
|  | PD9 | 78 | I/O | 5VT | Default: PD9 Alternate: USART2_RX, EXMC_D14, EVENTOUT |
|  | PD10 | 79 | I/O | 5VT | Default: PD10 Alternate: USART2_CK, EXMC_D15, TLI_B3, EVENTOUT |
|  | PD11 | 80 | I/O | 5VT | Default: PD11 Alternate: USART2_CTS, EXMC_A16/EXMC_CLE, EVENTOUT |
|  | PD12 | 81 | I/O | 5VT | Default: PD12 Alternate: TIMER3_CH0, USART2_RTS, EXMC_A17/EXMC_ALE, EVENTOUT |
|  | PD13 | 82 | I/O | 5VT | Default: PD13 Alternate: TIMER3_CH1, EXMC_A18, EVENTOUT |
|  | VSS | 83 | P | - | Default: VSS |
|  | VDD | 84 | P | - | Default: VDD |
|  | PD14 | 85 | I/O | 5VT | Default: PD14 Alternate: TIMER3_CH2, EXMC_D0, EVENTOUT |
|  | PD15 | 86 | I/O | 5VT | Default: PD15 Alternate: TIMER3_CH3, EXMC_D1, EVENTOUT, CTC_SYNC |
|  | PG2 | 87 | I/O | 5VT | Default: PG2 Alternate: EXMC_A12, EVENTOUT |
|  | PG3 | 88 | I/O | 5VT | Default: PG3 Alternate: EXMC_A13, EVENTOUT |
|  | PG4 | 89 | I/O | 5VT | Default: PG4 Alternate: EXMC_A14, EVENTOUT |
|  | PG5 | 90 | I/O | 5VT | Default: PG5 Alternate: EXMC_A15, EVENTOUT |
|  | PG6 | 91 | I/O | 5VT | Default: PG6 Alternate: EXMC_INT1, DCI_D12, TLI_R7, EVENTOUT |
|  | PG7 | 92 | I/O | 5VT | Default: PG7 Alternate: USART5_CK, EXMC_INT2, DCI_D13, TLI_PIXCLK, EVENTOUT |
|  | PG8 | 93 | I/O | 5VT | Default: PG8 Alternate: SPI5_NSS, USART5_RTS, ENET_PPS_OUT, EXMC_SDCLK, EVENTOUT |
|  | VSS | 94 | P | - | Default: VSS |

34

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | VDD | 95 | P | - | Default: VDD |
|  | PC6 | 96 | I/O | 5VT | Default: PC6 Alternate: TIMER2_CH0, TIMER7_CH0, I2S1_MCK, USART5_TX, SDIO_D6, DCI_D0, TLI_HSYNC, EVENTOUT |
|  | PC7 | 97 | I/O | 5VT | Default: PC7 Alternate: TIMER2_CH1, TIMER7_CH1, SPI1_SCK, I2S1_CK, I2S2_MCK, USART5_RX, SDIO_D7, DCI_D1, TLI_G6, EVENTOUT |
|  | PC8 | 98 | I/O | 5VT | Default: PC8 Alternate: TIMER2_CH2, TIMER7_CH2, USART5_CK, SDIO_D0, DCI_D2, EVENTOUT |
|  | PC9 | 99 | I/O | 5VT | Default: PC9 Alternate: CK_OUT1, TIMER2_CH3, TIMER7_CH3, I2C2_SDA, I2S_CKIN, SDIO_D1, DCI_D3, EVENTOUT |
|  | PA8 | 100 | I/O | 5VT | Default: PA8 Alternate: CK_OUT0, TIMER0_CH0, I2C2_SCL, USART0_CK, USBFS_SOF, SDIO_D1, TLI_R6, EVENTOUT, CTC_SYNC |
|  | PA9 | 101 | I/O | 5VT | Default: PA9 Alternate: TIMER0_CH1, I2C2_SMBA, SPI1_SCK, I2S1_CK, USART0_TX, SDIO_D2, DCI_D0, EVENTOUT Additional: USBFS_VBUS |
|  | PA10 | 102 | I/O | 5VT | Default: PA10 Alternate: TIMER0_CH2, SPI4_MOSI, USART0_RX, USBFS_ID, DCI_D1, EVENTOUT, I2C2_TXFRAME |
|  | PA11 | 103 | I/O | 5VT | Default: PA11 Alternate: TIMER0_CH3, SPI3_MISO, USART0_CTS, USART5_TX, CAN0_RX, USBFS_DM, TLI_R4, EVENTOUT |
|  | PA12 | 104 | I/O | 5VT | Default: PA12 Alternate: TIMER0_ETI, SPI4_MISO, USART0_RTS, USART5_RX, CAN0_TX, USBFS_DP, TLI_R5, EVENTOUT |
|  | PA13 | 105 | I/O | 5VT | Default: JTMS, SWDIO, PA13 Alternate: EVENTOUT |
|  | NC | 106 | - | - | - |
|  | VSS | 107 | P | - | Default: VSS |
|  | VDD | 108 | P | - | Default: VDD |
|  | PA14 | 109 | I/O | 5VT | Default: JTCK, SWCLK, PA14 Alternate: EVENTOUT |
|  | PA15 | 110 | I/O | 5VT | Default: JTDI, PA15 Alternate: TIMER1_CH0, TIMER1_ETI, SPI0_NSS, SPI2_NSS, I2S2_WS, USART0_TX, EVENTOUT |

35

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | PC10 | 111 | I/O | 5VT | Default: PC10 Alternate: SPI2_SCK, I2S2_CK, USART2_TX, UART3_TX, SDIO_D2, DCI_D8, TLI_R2, EVENTOUT |
|  | PC11 | 112 | I/O | 5VT | Default: PC11 Alternate: I2S2_ADD_SD, SPI2_MISO, USART2_RX, UART3_RX, SDIO_D3, DCI_D4, EVENTOUT |
|  | PC12 | 113 | I/O | 5VT | Default: PC12 Alternate: I2C1_SDA, SPI2_MOSI, I2S2_SD, USART2_CK, UART4_TX, SDIO_CK, DCI_D9, EVENTOUT |
|  | PD0 | 114 | I/O | 5VT | Default: PD0 Alternate: SPI3_MISO, SPI2_MOSI, I2S2_SD, CAN0_RX, EXMC_D2, EVENTOUT |
|  | PD1 | 115 | I/O | 5VT | Default: PD1 Alternate: SPI1_NSS, I2S1_WS, CAN0_TX, EXMC_D3, EVENTOUT |
|  | PD2 | 116 | I/O | 5VT | Default: PD2 Alternate: TIMER2_ETI, UART4_RX, SDIO_CMD, DCI_D11, EVENTOUT |
|  | PD3 | 117 | I/O | 5VT | Default: PD3 Alternate: SPI1_SCK, I2S1_CK, USART1_CTS, EXMC_CLK, DCI_D5, TLI_G7, EVENTOUT |
|  | PD4 | 118 | I/O | 5VT | Default: PD4 Alternate: USART1_RTS, EXMC_NOE, EVENTOUT |
|  | PD5 | 119 | I/O | 5VT | Default: PD5 Alternate: USART1_TX, EXMC_NWE, EVENTOUT |
|  | VSS | 120 | P | - | Default: VSS |
|  | VDD | 121 | P | - | Default: VDD |
|  | PD6 | 122 | I/O | 5VT | Default: PD6 Alternate: SPI2_MOSI, I2S2_SD, USART1_RX, EXMC_NWAIT, DCI_D10, TLI_B2, EVENTOUT |
|  | PD7 | 123 | I/O | 5VT | Default: PD7 Alternate: USART1_CK, EXMC_NE0, EXMC_NCE1, EVENTOUT |
|  | PG9 | 124 | I/O | 5VT | Default: PG9 Alternate: USART5_RX, EXMC_NE1, EXMC_NCE2, DCI_VSYNC, EVENTOUT |
|  | PG10 | 125 | I/O | 5VT | Default: PG10 Alternate: SPI5_IO2, TLI_G3, EXMC_NCE3_0, EXMC_NE2, DCI_D2, TLI_B2, EVENTOUT |
|  | PG11 | 126 | I/O | 5VT | Default: PG11 Alternate: SPI5_IO3, SPI3_SCK, ENET_MII_TX_EN, ENET_RMII_TX_EN, EXMC_NCE3_1, DCI_D3, TLI_B3, EVENTOUT |
|  | PG12 | 127 | I/O | 5VT | Default: PG12 |

36

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Alternate: SPI5_MISO, SPI3_MISO, USART5_RTS, TLI_B4, EXMC_NE3, TLI_B1, EVENTOUT |
|  | PG13 | 128 | I/O | 5VT | Default: PG13 Alternate: SPI5_SCK, SPI3_MOSI, USART5_CTS, ENET_MII_TXD0, ENET_RMII_TXD0, EXMC_A24, EVENTOUT |
|  | PG14 | 129 | I/O | 5VT | Default: PG14 Alternate: SPI5_MOSI, SPI3_NSS, USART5_TX, ENET_MII_TXD1, ENET_RMII_TXD1, EXMC_A25, EVENTOUT |
|  | VSS | 130 | P | - | Default: VSS |
|  | VDD | 131 | P | - | Default: VDD |
|  | PG15 | 132 | I/O | 5VT | Default: PG15 Alternate: USART5_CTS, EXMC_SDNCAS, DCI_D13, EVENTOUT |
|  | PB3 | 133 | I/O | 5VT | Default: JTDO, PB3 Alternate: TRACESWO, TIMER1_CH1, SPI0_SCK, SPI2_SCK, I2S2_CK, USART0_RX, I2C1_SDA, EVENTOUT |
|  | PB4 | 134 | I/O | 5VT | Default: NJTRST, PB4 Alternate:TIMER2_CH0, SPI0_MISO, SPI2_MISO, I2S2_ADD_SD, I2C2_SDA, SDIO_D0, EVENTOUT, I2C0_TXFRAME |
|  | PB5 | 135 | I/O | 5VT | Default: PB5 Alternate:TIMER2_CH1, I2C0_SMBA, SPI0_MOSI, SPI2_MOSI, I2S2_SD, CAN1_RX, USBHS_ULPI_D7, ENET_PPS_OUT, EXMC_SDCKE1, DCI_D10, EVENTOUT |
|  | PB6 | 136 | I/O | 5VT | Default: PB6 Alternate:TIMER3_CH0, I2C0_SCL, USART0_TX, CAN1_TX, EXMC_SDNE1, DCI_D5, EVENTOUT |
|  | PB7 | 137 | I/O | 5VT | Default: PB7 Alternate:TIMER3_CH1, I2C0_SDA, USART0_RX, EXMC_NL/EXMC_NADV, DCI_VSYNC, EVENTOUT |
|  | BOOT0 | 138 | I/O | 5VT | Default: BOOT0 |
|  | PB8 | 139 | I/O | 5VT | Default: PB8 Alternate:TIMER1_CH0, TIMER1_ETI, TIMER3_CH2, TIMER9_CH0, I2C0_SCL, SPI4_MOSI, CAN0_RX, ENET_MII_TXD3, SDIO_D4, DCI_D6, TLI_B6, EVENTOUT |
|  | PB9 | 140 | I/O | 5VT | Default: PB9 Alternate:TIMER1_CH1, TIMER3_CH3, TIMER10_CH0, I2C0_SDA, SPI1_NSS, I2S1_WS, CAN0_TX, SDIO_D5, DCI_D7, TLI_B7, EVENTOUT |
|  | PE0 | 141 | I/O | 5VT | Default: PE0 Alternate: TIMER3_ETI, UART7_RX, EXMC_NBL0, |

37

GD32F470xx Datasheet

|  | GD32F470Zx LQFP144 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | DCI_D2, EVENTOUT |
|  | PE1 | 142 | I/O | 5VT | Default: PE1 Alternate: TIMER0_CH1_ON, UART7_TX, EXMC_NBL1, DCI_D3, EVENTOUT |
|  | PDR_ON | 143 | P | - | Default: PDR_ON(3) |
|  | VDD | 144 | P | - | Default: VDD |

Notes:

(1)  Type: I = input, O = output, P = power.

(2)  I/O Level: 5VT = 5 V tolerant.

(3)  PDR_ON pin·should·be·pulled-up·to·VDD, refer to Figure 4-4. Recommended PDR_ON

pin circuit.

2.6.3.

GD32F470Vx BGA100 pin definitions

Table 2-5. GD32F470Vx BGA100 pin definitions

| GD32F470Vx BGA100 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
| PE2 | B2 | I/O | 5VT | Default: PE2 Alternate: SPI3_SCK, ETH_MII_TXD3, EXMC_A23, EVENTOUT |
| PE3 | A1 | I/O | 5VT | Default: PE3 Alternate: EXMC_A19, EVENTOUT |
| PE4 | B1 | I/O | 5VT | Default: PE4 Alternate: SPI3_NSS, EXMC_A20, DCI_D4, TLI_B0, EVENTOUT |
| PE5 | C2 | I/O | 5VT | Default: PE5 Alternate: TIMER8_CH0, SPI3_MISO, EXMC_A21, DCI_D6, TLI_G0, EVENTOUT |
| PE6 | D2 | I/O | 5VT | Default: PE6 Alternate: TIMER8_CH1, SPI3_MOSI, EXMC_A22, DCI_D7, TLI_G1, EVENTOUT |
| VBAT | E2 | P | - | Default: VBAT |
| PC13- TAMPER- RTC | C1 | I/O | 5VT | Default: PC13 Alternate: EVENTOUT Additional: RTC_TAMP0, RTC_OUT, RTC_TS |
| PC14- OSC32IN | D1 | I/O | 5VT | Default: PC14 Alternate: EVENTOUT Additional: OSC32IN |
| PC15- OSC32OU T | E1 | I/O | 5VT | Default: PC15 Alternate: EVENTOUT Additional: OSC32OUT |

38

GD32F470xx Datasheet

|  | GD32F470Vx BGA100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | VSS | F2 | P | - | Default: VSS |
|  | VDD | G2 | P | - | Default: VDD |
|  | PH0/OSCI N | F1 | I/O | 5VT | Default: PH0, OSCIN Alternate: EVENTOUT Additional: OSCIN |
|  | PH1/OSCO UT | G1 | I/O | 5VT | Default: PH1, OSCOUT Alternate: EVENTOUT Additional: OSCOUT |
|  | NRST | H2 | - | - | Default: NRST |
|  | PC0 | H1 | I/O | 5VT | Default: PC0 Alternate: USBHS_ULPI_STP, EVENTOUT Additional: ADC012_IN10 |
|  | PC1 | J2 | I/O | 5VT | Default: PC1 Alternate: SPI2_MOSI, I2S2_SD, SPI1_MOSI, I2S1_SD, ETH_MDC, EVENTOUT Additional: ADC012_IN11 |
|  | PC2 | J3 | I/O | 5VT | Default: PC2 Alternate: SPI1_MISO, I2S1_ADD_SD, USBHS_ULPI_DIR, ETH_MII_TXD2, EVENTOUT Additional: ADC012_IN12 |
|  | PC3 | K2 | I/O | 5VT | Default: PC3 Alternate: SPI1_MOSI, I2S1_SD, USBHS_ULPI_NXT, ETH_MII_TX_CLK, EXMC_SDCKE0, EVENTOUT Additional: ADC012_IN13 |
|  | VSSA | J1 | P | - | Default: VSSA |
|  | VREFN | K1 | P | - | Default: VREFN |
|  | VREFP | L1 | P | - | Default: VREFP |
|  | VDDA | M1 | P | - | Default: VDDA |
|  | PA0-WKUP | L2 | I/O | 5VT | Default: PA0 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER4_CH0, TIMER7_ETI, USART1_CTS, UART3_TX, ETH_MII_CRS, EVENTOUT Additional: ADC012_IN0, WKUP |
|  | PA1 | M2 | I/O | 5VT | Default: PA1 Alternate: TIMER1_CH1, TIMER4_CH1, SPI3_MOSI, USART1_RTS, UART3_RX, ETH_MII_RX_CLK, ETH_RMII_REF_CLK, EVENTOUT Additional: ADC012_IN1 |
|  | PA2 | K3 | I/O | 5VT | Default: PA2 Alternate: TIMER1_CH2, TIMER4_CH2, TIMER8_CH0, I2S_CKIN, USART1_TX, ETH_MDIO, EVENTOUT Additional: ADC012_IN2 |
|  | PA3 | L3 | I/O | 5VT | Default: PA3 Alternate: TIMER1_CH3, TIMER4_CH3, TIMER8_CH1, |

39

GD32F470xx Datasheet

|  | GD32F470Vx BGA100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | I2S1_MCK, USART1_RX, USBHS_ULPI_D0, ETH_MII_COL, TLI_B5, EVENTOUT Additional: ADC012_IN3 |
|  | NC | E3 | - | - | - |
|  | PA4 | M3 | I/O | TTa | Default: PA4 Alternate: SPI0_NSS, SPI2_NSS, I2S2_WS, USART1_CK, USBHS_SOF, DCI_HSYNC, TLI_VSYNC, EVENTOUT Additional: ADC01_IN4, DAC0_OUT0 |
|  | PA5 | K4 | I/O | TTa | Default: PA5 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER7_CH0_ON, SPI0_SCK, USBHS_ULPI_CK, EVENTOUT Additional: ADC01_IN5, DAC0_OUT1 |
|  | PA6 | L4 | I/O | 5VT | Default: PA6 Alternate: TIMER0_BRKIN, TIMER2_CH0, TIMER7_BRKIN, SPI0_MISO, I2S1_MCK, TIMER12_CH0, SDIO_CMD, DCI_PIXCLK, TLI_G2, EVENTOUT Additional: ADC01_IN6 |
|  | PA7 | M4 | I/O | 5VT | Default: PA7 Alternate: TIMER0_CH0_ON, TIMER2_CH1, TIMER7_CH0_ON, SPI0_MOSI, TIMER13_CH0, ETH_MII_RX_DV, ETH_RMII_CRS_DV, EVENTOUT Additional: ADC01_IN7 |
|  | PC4 | K5 | I/O | 5VT | Default: PC4 Alternate: ETH_MII_RXD0, ETH_RMII_RXD0, EVENTOUT Additional: ADC01_IN14 |
|  | PC5 | L5 | I/O | 5VT | Default: PC5 Alternate: USART2_RX, ETH_MII_RXD1, ETH_RMII_RXD1, EXMC_SDCKE0, EVENTOUT Additional: ADC01_IN15 |
|  | PB0 | M5 | I/O | 5VT | Default: PB0 Alternate: TIMER0_CH1_ON, TIMER2_CH2, TIMER7_CH1_ON, SPI4_SCK, SPI2_MOSI, I2S2_SD, TLI_R3, USBHS_ULPI_D1, ETH_MII_RXD2, SDIO_D1, EVENTOUT Additional: ADC01_IN8, IREF |
|  | PB1 | M6 | I/O | 5VT | Default: PB1 Alternate: TIMER0_CH2_ON, TIMER2_CH3, TIMER7_CH2_ON, SPI4_NSS, TLI_R6, USBHS_ULPI_D2, ETH_MII_RXD3, SDIO_D2, EVENTOUT Additional: ADC01_IN9 |
|  | PB2 | L6 | I/O | 5VT | Default: PB2, BOOT1 Alternate:TIMER1_CH3, SPI2_MOSI, I2S2_SD, USBHS_ULPI_D4, SDIO_CK, EVENTOUT |
|  | PE7 | M7 | I/O | 5VT | Default: PE7 Alternate: TIMER0_ETI, UART6_RX, EXMC_D4, |

40

GD32F470xx Datasheet

|  | GD32F470Vx BGA100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | EVENTOUT |
|  | PE8 | L7 | I/O | 5VT | Default: PE8 Alternate: TIMER0_CH0_ON, UART6_TX, EXMC_D5, EVENTOUT |
|  | PE9 | M8 | I/O | 5VT | Default: PE9 Alternate: TIMER0_CH0, EXMC_D6, EVENTOUT |
|  | PE10 | L8 | I/O | 5VT | Default: PE10 Alternate: TIMER0_CH1_ON, EXMC_D7, EVENTOUT |
|  | PE11 | M9 | I/O | 5VT | Default: PE11 Alternate:TIMER0_CH1, SPI3_NSS, SPI4_NSS, TLI_G3, EVENTOUT |
|  | PE12 | L9 | I/O | 5VT | Default: PE12 Alternate:TIMER0_CH2_ON, SPI3_SCK, SPI4_SCK, TLI_B4, EVENTOUT |
|  | PE13 | M10 | I/O | 5VT | Default: PE13 Alternate:TIMER0_CH2, SPI3_MISO, SPI4_MISO, TLI_DE, EVENTOUT |
|  | PE14 | M11 | I/O | 5VT | Default: PE14 Alternate:TIMER0_CH3, SPI3_MOSI, SPI4_MOSI, TLI_PIXCLK, EVENTOUT |
|  | PE15 | M12 | I/O | 5VT | Default: PE15 Alternate: TIMER0_BRKIN, TLI_R7, EVENTOUT |
|  | PB10 | L10 | I/O | 5VT | Default: PB10 Alternate:TIMER1_CH2, I2C1_SCL, SPI1_SCK, I2S1_CK, I2S2_MCK, USART2_TX, USBHS_ULPI_D3, ETH_MII_RX_ER, SDIO_D7, TLI_G4, EVENTOUT |
|  | PB11 | K9 | I/O | 5VT | Default: PB11 Alternate:TIMER1_CH3, I2C1_SDA, I2S_CKIN, USART2_RX, USBHS_ULPI_D4, ETH_MII_TX_EN, ETH_RMII_TX_EN, TLI_G5, EVENTOUT |
|  | NC | L11 | P | - | - |
|  | VSS | F12 | P | - | Default: VSS |
|  | VDD | G12 | P | - | Default: VDD |
|  | PB12 | L12 | I/O | 5VT | Default: PB12 Alternate:TIMER0_BRKIN, I2C1_SMBA, SPI1_NSS, I2S1_WS, SPI3_NSS, USART2_CK, CAN1_RX, USBHS_ULPI_D5, ETH_MII_TXD0, ETH_RMII_TXD0, USBHS_ID, EVENTOUT |
|  | PB13 | K12 | I/O | 5VT | Default: PB13 Alternate: TIMER0_CH0_ON, SPI1_SCK, I2S1_CK, SPI3_SCK, USART2_CTS, CAN1_TX, USBHS_ULPI_D6, ETH_MII_TXD1, ETH_RMII_TXD1, EVENTOUT, I2C1_TXFRAME Additional: USBHS_VBUS |

41

GD32F470xx Datasheet

|  | GD32F470Vx BGA100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | PB14 | K11 | I/O | 5VT | Default: PB14 Alternate:TIMER0_CH1_ON, TIMER7_CH1_ON, SPI1_MISO, I2S1_ADD_SD, USART2_RTS, TIMER11_CH0, USBHS_DM, EVENTOUT |
|  | PB15 | K10 | I/O | 5VT | Default: PB15 Alternate: RTC_REFIN, TIMER0_CH2_ON, TIMER7_CH2_ON, SPI1_MOSI, I2S1_SD, TIMER11_CH1, USBHS_DP, EVENTOUT |
|  | PD9 | K8 | I/O | 5VT | Default: PD9 Alternate: USART2_RX, EVENTOUT |
|  | PD10 | J12 | I/O | 5VT | Default: PD10 Alternate: USART2_CK, TLI_B3, EVENTOUT |
|  | PD11 | J11 | I/O | 5VT | Default: PD11 Alternate: USART2_CTS, EXMC_A16/EXMC_CLE, EVENTOUT |
|  | PD12 | J10 | I/O | 5VT | Default: PD12 Alternate: TIMER3_CH0, USART2_RTS, EXMC_A17/EXMC_ALE, EVENTOUT |
|  | PD13 | H12 | I/O | 5VT | Default: PD13 Alternate: TIMER3_CH1, EXMC_A18, EVENTOUT |
|  | PD14 | H11 | I/O | 5VT | Default: PD14 Alternate: TIMER3_CH2, EXMC_D0, EVENTOUT |
|  | PD15 | H10 | I/O | 5VT | Default: PD15 Alternate: TIMER3_CH3, EXMC_D1, EVENTOUT, CTC_SYNC |
|  | PC6 | E12 | I/O | 5VT | Default: PC6 Alternate:TIMER2_CH0, TIMER7_CH0, I2S1_MCK, USART5_TX, SDIO_D6, DCI_D0, TLI_HSYNC, EVENTOUT |
|  | PC7 | E11 | I/O | 5VT | Default: PC7 Alternate:TIMER2_CH1, TIMER7_CH1, SPI1_SCK, I2S1_CK, I2S2_MCK, USART5_RX, SDIO_D7, DCI_D1, TLI_G6, EVENTOUT |
|  | PC8 | E10 | I/O | 5VT | Default: PC8 Alternate: TIMER2_CH2, TIMER7_CH2, USART5_CK, SDIO_D0, DCI_D2, EVENTOUT |
|  | PC9 | D12 | I/O | 5VT | Default: PC9 Alternate:CK_OUT1, TIMER2_CH3, TIMER7_CH3, I2C2_SDA, I2S_CKIN, SDIO_D1, DCI_D3, EVENTOUT |
|  | PA8 | D11 | I/O | 5VT | Default: PA8 Alternate: CK_OUT0, TIMER0_CH0, I2C2_SCL, USART0_CK, USBFS_SOF, SDIO_D1, TLI_R6, EVENTOUT, CTC_SYNC |
|  | PA9 | D10 | I/O | 5VT | Default: PA9 Alternate: TIMER0_CH1, I2C2_SMBA, SPI1_SCK, I2S1_CK, USART0_TX, SDIO_D2, DCI_D0, EVENTOUT |

42

GD32F470xx Datasheet

|  | GD32F470Vx BGA100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Additional: USBFS_VBUS |
|  | PA10 | C12 | I/O | 5VT | Default: PA10 Alternate:TIMER0_CH2, SPI4_MOSI, USART0_RX, USBFS_ID, DCI_D1, EVENTOUT, I2C2_TXFRAME |
|  | PA11 | B12 | I/O | 5VT | Default: PA11 Alternate:TIMER0_CH3, SPI3_MISO, USART0_CTS, USART5_TX, CAN0_RX, USBFS_DM, TLI_R4, EVENTOUT |
|  | PA12 | A12 | I/O | 5VT | Default: PA12 Alternate:TIMER0_ETI, SPI4_MISO, USART0_RTS, USART5_RX, CAN0_TX, USBFS_DP, TLI_R5, EVENTOUT |
|  | PA13 | A11 | I/O | 5VT | Default: JTMS, SWDIO, PA13 Alternate: EVENTOUT |
|  | NC | C11 | - | - | - |
|  | VSS | F11 | P | - | Default: VSS |
|  | VDD | G11 | P | - | Default: VDD |
|  | PA14 | A10 | I/O | 5VT | Default: JTCK, SWCLK, PA14 Alternate: EVENTOUT |
|  | PA15 | A9 | I/O | 5VT | Default: JTDI, PA15 Alternate:TIMER1_CH0, TIMER1_ETI, SPI0_NSS, SPI2_NSS, I2S2_WS, USART0_TX, EVENTOUT |
|  | PC10 | B11 | I/O | 5VT | Default: PC10 Alternate:SPI2_SCK, I2S2_CK, USART2_TX, UART3_TX, SDIO_D2, DCI_D8, TLI_R2, EVENTOUT |
|  | PC11 | C10 | I/O | 5VT | Default: PC11 Alternate:I2S2_ADD_SD, SPI2_MISO, USART2_RX, UART3_RX, SDIO_D3, DCI_D4, EVENTOUT |
|  | PC12 | B10 | I/O | 5VT | Default: PC12 Alternate:I2C1_SDA, SPI2_MOSI, I2S2_SD, USART2_CK, UART4_TX, SDIO_CK, DCI_D9, EVENTOUT |
|  | PD0 | C9 | I/O | 5VT | Default: PD0 Alternate:SPI3_MISO, SPI2_MOSI, I2S2_SD, CAN0_RX, EXMC_D2, EVENTOUT |
|  | PD1 | B9 | I/O | 5VT | Default: PD1 Alternate: SPI1_NSS, I2S1_WS, CAN0_TX, EXMC_D3, EVENTOUT |
|  | PD2 | C8 | I/O | 5VT | Default: PD2 Alternate: TIMER2_ETI, UART4_RX, SDIO_CMD, DCI_D11, EVENTOUT |
|  | PD3 | B8 | I/O | 5VT | Default: PD3 Alternate: SPI1_SCK, I2S1_CK, USART1_CTS, EXMC_CLK, DCI_D5, TLI_G7, EVENTOUT |
|  | PD4 | B7 | I/O | 5VT | Default: PD4 Alternate: USART1_RTS, EXMC_NOE, EVENTOUT |
|  | PD5 | A6 | I/O | 5VT | Default: PD5 |

43

GD32F470xx Datasheet

|  | GD32F470Vx BGA100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Alternate: USART1_TX, EXMC_NWE, EVENTOUT |
|  | PD6 | B6 | I/O | 5VT | Default: PD6 Alternate:SPI2_MOSI, I2S2_SD, USART1_RX, EXMC_NWAIT, DCI_D10, TLI_B2, EVENTOUT |
|  | PD7 | A5 | I/O | 5VT | Default: PD7 Alternate: USART1_CK, EXMC_NE0, EXMC_NCE1, EVENTOUT |
|  | PB3 | A8 | I/O | 5VT | Default: JTDO, PB3 Alternate: TRACESWO, TIMER1_CH1, SPI0_SCK, SPI2_SCK, I2S2_CK, USART0_RX, I2C1_SDA, EVENTOUT |
|  | PB4 | A7 | I/O | 5VT | Default: JNTRST, PB4 Alternate:TIMER2_CH0, SPI0_MISO, SPI2_MISO, I2S2_ADD_SD, I2C2_SDA, SDIO_D0, EVENTOUT, I2C0_TXFRAME |
|  | PB5 | C5 | I/O | 5VT | Default: PB5 Alternate:TIMER2_CH1, I2C0_SMBA, SPI0_MOSI, SPI2_MOSI, I2S2_SD, CAN1_RX, USBHS_ULPI_D7, ETH_PPS_OUT, EXMC_SDCKE1, DCI_D10, EVENTOUT |
|  | PB6 | B5 | I/O | 5VT | Default: PB6 Alternate:TIMER3_CH0, I2C0_SCL, USART0_TX, CAN1_TX, DCI_D5, EVENTOUT |
|  | PB7 | B4 | I/O | 5VT | Default: PB7 Alternate:TIMER3_CH1, I2C0_SDA, USART0_RX, EXMC_NL/EXMC_NADV, DCI_VSYNC, EVENTOUT |
|  | BOOT0 | A4 | I/O | 5VT | Default: BOOT0 |
|  | PB8 | A3 | I/O | 5VT | Default: PB8 Alternate:TIMER1_CH0, TIMER1_ETI, TIMER3_CH2, TIMER9_CH0, I2C0_SCL, SPI4_MOSI, CAN0_RX, ETH_MII_TXD3, SDIO_D4, DCI_D6, TLI_B6, EVENTOUT |
|  | PB9 | B3 | I/O | 5VT | Default: PB9 Alternate:TIMER1_CH1, TIMER3_CH3, TIMER10_CH0, I2C0_SDA, SPI1_NSS, I2S1_WS, CAN0_TX, SDIO_D5, DCI_D7, TLI_B7, EVENTOUT |
|  | PE0 | C3 | I/O | 5VT | Default: PE0 Alternate: TIMER3_ETI, UART7_RX, EXMC_NBL0, DCI_D2, EVENTOUT |
|  | PE1 | A2 | I/O | 5VT | Default: PE1 Alternate: TIMER0_CH1_ON, UART7_TX, EXMC_NBL1, DCI_D3, EVENTOUT |
|  | VSS | D3 | P | - | Default: VSS |
|  | PDR_ON | H3 | P | - | Default: PDR_ON(3) |
|  | VDD | C4 | P | - | Default: VDD |

Notes:

(1)  Type: I = input, O = output, P = power.

44

GD32F470xx Datasheet

(2)  I/O Level: 5VT = 5 V tolerant.

(3)  PDR_ON pin·should·be·pulled-up·to·VDD, refer to Figure 4-4. Recommended PDR_ON

pin circuit.

2.6.4.

GD32F470Vx LQFP100 pin definitions

Table 2-6. GD32F470Vx LQFP100 pin definitions

| GD32F470Vx LQFP100 |  |  |  |  |
| --- | --- | --- | --- | --- |
| Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
| PE2 | 1 | I/O | 5VT | Default: PE2 Alternate: SPI3_SCK, ENET_MII_TXD3, EXMC_A23, EVENTOUT |
| PE3 | 2 | I/O | 5VT | Default: PE3 Alternate: EXMC_A19, EVENTOUT |
| PE4 | 3 | I/O | 5VT | Default: PE4 Alternate: SPI3_NSS, EXMC_A20, DCI_D4, TLI_B0, EVENTOUT |
| PE5 | 4 | I/O | 5VT | Default: PE5 Alternate: TIMER8_CH0, SPI3_MISO, EXMC_A21, DCI_D6, TLI_G0, EVENTOUT |
| PE6 | 5 | I/O | 5VT | Default: PE6 Alternate: TIMER8_CH1, SPI3_MOSI, EXMC_A22, DCI_D7, TLI_G1, EVENTOUT |
| VBAT | 6 | P | - | Default: VBAT |
| PC13- TAMPER- RTC | 7 | I/O | 5VT | Default: PC13 Alternate: EVENTOUT Additional: RTC_TAMP0, RTC_OUT, RTC_TS |
| PC14- OSC32IN | 8 | I/O | 5VT | Default: PC14 Alternate: EVENTOUT Additional: OSC32IN |
| PC15- OSC32OU T | 9 | I/O | 5VT | Default: PC15 Alternate: EVENTOUT Additional: OSC32OUT |
| VSS | 10 | P | - | Default: VSS |
| VDD | 11 | P | - | Default: VDD |
| PH0/OSCI N | 12 | I/O | 5VT | Default: PH0, OSCIN Alternate: EVENTOUT Additional: OSCIN |
| PH1/OSC OUT | 13 | I/O | 5VT | Default: PH1, OSCOUT Alternate: EVENTOUT Additional: OSCOUT |
| NRST | 14 | - | - | Default: NRST |
| PC0 | 15 | I/O | 5VT | Default: PC0 Alternate: USBHS_ULPI_STP, EVENTOUT Additional: ADC012_IN10 |

45

GD32F470xx Datasheet

|  | GD32F470Vx LQFP100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | PC1 | 16 | I/O | 5VT | Default: PC1 Alternate: SPI2_MOSI, I2S2_SD, SPI1_MOSI, I2S1_SD, ENET_MDC, EVENTOUT Additional: ADC012_IN11 |
|  | PC2 | 17 | I/O | 5VT | Default: PC2 Alternate: SPI1_MISO, I2S1_ADD_SD, USBHS_ULPI_DIR, ENET_MII_TXD2, EVENTOUT Additional: ADC012_IN12 |
|  | PC3 | 18 | I/O | 5VT | Default: PC3 Alternate: SPI1_MOSI, I2S1_SD, USBHS_ULPI_NXT, ENET_MII_TX_CLK, EVENTOUT Additional: ADC012_IN13 |
|  | VDD | 19 | P | - | Default: VDD |
|  | VSSA | 20 | P | - | Default: VSSA |
|  | VREFP | 21 | P | - | Default: VREFP |
|  | VDDA | 22 | P | - | Default: VDDA |
|  | PA0- WKUP | 23 | I/O | 5VT | Default: PA0 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER4_CH0, TIMER7_ETI, USART1_CTS, UART3_TX, ENET_MII_CRS, EVENTOUT Additional: ADC012_IN0, WKUP |
|  | PA1 | 24 | I/O | 5VT | Default: PA1 Alternate: TIMER1_CH1, TIMER4_CH1, SPI3_MOSI, USART1_RTS, UART3_RX, ENET_MII_RX_CLK, ENET_RMII_REF_CLK, EVENTOUT Additional: ADC012_IN1 |
|  | PA2 | 25 | I/O | 5VT | Default: PA2 Alternate: TIMER1_CH2, TIMER4_CH2, TIMER8_CH0, I2S_CKIN, USART1_TX, ENET_MDIO, EVENTOUT Additional: ADC012_IN2 |
|  | PA3 | 26 | I/O | 5VT | Default: PA3 Alternate: TIMER1_CH3, TIMER4_CH3, TIMER8_CH1, I2S1_MCK, USART1_RX, USBHS_ULPI_D0, ENET_MII_COL, TLI_B5, EVENTOUT Additional: ADC012_IN3 |
|  | VSS | 27 | P | - | Default: VSS |
|  | VDD | 28 | P | - | Default: VDD |
|  | PA4 | 29 | I/O |  | Default: PA4 Alternate: SPI0_NSS, SPI2_NSS, I2S2_WS, USART1_CK, USBHS_SOF, DCI_HSYNC, TLI_VSYNC, EVENTOUT Additional: ADC01_IN4, DAC0_OUT0 |
|  | PA5 | 30 | I/O |  | Default: PA5 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER7_CH0_ON, SPI0_SCK, USBHS_ULPI_CK, EVENTOUT |

46

GD32F470xx Datasheet

|  | GD32F470Vx LQFP100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Additional: ADC01_IN5, DAC0_OUT1 |
|  | PA6 | 31 | I/O | 5VT | Default: PA6 Alternate: TIMER0_BRKIN, TIMER2_CH0, TIMER7_BRKIN, SPI0_MISO, I2S1_MCK, TIMER12_CH0, SDIO_CMD, DCI_PIXCLK, TLI_G2, EVENTOUT Additional: ADC01_IN6 |
|  | PA7 | 32 | I/O | 5VT | Default: PA7 Alternate: TIMER0_CH0_ON, TIMER2_CH1, TIMER7_CH0_ON, SPI0_MOSI, TIMER13_CH0, ENET_MII_RX_DV, ENET_RMII_CRS_DV, EVENTOUT Additional: ADC01_IN7 |
|  | PC4 | 33 | I/O | 5VT | Default: PC4 Alternate: ENET_MII_RXD0, ENET_RMII_RXD0, EVENTOUT Additional: ADC01_IN14 |
|  | PC5 | 34 | I/O | 5VT | Default: PC5 Alternate: USART2_RX, ENET_MII_RXD1, ENET_RMII_RXD1, EVENTOUT Additional: ADC01_IN15 |
|  | PB0 | 35 | I/O | 5VT | Default: PB0 Alternate: TIMER0_CH1_ON, TIMER2_CH2, TIMER7_CH1_ON, SPI4_SCK, SPI2_MOSI, I2S2_SD, TLI_R3, USBHS_ULPI_D1, ENET_MII_RXD2, SDIO_D1, EVENTOUT Additional: ADC01_IN8, IREF |
|  | PB1 | 36 | I/O | 5VT | Default: PB1 Alternate: TIMER0_CH2_ON, TIMER2_CH3, TIMER7_CH2_ON, SPI4_NSS, TLI_R6, USBHS_ULPI_D2, ENET_MII_RXD3, SDIO_D2, EVENTOUT Additional: ADC01_IN9 |
|  | PB2 | 37 | I/O | 5VT | Default: PB2, BOOT1 Alternate: TIMER1_CH3, SPI2_MOSI, I2S2_SD, USBHS_ULPI_D4, SDIO_CK, EVENTOUT |
|  | PE7 | 38 | I/O | 5VT | Default: PE7 Alternate: TIMER0_ETI, UART6_RX, EXMC_D4, EVENTOUT |
|  | PE8 | 39 | I/O | 5VT | Default: PE8 Alternate: TIMER0_CH0_ON, UART6_TX, EXMC_D5, EVENTOUT |
|  | PE9 | 40 | I/O | 5VT | Default: PE9 Alternate: TIMER0_CH0, EXMC_D6, EVENTOUT |
|  | PE10 | 41 | I/O | 5VT | Default: PE10 Alternate: TIMER0_CH1_ON, EXMC_D7, EVENTOUT |
|  | PE11 | 42 | I/O | 5VT | Default: PE11 Alternate: TIMER0_CH1, SPI3_NSS, SPI4_NSS, |

47

GD32F470xx Datasheet

|  | GD32F470Vx LQFP100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | EXMC_D8, TLI_G3, EVENTOUT |
|  | PE12 | 43 | I/O | 5VT | Default: PE12 Alternate: TIMER0_CH2_ON, SPI3_SCK, SPI4_SCK, EXMC_D9, TLI_B4, EVENTOUT |
|  | PE13 | 44 | I/O | 5VT | Default: PE13 Alternate: TIMER0_CH2, SPI3_MISO, SPI4_MISO, EXMC_D10, TLI_DE, EVENTOUT |
|  | PE14 | 45 | I/O | 5VT | Default: PE14 Alternate: TIMER0_CH3, SPI3_MOSI, SPI4_MOSI, EXMC_D11, TLI_PIXCLK, EVENTOUT |
|  | PE15 | 46 | I/O | 5VT | Default: PE15 Alternate: TIMER0_BRKIN, EXMC_D12, TLI_R7, EVENTOUT |
|  | PB10 | 47 | I/O | 5VT | Default: PB10 Alternate: TIMER1_CH2, I2C1_SCL, SPI1_SCK, I2S1_CK, I2S2_MCK, USART2_TX, USBHS_ULPI_D3, ENET_MII_RX_ER, SDIO_D7, TLI_G4, EVENTOUT |
|  | PB11 | 48 | I/O | 5VT | Default: PB11 Alternate: TIMER1_CH3, I2C1_SDA, I2S_CKIN, USART2_RX, USBHS_ULPI_D4, ENET_MII_TX_EN, ENET_RMII_TX_EN, TLI_G5, EVENTOUT |
|  | NC | 49 | - | - | - |
|  | VDD | 50 | P | - | Default: VDD |
|  | PB12 | 51 | I/O | 5VT | Default: PB12 Alternate: TIMER0_BRKIN, I2C1_SMBA, SPI1_NSS, I2S1_WS, SPI3_NSS, USART2_CK, CAN1_RX, USBHS_ULPI_D5, ENET_MII_TXD0, ENET_RMII_TXD0, USBHS_ID, EVENTOUT |
|  | PB13 | 52 | I/O | 5VT | Default: PB13 Alternate: TIMER0_CH0_ON, SPI1_SCK, I2S1_CK, SPI3_SCK, USART2_CTS, CAN1_TX, USBHS_ULPI_D6, ENET_MII_TXD1, ENET_RMII_TXD1, EVENTOUT, I2C1_TXFRAME Additional: USBHS_VBUS |
|  | PB14 | 53 | I/O | 5VT | Default: PB14 Alternate: TIMER0_CH1_ON, TIMER7_CH1_ON, SPI1_MISO, I2S1_ADD_SD, USART2_RTS, TIMER11_CH0, USBHS_DM, EVENTOUT |
|  | PB15 | 54 | I/O | 5VT | Default: PB15 Alternate: RTC_REFIN, TIMER0_CH2_ON, TIMER7_CH2_ON, SPI1_MOSI, I2S1_SD, TIMER11_CH1, USBHS_DP, EVENTOUT |
|  | PD8 | 55 | I/O | 5VT | Default: PD8 Alternate: USART2_TX, EXMC_D13, EVENTOUT |
|  | PD9 | 56 | I/O | 5VT | Default: PD9 |

48

GD32F470xx Datasheet

|  | GD32F470Vx LQFP100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | Alternate: USART2_RX, EXMC_D14, EVENTOUT |
|  | PD10 | 57 | I/O | 5VT | Default: PD10 Alternate: USART2_CK, EXMC_D15, TLI_B3, EVENTOUT |
|  | PD11 | 58 | I/O | 5VT | Default: PD11 Alternate: USART2_CTS, EXMC_A16/EXMC_CLE, EVENTOUT |
|  | PD12 | 59 | I/O | 5VT | Default: PD12 Alternate: TIMER3_CH0, USART2_RTS, EXMC_A17/EXMC_ALE, EVENTOUT |
|  | PD13 | 60 | I/O | 5VT | Default: PD13 Alternate: TIMER3_CH1, EXMC_A18, EVENTOUT |
|  | PD14 | 61 | I/O | 5VT | Default: PD14 Alternate: TIMER3_CH2, EXMC_D0, EVENTOUT |
|  | PD15 | 62 | I/O | 5VT | Default: PD15 Alternate: TIMER3_CH3, EXMC_D1, EVENTOUT, CTC_SYNC |
|  | PC6 | 63 | I/O | 5VT | Default: PC6 Alternate: TIMER2_CH0, TIMER7_CH0, I2S1_MCK, USART5_TX, SDIO_D6, DCI_D0, TLI_HSYNC, EVENTOUT |
|  | PC7 | 64 | I/O | 5VT | Default: PC7 Alternate: TIMER2_CH1, TIMER7_CH1, SPI1_SCK, I2S1_CK, I2S2_MCK, USART5_RX, SDIO_D7, DCI_D1, TLI_G6, EVENTOUT |
|  | PC8 | 65 | I/O | 5VT | Default: PC8 Alternate: TIMER2_CH2, TIMER7_CH2, USART5_CK, SDIO_D0, DCI_D2, EVENTOUT |
|  | PC9 | 66 | I/O | 5VT | Default: PC9 Alternate: CK_OUT1, TIMER2_CH3, TIMER7_CH3, I2C2_SDA, I2S_CKIN, SDIO_D1, DCI_D3, EVENTOUT |
|  | PA8 | 67 | I/O | 5VT | Default: PA8 Alternate: CK_OUT0, TIMER0_CH0, I2C2_SCL, USART0_CK, USBFS_SOF, SDIO_D1, TLI_R6, EVENTOUT, CTC_SYNC |
|  | PA9 | 68 | I/O | 5VT | Default: PA9 Alternate: TIMER0_CH1, I2C2_SMBA, SPI1_SCK, I2S1_CK, USART0_TX, SDIO_D2, DCI_D0, EVENTOUT Additional: USBFS_VBUS |
|  | PA10 | 69 | I/O | 5VT | Default: PA10 Alternate: TIMER0_CH2, SPI4_MOSI, USART0_RX, USBFS_ID, DCI_D1, EVENTOUT, I2C2_TXFRAME |
|  | PA11 | 70 | I/O | 5VT | Default: PA11 Alternate: TIMER0_CH3, SPI3_MISO, USART0_CTS, USART5_TX, CAN0_RX, USBFS_DM, TLI_R4, EVENTOUT |

49

GD32F470xx Datasheet

|  | GD32F470Vx LQFP100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  | PA12 | 71 | I/O | 5VT | Default: PA12 Alternate: TIMER0_ETI, SPI4_MISO, USART0_RTS, USART5_RX, CAN0_TX, USBFS_DP, TLI_R5, EVENTOUT |
|  | PA13 | 72 | I/O | 5VT | Default: JTMS, SWDIO, PA13 Alternate: EVENTOUT |
|  | NC | 73 | - | - | - |
|  | VSS | 74 | P | - | Default: VSS |
|  | VDD | 75 | P | - | Default: VDD |
|  | PA14 | 76 | I/O | 5VT | Default: JTCK, SWCLK, PA14 Alternate: EVENTOUT |
|  | PA15 | 77 | I/O | 5VT | Default: JTDI, PA15 Alternate: TIMER1_CH0, TIMER1_ETI, SPI0_NSS, SPI2_NSS, I2S2_WS, USART0_TX, EVENTOUT |
|  | PC10 | 78 | I/O | 5VT | Default: PC10 Alternate: SPI2_SCK, I2S2_CK, USART2_TX, UART3_TX, SDIO_D2, DCI_D8, TLI_R2, EVENTOUT |
|  | PC11 | 79 | I/O | 5VT | Default: PC11 Alternate: I2S2_ADD_SD, SPI2_MISO, USART2_RX, UART3_RX, SDIO_D3, DCI_D4, EVENTOUT |
|  | PC12 | 80 | I/O | 5VT | Default: PC12 Alternate: I2C1_SDA, SPI2_MOSI, I2S2_SD, USART2_CK, UART4_TX, SDIO_CK, DCI_D9, EVENTOUT |
|  | PD0 | 81 | I/O | 5VT | Default: PD0 Alternate: SPI3_MISO, SPI2_MOSI, I2S2_SD, CAN0_RX, EXMC_D2, EVENTOUT |
|  | PD1 | 82 | I/O | 5VT | Default: PD1 Alternate: SPI1_NSS, I2S1_WS, CAN0_TX, EXMC_D3, EVENTOUT |
|  | PD2 | 83 | I/O | 5VT | Default: PD2 Alternate: TIMER2_ETI, UART4_RX, SDIO_CMD, DCI_D11, EVENTOUT |
|  | PD3 | 84 | I/O | 5VT | Default: PD3 Alternate: SPI1_SCK, I2S1_CK, USART1_CTS, EXMC_CLK, DCI_D5, TLI_G7, EVENTOUT |
|  | PD4 | 85 | I/O | 5VT | Default: PD4 Alternate: USART1_RTS, EXMC_NOE, EVENTOUT |
|  | PD5 | 86 | I/O | 5VT | Default: PD5 Alternate: USART1_TX, EXMC_NWE, EVENTOUT |
|  | PD6 | 87 | I/O | 5VT | Default: PD6 Alternate: SPI2_MOSI, I2S2_SD, USART1_RX, EXMC_NWAIT, DCI_D10, TLI_B2, EVENTOUT |
|  | PD7 | 88 | I/O | 5VT | Default: PD7 Alternate: USART1_CK, EXMC_NE0, EXMC_NCE1, |

50

GD32F470xx Datasheet

|  | GD32F470Vx LQFP100 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Pin Name | Pins | Pin Type(1) | I/O Level(2) | Functions description |
|  |  |  |  |  | EVENTOUT |
|  | PB3 | 89 | I/O | 5VT | Default: JTDO, PB3 Alternate: TRACESWO, TIMER1_CH1, SPI0_SCK, SPI2_SCK, I2S2_CK, USART0_RX, I2C1_SDA, EVENTOUT |
|  | PB4 | 90 | I/O | 5VT | Default: JNTRST, PB4 Alternate: TIMER2_CH0, SPI0_MISO, SPI2_MISO, I2S2_ADD_SD, I2C2_SDA, SDIO_D0, EVENTOUT, I2C0_TXFRAME |
|  | PB5 | 91 | I/O | 5VT | Default: PB5 Alternate: TIMER2_CH1, I2C0_SMBA, SPI0_MOSI, SPI2_MOSI, I2S2_SD, CAN1_RX, USBHS_ULPI_D7, ENET_PPS_OUT, DCI_D10, EVENTOUT |
|  | PB6 | 92 | I/O | 5VT | Default: PB6 Alternate: TIMER3_CH0, I2C0_SCL, USART0_TX, CAN1_TX, DCI_D5, EVENTOUT |
|  | PB7 | 93 | I/O | 5VT | Default: PB7 Alternate: TIMER3_CH1, I2C0_SDA, USART0_RX, EXMC_NL/EXMC_NADV, DCI_VSYNC, EVENTOUT |
|  | BOOT0 | 94 | I/O | 5VT | Default: BOOT0 |
|  | PB8 | 95 | I/O | 5VT | Default: PB8 Alternate: TIMER1_CH0, TIMER1_ETI, TIMER3_CH2, TIMER9_CH0, I2C0_SCL, SPI4_MOSI, CAN0_RX, ENET_MII_TXD3, SDIO_D4, DCI_D6, TLI_B6, EVENTOUT |
|  | PB9 | 96 | I/O | 5VT | Default: PB9 Alternate: TIMER1_CH1, TIMER3_CH3, TIMER10_CH0, I2C0_SDA, SPI1_NSS, I2S1_WS, CAN0_TX, SDIO_D5, DCI_D7, TLI_B7, EVENTOUT |
|  | PE0 | 97 | I/O | 5VT | Default: PE0 Alternate: TIMER3_ETI, UART7_RX, EXMC_NBL0, DCI_D2, EVENTOUT |
|  | PE1 | 98 | I/O | 5VT | Default: PE1 Alternate: TIMER0_CH1_ON, UART7_TX, EXMC_NBL1, DCI_D3, EVENTOUT |
|  | VSS | 99 | P | - | Default: VSS |
|  | VDD | 100 | P | - | Default: VDD |

Notes:

(1)  Type: I = input, O = output, P = power.

(2)  I/O Level: 5VT = 5 V tolerant.

51

GD32F470xx Datasheet

2.6.5.

GD32F470xx pin alternate functions

Table 2-7. Port A alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PA0 |  | TIMER1_C H0/TIMER 1_ETI | TIMER4_C H0 | TIMER7_E TI |  |  |  | USART1_ CTS | UART3_TX |  |  | ENET_MII _CRS |  |  |  | EVENTOU T |
| PA1 |  | TIMER1_C H1 | TIMER4_C H1 |  |  | SPI3_MOS I |  | USART1_ RTS | UART3_R X |  |  | ENET_MII _RX_CLK/ ENET_RMI I_REF_CL K |  |  |  | EVENTOU T |
| PA2 |  | TIMER1_C H2 | TIMER4_C H2 | TIMER8_C H0 |  | I2S_CKIN |  | USART1_T X |  |  |  | ENET_MDI O |  |  |  | EVENTOU T |
| PA3 |  | TIMER1_C H3 | TIMER4_C H3 | TIMER8_C H1 |  | I2S1_MCK |  | USART1_ RX |  |  | USBHS_U LPI_D0 | ENET_MII _COL |  |  | TLI_B5 | EVENTOU T |
| PA4 |  |  |  |  |  | SPI0_NSS | SPI2_NSS/ I2S2_WS | USART1_ CK |  |  |  |  | USBHS_S OF | DCI_HSYN C | TLI_VSYN C | EVENTOU T |
| PA5 |  | TIMER1_C H0/TIMER 1_ETI |  | TIMER7_C H0_ON |  | SPI0_SCK |  |  |  |  | USBHS_U LPI_CK |  |  |  |  | EVENTOU T |
| PA6 |  | TIMER0_B RKIN | TIMER2_C H0 | TIMER7_B RKIN |  | SPI0_MIS O | I2S1_MCK |  |  | TIMER12_ CH0 |  |  | SDIO_CM D | DCI_PIXC LK | TLI_G2 | EVENTOU T |
| PA7 |  | TIMER0_C H0_ON | TIMER2_C H1 | TIMER7_C H0_ON |  | SPI0_MOS I |  |  |  | TIMER13_ CH0 |  | ENET_MII _RX_DV/E NET_RMII _CRS_DV | EXMC_SD NWE |  |  | EVENTOU T |
| PA8 | CK_OUT0 | TIMER0_C H0 |  |  | I2C2_SCL |  |  | USART0_ CK |  | CTC_SYN C | USBFS_S OF |  | SDIO_D1 |  | TLI_R6 | EVENTOU T |
| PA9 |  | TIMER0_C H1 |  |  | I2C2_SMB A | SPI1_SCK/ I2S1_CK |  | USART0_T X |  |  |  |  | SDIO_D2 | DCI_D0 |  | EVENTOU T |
| PA10 |  | TIMER0_C H2 |  |  | I2C2_TXF RAME |  | SPI4_MOS I | USART0_ RX |  |  | USBFS_ID |  |  | DCI_D1 |  | EVENTOU T |
| PA11 |  | TIMER0_C H3 |  |  |  |  | SPI3_MIS O | USART0_ CTS | USART5_T X | CAN0_RX | USBFS_D M |  |  |  | TLI_R4 | EVENTOU T |
| PA12 |  | TIMER0_E TI |  |  |  |  | SPI4_MIS O | USART0_ RTS | USART5_ RX | CAN0_TX | USBFS_D P |  |  |  | TLI_R5 | EVENTOU T |
| PA13 | JTMS/SW DIO |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |

52

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PA14 | JTCK/SWC LK |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |
| PA15 | JTDI | TIMER1_C H0/TIMER 1_ETI |  |  |  | SPI0_NSS | SPI2_NSS/ I2S2_WS | USART0_T X |  |  |  |  |  |  |  | EVENTOU T |

Table 2-8. Port B alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB0 |  | TIMER0_C H1_ON | TIMER2_C H2 | TIMER7_C H1_ON |  |  | SPI4_SCK | SPI2_MOS I/I2S2_SD |  | TLI_R3 | USBHS_U LPI_D1 | ENET_MII _RXD2 | SDIO_D1 |  |  | EVENTOU T |
| PB1 |  | TIMER0_C H2_ON | TIMER2_C H3 | TIMER7_C H2_ON |  |  | SPI4_NSS |  |  | TLI_R6 | USBHS_U LPI_D2 | ENET_MII _RXD3 | SDIO_D2 |  |  | EVENTOU T |
| PB2 |  | TIMER1_C H3 |  |  |  |  |  | SPI2_MOS I/I2S2_SD |  |  | USBHS_U LPI_D4 |  | SDIO_CK |  |  | EVENTOU T |
| PB3 | JTDO/TRA CESWO | TIMER1_C H1 |  |  |  | SPI0_SCK | SPI2_SCK/ I2S2_CK | USART0_ RX |  | I2C1_SDA |  |  |  |  |  | EVENTOU T |
| PB4 | NJTRST |  | TIMER2_C H0 |  | I2C0_TXF RAME | SPI0_MIS O | SPI2_MIS O | I2S2_ADD _SD |  | I2C2_SDA |  |  | SDIO_D0 |  |  | EVENTOU T |
| PB5 |  |  | TIMER2_C H1 |  | I2C0_SMB A | SPI0_MOS I | SPI2_MOS I/I2S2_SD |  |  | CAN1_RX | USBHS_U LPI_D7 | ENET_PP S_OUT | EXMC_SD CKE1 | DCI_D10 |  | EVENTOU T |
| PB6 |  |  | TIMER3_C H0 |  | I2C0_SCL |  |  | USART0_T X |  | CAN1_TX |  |  | EXMC_SD NE1 | DCI_D5 |  | EVENTOU T |
| PB7 |  |  | TIMER3_C H1 |  | I2C0_SDA |  |  | USART0_ RX |  |  |  |  | EXMC_NL/ EXMC_NA DV | DCI_VSYN C |  | EVENTOU T |
| PB8 |  | TIMER1_C H0/TIMER 1_ETI | TIMER3_C H2 | TIMER9_C H0 | I2C0_SCL |  | SPI4_MOS I |  |  | CAN0_RX |  | ENET_MII _TXD3 | SDIO_D4 | DCI_D6 | TLI_B6 | EVENTOU T |
| PB9 |  | TIMER1_C H1 | TIMER3_C H3 | TIMER10_ CH0 | I2C0_SDA | SPI1_NSS/ I2S1_WS |  |  |  | CAN0_TX |  |  | SDIO_D5 | DCI_D7 | TLI_B7 | EVENTOU T |
| PB10 |  | TIMER1_C H2 |  |  | I2C1_SCL | SPI1_SCK/ I2S1_CK | I2S2_MCK | USART2_T X |  |  | USBHS_U LPI_D3 | ENET_MII _RX_ER | SDIO_D7 |  | TLI_G4 | EVENTOU T |
| PB11 |  | TIMER1_C H3 |  |  | I2C1_SDA | I2S_CKIN |  | USART2_ RX |  |  | USBHS_U LPI_D4 | ENET_MII _TX_EN/E NET_RMII _TX_EN |  |  | TLI_G5 | EVENTOU T |
| PB12 |  | TIMER0_B RKIN |  |  | I2C1_SMB A | SPI1_NSS/ I2S1_WS | SPI3_NSS | USART2_ CK |  | CAN1_RX | USBHS_U LPI_D5 | ENET_MII _TXD0/EN ET_RMII_T XD0 | USBHS_ID |  |  | EVENTOU T |

53

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PB13 |  | TIMER0_C H0_ON |  |  | I2C1_TXF RAME | SPI1_SCK/ I2S1_CK | SPI3_SCK | USART2_ CTS |  | CAN1_TX | USBHS_U LPI_D6 | ENET_MII _TXD1/EN ET_RMII_T XD1 |  |  |  | EVENTOU T |
| PB14 |  | TIMER0_C H1_ON |  | TIMER7_C H1_ON |  | SPI1_MIS O | I2S1_ADD _SD | USART2_ RTS |  | TIMER11_ CH0 |  |  | USBHS_D M |  |  | EVENTOU T |
| PB15 | RTC_REFI N | TIMER0_C H2_ON |  | TIMER7_C H2_ON |  | SPI1_MOS I/I2S1_SD |  |  |  | TIMER11_ CH1 |  |  | USBHS_D P |  |  | EVENTOU T |

Table 2-9. Port C alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC0 |  |  |  |  |  |  |  |  |  |  | USBHS_U LPI_STP |  | EXMC_SD NWE |  |  | EVENTOU T |
| PC1 |  |  |  |  |  | SPI2_MOS I/I2S2_SD |  | SPI1_MOS I/I2S1_SD |  |  |  | ENET_MD C |  |  |  | EVENTOU T |
| PC2 |  |  |  |  |  | SPI1_MIS O | I2S1_ADD _SD |  |  |  | USBHS_U LPI_DIR | ENET_MII _TXD2 | EXMC_SD NE0 |  |  | EVENTOU T |
| PC3 |  |  |  |  |  | SPI1_MOS I/I2S1_SD |  |  |  |  | USBHS_U LPI_NXT | ENET_MII _TX_CLK | EXMC_SD CKE0 |  |  | EVENTOU T |
| PC4 |  |  |  |  |  |  |  |  |  |  |  | ENET_MII _RXD0/EN ET_RMII_ RXD0 | EXMC_SD NE0 |  |  | EVENTOU T |
| PC5 |  |  |  |  |  |  |  | USART2_ RX |  |  |  | ENET_MII _RXD1/EN ET_RMII_ RXD1 | EXMC_SD CKE0 |  |  | EVENTOU T |
| PC6 |  |  | TIMER2_C H0 | TIMER7_C H0 |  | I2S1_MCK |  |  | USART5_T X |  |  |  | SDIO_D6 | DCI_D0 | TLI_HSYN C | EVENTOU T |
| PC7 |  |  | TIMER2_C H1 | TIMER7_C H1 |  | SPI1_SCK/ I2S1_CK | I2S2_MCK |  | USART5_ RX |  |  |  | SDIO_D7 | DCI_D1 | TLI_G6 | EVENTOU T |
| PC8 |  |  | TIMER2_C H2 | TIMER7_C H2 |  |  |  |  | USART5_ CK |  |  |  | SDIO_D0 | DCI_D2 |  | EVENTOU T |
| PC9 | CK_OUT1 |  | TIMER2_C H3 | TIMER7_C H3 | I2C2_SDA | I2S_CKIN |  |  |  |  |  |  | SDIO_D1 | DCI_D3 |  | EVENTOU T |
| PC10 |  |  |  |  |  |  | SPI2_SCK/ I2S2_CK | USART2_T X | UART3_TX |  |  |  | SDIO_D2 | DCI_D8 | TLI_R2 | EVENTOU T |
| PC11 |  |  |  |  |  | I2S2_ADD _SD | SPI2_MIS O | USART2_ RX | UART3_R X |  |  |  | SDIO_D3 | DCI_D4 |  | EVENTOU T |

54

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PC12 |  |  |  |  | I2C1_SDA |  | SPI2_MOS I/I2S2_SD | USART2_ CK | UART4_TX |  |  |  | SDIO_CK | DCI_D9 |  | EVENTOU T |
| PC13 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |
| PC14 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |
| PC15 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |

Table 2-10. Port D alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PD0 |  |  |  |  |  | SPI3_MIS O | SPI2_MOS I/I2S2_SD |  |  | CAN0_RX |  |  | EXMC_D2 |  |  | EVENTOU T |
| PD1 |  |  |  |  |  |  |  | SPI1_NSS/ I2S1_WS |  | CAN0_TX |  |  | EXMC_D3 |  |  | EVENTOU T |
| PD2 |  |  | TIMER2_E TI |  |  |  |  |  | UART4_R X |  |  |  | SDIO_CM D | DCI_D11 |  | EVENTOU T |
| PD3 |  |  |  |  |  | SPI1_SCK/ I2S1_CK |  | USART1_ CTS |  |  |  |  | EXMC_CL K | DCI_D5 | TLI_G7 | EVENTOU T |
| PD4 |  |  |  |  |  |  |  | USART1_ RTS |  |  |  |  | EXMC_NO E |  |  | EVENTOU T |
| PD5 |  |  |  |  |  |  |  | USART1_T X |  |  |  |  | EXMC_NW E |  |  | EVENTOU T |
| PD6 |  |  |  |  |  | SPI2_MOS I/I2S2_SD |  | USART1_ RX |  |  |  |  | EXMC_NW AIT | DCI_D10 | TLI_B2 | EVENTOU T |
| PD7 |  |  |  |  |  |  |  | USART1_ CK |  |  |  |  | EXMC_NE 0/EXMC_N CE1 |  |  | EVENTOU T |
| PD8 |  |  |  |  |  |  |  | USART2_T X |  |  |  |  | EXMC_D1 3 |  |  | EVENTOU T |
| PD9 |  |  |  |  |  |  |  | USART2_ RX |  |  |  |  | EXMC_D1 4 |  |  | EVENTOU T |
| PD10 |  |  |  |  |  |  |  | USART2_ CK |  |  |  |  | EXMC_D1 5 |  | TLI_B3 | EVENTOU T |
| PD11 |  |  |  |  |  |  |  | USART2_ CTS |  |  |  |  | EXMC_A1 6/EXMC_C LE |  |  | EVENTOU T |

55

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PD12 |  |  | TIMER3_C H0 |  |  |  |  | USART2_ RTS |  |  |  |  | EXMC_A1 7/EXMC_A LE |  |  | EVENTOU T |
| PD13 |  |  | TIMER3_C H1 |  |  |  |  |  |  |  |  |  | EXMC_A1 8 |  |  | EVENTOU T |
| PD14 |  |  | TIMER3_C H2 |  |  |  |  |  |  |  |  |  | EXMC_D0 |  |  | EVENTOU T |
| PD15 | CTC_SYN C |  | TIMER3_C H3 |  |  |  |  |  |  |  |  |  | EXMC_D1 |  |  | EVENTOU T |

Table 2-11. Port E alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PE0 |  |  | TIMER3_E TI |  |  |  |  |  | UART7_R X |  |  |  | EXMC_NB L0 | DCI_D2 |  | EVENTOU T |
| PE1 |  | TIMER0_C H1_ON |  |  |  |  |  |  | UART7_TX |  |  |  | EXMC_NB L1 | DCI_D3 |  | EVENTOU T |
| PE2 |  |  |  |  |  | SPI3_SCK |  |  |  |  |  | ENET_MII _TXD3 | EXMC_A2 3 |  |  | EVENTOU T |
| PE3 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A1 9 |  |  | EVENTOU T |
| PE4 |  |  |  |  |  | SPI3_NSS |  |  |  |  |  |  | EXMC_A2 0 | DCI_D4 | TLI_B0 | EVENTOU T |
| PE5 |  |  |  | TIMER8_C H0 |  | SPI3_MIS O |  |  |  |  |  |  | EXMC_A2 1 | DCI_D6 | TLI_G0 | EVENTOU T |
| PE6 |  |  |  | TIMER8_C H1 |  | SPI3_MOS I |  |  |  |  |  |  | EXMC_A2 2 | DCI_D7 | TLI_G1 | EVENTOU T |
| PE7 |  | TIMER0_E TI |  |  |  |  |  |  | UART6_R X |  |  |  | EXMC_D4 |  |  | EVENTOU T |
| PE8 |  | TIMER0_C H0_ON |  |  |  |  |  |  | UART6_TX |  |  |  | EXMC_D5 |  |  | EVENTOU T |
| PE9 |  | TIMER0_C H0 |  |  |  |  |  |  |  |  |  |  | EXMC_D6 |  |  | EVENTOU T |
| PE10 |  | TIMER0_C H1_ON |  |  |  |  |  |  |  |  |  |  | EXMC_D7 |  |  | EVENTOU T |
| PE11 |  | TIMER0_C H1 |  |  |  | SPI3_NSS | SPI4_NSS |  |  |  |  |  | EXMC_D8 |  | TLI_G3 | EVENTOU T |

56

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PE12 |  | TIMER0_C H2_ON |  |  |  | SPI3_SCK | SPI4_SCK |  |  |  |  |  | EXMC_D9 |  | TLI_B4 | EVENTOU T |
| PE13 |  | TIMER0_C H2 |  |  |  | SPI3_MIS O | SPI4_MIS O |  |  |  |  |  | EXMC_D1 0 |  | TLI_DE | EVENTOU T |
| PE14 |  | TIMER0_C H3 |  |  |  | SPI3_MOS I | SPI4_MOS I |  |  |  |  |  | EXMC_D1 1 |  | TLI_PIXCL K | EVENTOU T |
| PE15 |  | TIMER0_B RKIN |  |  |  |  |  |  |  |  |  |  | EXMC_D1 2 |  | TLI_R7 | EVENTOU T |

Table 2-12. Port F alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF0 | CTC_SYN C |  |  |  | I2C1_SDA |  |  |  |  |  |  |  | EXMC_A0 |  |  | EVENTOU T |
| PF1 |  |  |  |  | I2C1_SCL |  |  |  |  |  |  |  | EXMC_A1 |  |  | EVENTOU T |
| PF2 |  |  |  |  | I2C1_SMB A |  |  |  |  |  |  |  | EXMC_A2 |  |  | EVENTOU T |
| PF3 |  |  |  |  | I2C1_TXF RAME |  |  |  |  |  |  |  | EXMC_A3 |  |  | EVENTOU T |
| PF4 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A4 |  |  | EVENTOU T |
| PF5 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A5 |  |  | EVENTOU T |
| PF6 |  |  |  | TIMER9_C H0 |  | SPI4_NSS |  |  | UART6_R X |  |  |  | EXMC_NI ORD |  |  | EVENTOU T |
| PF7 |  |  |  | TIMER10_ CH0 |  | SPI4_SCK |  |  | UART6_TX |  |  |  | EXMC_NR EG |  |  | EVENTOU T |
| PF8 |  |  |  |  |  | SPI4_MIS O |  |  |  | TIMER12_ CH0 |  |  | EXMC_NI OWR |  |  | EVENTOU T |
| PF9 |  |  |  |  |  | SPI4_MOS I |  |  |  | TIMER13_ CH0 |  |  | EXMC_CD |  |  | EVENTOU T |
| PF10 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_INT R | DCI_D11 | TLI_DE | EVENTOU T |
| PF11 |  |  |  |  |  | SPI4_MOS I |  |  |  |  |  |  | EXMC_SD NRAS | DCI_D12 |  | EVENTOU T |

57

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PF12 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A6 |  |  | EVENTOU T |
| PF13 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A7 |  |  | EVENTOU T |
| PF14 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A8 |  |  | EVENTOU T |
| PF15 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A9 |  |  | EVENTOU T |

Table 2-13. Port G alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PG0 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A1 0 |  |  | EVENTOU T |
| PG1 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A1 1 |  |  | EVENTOU T |
| PG2 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A1 2 |  |  | EVENTOU T |
| PG3 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A1 3 |  |  | EVENTOU T |
| PG4 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A1 4 |  |  | EVENTOU T |
| PG5 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_A1 5 |  |  | EVENTOU T |
| PG6 |  |  |  |  |  |  |  |  |  |  |  |  | EXMC_INT 1 | DCI_D12 | TLI_R7 | EVENTOU T |
| PG7 |  |  |  |  |  |  |  |  | USART5_ CK |  |  |  | EXMC_INT 2 | DCI_D13 | TLI_PIXCL K | EVENTOU T |
| PG8 |  |  |  |  |  | SPI5_NSS |  |  | USART5_ RTS |  |  | ENET_PP S_OUT | EXMC_SD CLK |  |  | EVENTOU T |
| PG9 |  |  |  |  |  |  |  |  | USART5_ RX |  |  |  | EXMC_NE 1/EXMC_N CE2 | DCI_VSYN C |  | EVENTOU T |
| PG10 |  |  |  |  |  | SPI5_IO2 |  |  |  | TLI_G3 |  |  | EXMC_NC E3_0/EXM C_NE2 | DCI_D2 | TLI_B2 | EVENTOU T |
| PG11 |  |  |  |  |  | SPI5_IO3 | SPI3_SCK |  |  |  |  | ENET_MII _TX_EN/E NET_RMII _TX_EN | EXMC_NC E3_1 | DCI_D3 | TLI_B3 | EVENTOU T |

58

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PG12 |  |  |  |  |  | SPI5_MIS O | SPI3_MIS O |  | USART5_ RTS | TLI_B4 |  |  | EXMC_NE 3 |  | TLI_B1 | EVENTOU T |
| PG13 |  |  |  |  |  | SPI5_SCK | SPI3_MOS I |  | USART5_ CTS |  |  | ENET_MII _TXD0/EN ET_RMII_T XD0 | EXMC_A2 4 |  |  | EVENTOU T |
| PG14 |  |  |  |  |  | SPI5_MOS I | SPI3_NSS |  | USART5_T X |  |  | ENET_MII _TXD1/EN ET_RMII_T XD1 | EXMC_A2 5 |  |  | EVENTOU T |
| PG15 |  |  |  |  |  |  |  |  | USART5_ CTS |  |  |  | EXMC_SD NCAS | DCI_D13 |  | EVENTOU T |

Table 2-14. Port H alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PH0 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |
| PH1 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |
| PH2 |  |  |  |  |  |  |  |  |  |  |  | ENET_MII _CRS | EXMC_SD CKE0 |  | TLI_R0 | EVENTOU T |
| PH3 |  |  |  |  | I2C1_TXF RAME |  |  |  |  |  |  | ENET_MII _COL | EXMC_SD NE0 |  | TLI_R1 | EVENTOU T |
| PH4 |  |  |  |  | I2C1_SCL |  |  |  |  |  | USBHS_U LPI_NXT |  |  |  |  | EVENTOU T |
| PH5 |  |  |  |  | I2C1_SDA | SPI4_NSS |  |  |  |  |  |  | EXMC_SD NWE |  |  | EVENTOU T |
| PH6 |  |  |  |  | I2C1_SMB A | SPI4_SCK |  |  |  | TIMER11_ CH0 |  | ENET_MII _RXD2 | EXMC_SD NE1 | DCI_D8 |  | EVENTOU T |
| PH7 |  |  |  |  | I2C2_SCL | SPI4_MIS O |  |  |  |  |  | ENET_MII _RXD3 | EXMC_SD CKE1 | DCI_D9 |  | EVENTOU T |
| PH8 |  |  |  |  | I2C2_SDA |  |  |  |  |  |  |  | EXMC_D1 6 | DCI_HSYN C | TLI_R2 | EVENTOU T |
| PH9 |  |  |  |  | I2C2_SMB A |  |  |  |  | TIMER11_ CH1 |  |  | EXMC_D1 7 | DCI_D0 | TLI_R3 | EVENTOU T |
| PH10 |  |  | TIMER4_C H0 |  | I2C2_TXF RAME |  |  |  |  |  |  |  | EXMC_D1 8 | DCI_D1 | TLI_R4 | EVENTOU T |

59

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PH11 |  |  | TIMER4_C H1 |  |  |  |  |  |  |  |  |  | EXMC_D1 9 | DCI_D2 | TLI_R5 | EVENTOU T |
| PH12 |  |  | TIMER4_C H2 |  |  |  |  |  |  |  |  |  | EXMC_D2 0 | DCI_D3 | TLI_R6 | EVENTOU T |
| PH13 |  |  |  | TIMER7_C H0_ON |  |  |  |  |  | CAN0_TX |  |  | EXMC_D2 1 |  | TLI_G2 | EVENTOU T |
| PH14 |  |  |  | TIMER7_C H1_ON |  |  |  |  |  |  |  |  | EXMC_D2 2 | DCI_D4 | TLI_G3 | EVENTOU T |
| PH15 |  |  |  | TIMER7_C H2_ON |  |  |  |  |  |  |  |  | EXMC_D2 3 | DCI_D11 | TLI_G4 | EVENTOU T |

Table 2-15. Port I alternate functions summary

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PI0 |  |  | TIMER4_C H3 |  |  | SPI1_NSS/ I2S1_WS |  |  |  |  |  |  | EXMC_D2 4 | DCI_D13 | TLI_G5 | EVENTOU T |
| PI1 |  |  |  |  |  | SPI1_SCK/ I2S1_CK |  |  |  |  |  |  | EXMC_D2 5 | DCI_D8 | TLI_G6 | EVENTOU T |
| PI2 |  |  |  | TIMER7_C H3 |  | SPI1_MIS O | I2S1_ADD _SD |  |  |  |  |  | EXMC_D2 6 | DCI_D9 | TLI_G7 | EVENTOU T |
| PI3 |  |  |  | TIMER7_E TI |  | SPI1_MOS I/I2S1_SD |  |  |  |  |  |  | EXMC_D2 7 | DCI_D10 |  | EVENTOU T |
| PI4 |  |  |  | TIMER7_B RKIN |  |  |  |  |  |  |  |  | EXMC_NB L2 | DCI_D5 | TLI_B4 | EVENTOU T |
| PI5 |  |  |  | TIMER7_C H0 |  |  |  |  |  |  |  |  | EXMC_NB L3 | DCI_VSYN C | TLI_B5 | EVENTOU T |
| PI6 |  |  |  | TIMER7_C H1 |  |  |  |  |  |  |  |  | EXMC_D2 8 | DCI_D6 | TLI_B6 | EVENTOU T |
| PI7 |  |  |  | TIMER7_C H2 |  |  |  |  |  |  |  |  | EXMC_D2 9 | DCI_D7 | TLI_B7 | EVENTOU T |
| PI8 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  | EVENTOU T |
| PI9 |  |  |  |  |  |  |  |  |  | CAN0_RX |  |  | EXMC_D3 0 |  | TLI_VSYN C | EVENTOU T |
| PI10 |  |  |  |  |  |  |  |  |  |  |  | ENET_MII _RX_ER | EXMC_D3 1 |  | TLI_HSYN C | EVENTOU T |

60

GD32F470xx Datasheet

| Pin Name | AF0 | AF1 | AF2 | AF3 | AF4 | AF5 | AF6 | AF7 | AF8 | AF9 | AF10 | AF11 | AF12 | AF13 | AF14 | AF15 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| PI11 |  |  |  |  |  |  |  |  |  |  | USBHS_U LPI_DIR |  |  |  |  | EVENTOU T |

61

GD32F470xx Datasheet

3.	Functional description

3.1.

Arm® Cortex®-M4 core

The Arm®  Cortex®-M4 processor is a high performance embedded processor with DSP instructions which allow efficient signal processing and complex algorithm execution. It brings an efficient, easy-to-use blend of control and signal processing capabilities to meet the digital signal control markets demand. The processor is highly configurable enabling a wide range of implementations from those requiring floating point operations, memory protection and powerful trace technology to cost sensitive devices requiring minimal area, while delivering outstanding computational performance and an advanced system response to interrupts.

32-bit Arm® Cortex®-M4 processor core

 Up to 240 MHz operation frequency

 Single-cycle multiplication and hardware divider

 Floating Point Unit (FPU)

 Integrated DSP instructions

 Integrated Nested Vectored Interrupt Controller (NVIC)

 24-bit SysTick timer

The Cortex®-M4 processor is based on the Armv7-M architecture and supports both Thumb and Thumb-2 instruction sets. Some system peripherals listed below are also provided by Cortex®-M4:

 Internal  Bus  Matrix  connected  with  ICode  bus,  DCode  bus,  system  bus,  Private Peripheral Bus (PPB) and debug accesses (AHB-AP)

 Nested Vectored Interrupt Controller (NVIC)

 Flash Patch and Breakpoint (FPB)

 Data Watchpoint and Trace (DWT)

 Instrument Trace Macrocell (ITM)

 Memory Protection Unit (MPU)

 Serial Wire JTAG Debug Port (SWJ-DP)

 Trace Port Interface Unit (TPIU)

3.2.

On-chip memory

 Up to 3072 Kbytes of Flash memory, including code Flash and data Flash.

 The region of the MCU executing instructions without waiting time is up to 1024K bytes (in case that Flash size equal to 512K, all memory is no waiting time). A long delay when CPU fetches the instructions out of the range.

 256 KB to 768 KB of SRAM.

The Arm® Cortex®-M4 processor is structured in Harvard architecture which can use separate

62

GD32F470xx Datasheet buses to fetch instructions and load/store data. 3072 Kbytes of inner Flash at most, which includes code Flash and data Flash is available for storing programs and data, and there is no waiting time within code Flash area when CPU executes instructions. Up to 768 Kbytes of inner SRAM is composed of SRAM0 (112KB), SRAM1 (16KB), and SRAM2 (64KB) and ADDSRAM (512KB) that can be accessed at same time, and including 64 KB of TCM (tightly- coupled memory) data RAM that can be accessed only by the data bus of the Cortex®-M4 core. The additional 4KB of backup SRAM (BKP SRAM) is implemented in the backup domain, which can keep its content even when the VDD power supply is down. Table 2-2. GD32F470xx memory map shows the memory map of the GD32F470xx series of devices, including Flash, SRAM, peripheral, and other pre-defined regions.

3.3.

Clock, reset and supply management

 Internal 16 MHz factory-trimmed RC and external 4 to 32 MHz crystal oscillator

 Internal 48 MHz RC oscillator

 Internal 32 KHz RC calibrated oscillator and external 32.768 KHz crystal oscillator

 Integrated system clock PLL

 2.6 to 3.6 V application supply and I/Os

 Supply Supervisor: POR (Power On Reset), PDR (Power Down Reset), and low voltage detector (LVD)

The Clock Control Unit (CCU) provides a range of oscillator and clock functions. These include internal RC oscillator and external crystal oscillator, high speed and low speed two types. Several prescalers allow the frequency configuration of the AHB and two APB domains. The maximum frequency of the two AHB domains are 240 MHz. The maximum frequency of the two APB domains including APB1 is 60 MHz and APB2 is 120 MHz. See Figure 2-6. GD32F470xx clock tree for details on the clock tree.

The Reset Control Unit (RCU) controls three kinds of reset: system reset resets the processor core and peripheral IP components. Power-on reset (POR) and power-down reset (PDR) are always active, and ensures proper operation starting from 2.4 V and down to 1.8V. The device remains in reset mode when VDD is below a specified threshold. The embedded low voltage detector (LVD) monitors the power supply, compares it to the voltage threshold and generates an interrupt as a warning message for leading the MCU into security.

Power supply schemes:

 VDD  range: 2.6 to 3.6 V, external power supply for I/Os and the internal regulator.

Provided externally through VDD pins.

 VSSA, VDDA  range: 2.6 to 3.6 V, external analog power supplies for ADC, reset blocks, RCs and PLL. VDDA and VSSA must be connected to VDD and VSS, respectively.

 VBAT  range: 1.8 to 3.6 V, power supply for RTC, external clock 32 KHz oscillator and backup registers (through power switch) when VDD is not present.

63

GD32F470xx Datasheet

3.4.

Boot modes

At startup, boot pins are used to select one of three boot options:

 Boot from main Flash memory (default)

 Boot from system memory

 Boot from on-chip SRAM

The boot loader is located in the internal 30KB of information blocks for the boot ROM memory (system memory). It is used to reprogram the Flash memory by using USART0 (PA9 and PA10), USART2 (PB10 and PB11, or PC10 and PC11), and USBFS (PA9, PA10, PA11 and PA12) in device mode. It also can be used to transfer and update the Flash memory code, the data and the vector table sections. In default condition, boot from bank 0 of Flash memory is selected. It also supports to boot from bank 1 of Flash memory by setting a bit in option bytes.

3.5.

Power saving modes

The  MCU  supports  three  kinds  of  power  saving  modes  to  achieve  even  lower  power consumption. They are sleep mode, deep-sleep mode, and standby mode. These operating modes reduce the power consumption and allow the application to achieve the best balance between the CPU operating time, speed and power consumption.

 Sleep mode

In sleep mode, only the clock of CPU core is off. All peripherals continue to operate and any interrupt/event can wake up the system.

 Deep-sleep mode

In deep-sleep mode, all clocks in the 1.2V domain are off, and all of the high speed crystal oscillator (IRC16M, HXTAL) and PLL are disabled. Only the contents of SRAM and registers are retained. Any interrupt or wakeup event from EXTI lines can wake up the system from the deep-sleep mode including the 16 external lines, the RTC alarm, RTC Tamper and TimeStamp event, the LVD output, ENET wakeup, RTC wakeup and USB wakeup. When exiting the deep-sleep mode, the IRC16M is selected as the system clock.

 Standby mode

In standby mode, the whole 1.2V domain is power off, the LDO is shut down, and all of IRC16M, HXTAL and PLL are disabled. The contents of SRAM and registers (except backup registers) are lost. There are four wakeup sources for the  standby mode, including the external reset from NRST pin, the RTC, the FWDGT reset, and the rising edge on WKUP pin.

3.6.

Analog to digital converter (ADC)

 12-bit SAR ADC's conversion rate is up to 2.6 MSPS

64

GD32F470xx Datasheet

 12-bit, 10-bit, 8-bit or 6-bit configurable resolution

 Hardware oversampling ratio adjustable from 2 to 256x improves resolution to 16-bit

 Input voltage range: VSSA to VDDA (2.6 V ≤ VDDA ≤ 3.6 V)

 Temperature sensor

Up to three 12-bit 2.6 MSPS multi-channel ADCs are integrated in the device. It has a total of

19 multiplexed channels: 16 external channels, 1 channel for internal temperature sensor (VSENSE), 1 channel for internal reference voltage (VREFINT) and 1 channel for external battery power supply (VBAT). The input voltage range is between 2.6 V and 3.6 V. An on-chip hardware oversampling scheme improves performance while off-loading the related computational burden from the CPU. An analog watchdog block can be used to detect the channels, which are  required  to  remain  within  a  specific  threshold  window.  A  configurable  channel management block can be used to perform conversions in single, continuous, scan or discontinuous mode to support more advanced use.

The ADC can be triggered from the events generated by the general level 0 timers (TIMERx) and the advanced timers (TIMER0 and TIMER7) with internal connection. The temperature sensor can be used to generate a voltage that varies linearly with temperature. It is internally connected to the ADC_IN16 input channel which is used to convert the sensor output voltage in a digital value.

3.7.

Digital to analog converter (DAC)

 Two 12-bit DAC converter of independent output channel

 8-bit or 12-bit mode in conjunction with the DMA controller

The 12-bit buffered DAC channel is used to generate variable analog outputs. The DACs are designed with integrated resistor strings structure. The DAC channels can be triggered by the timer update outputs or EXTI with DMA support. The maximum output value of the DAC is VREFP.

3.8.

DMA

 16 channels DMA controller and each channel are configurable (8 for DMA0 and 8 for

DMA1)

 Support independent 8, 16, 32-bit memory and peripheral transfer

 Peripherals supported: Timers, ADC, SPIs, I2Cs, USARTs, UARTs, DAC, I2S, SDIO and DCI

The flexible general-purpose DMA controllers provide a hardware method of transferring data between peripherals and/or memory without intervention from the CPU, thereby freeing up bandwidth  for  other  system  functions.  Three  types  of  access  method  are  supported: peripheral to memory, memory to peripheral, memory to memory.

Each channel is connected to fixed hardware DMA requests. The priorities of DMA channel

65

GD32F470xx Datasheet requests are determined by software configuration and hardware channel number. Transfer size of source and destination are independent and configurable.

3.9.

General-purpose inputs/outputs (GPIOs)

 Up to 140 fast GPIOs, all mappable on 16 external interrupt lines

 Analog input/output configurable

 Alternate function input/output configurable

There are up to 140 general purpose I/O pins (GPIO) in GD32F470xx, named PA0 ~ PA15, PB0 ~ PB15, PC0 ~ PC15, PD0 ~ PD15, PE0 ~ PE15, PF0 ~ PF15, PG0 ~ PG15, PH0 ~ PH15 and PI0 ~ PI11 to implement logic input/output functions. Each of the GPIO ports has related control and configuration registers to satisfy the requirements of specific applications. The external interrupts on the GPIO pins of the device have related control and configuration registers in the Interrupt/event controller (EXTI). The GPIO ports are pin-shared with other alternative functions (AFs) to obtain maximum flexibility on the package pins. Each of the GPIO pins can be configured by software as output (push-pull or open-drain), as input (with or without pull-up or pull-down) or as peripheral alternate function. Most of the GPIO pins are shared with digital or analog alternate functions. All GPIOs are high-current capable except for analog inputs.

3.10.

Timers and PWM generation

 Two 16-bit advanced timer (TIMER0 & TIMER7), eight 16-bit general timers (TIMER2, TIMER3, TIMER8 ~ TIMER13), two 32-bit general timers (TIMER1 & TIMER4) and two 16-bit basic timer (TIMER5 & TIMER6)

 Up to 4 independent channels of PWM, output compare or input capture for each general timer and external trigger input

 16-bit, motor control PWM advanced timer with programmable dead-time generation for output match

 Encoder interface controller with two inputs using quadrature decoder

 24-bit SysTick timer down counter

 2 watchdog timers (free watchdog timer and window watchdog timer)

The advanced timer (TIMER0 & TIMER7) can be used as a three-phase PWM multiplexed on 6 channels. It has complementary PWM outputs with programmable dead-time generation. It can also be used as a complete general timer. The 4 independent channels can be used for input capture, output compare, PWM generation (edge-aligned or center-aligned counting modes) and single pulse mode output. If configured as a general 16-bit timer, it has the same functions as the TIMERx timer. It can be synchronized with external signals or to interconnect with other general timers together which have the same architecture and features.

The general timer, can be used for a variety of purposes including general timer, input signal pulse width measurement or output waveform generation such as a single pulse generation

66

GD32F470xx Datasheet

or PWM output, up to 4 independent channels for input capture/output compare. TIMER1 &

TIMER4 is based on a 32-bit auto-reload up/downcounter and a 16-bit prescaler. TIMER2 &

TIMER3 is based on a 16-bit auto-reload up/downcounter and a 16-bit prescaler. TIMER8 ~ TIMER13 is based on a 16-bit auto-reload upcounter and a 16-bit prescaler. The general timer also supports an encoder interface with two inputs using quadrature decoder.

The basic timer, known as TIMER5 & TIMER6, are mainly used for DAC trigger generation. They can also be used as a simple 16-bit time base.

The GD32F470xx have two watchdog peripherals, free watchdog timer and window watchdog timer. They offer a combination of high safety level, flexibility of use and timing accuracy.

The free watchdog timer includes a 12-bit down-counting counter and an 8-bit prescaler. It is clocked from an independent 32 KHz internal RC and as it operates independently of the main clock, it can operate in deep-sleep and standby modes. It can be used either as a watchdog to reset the device when a problem occurs, or as a free-running timer for application timeout management.

The window watchdog timer is based on a 7-bit down counter that can be set as free-running. It can be used as a watchdog to reset the device when a problem occurs. It is clocked from the main clock. It has an early wakeup interrupt capability and the counter can be frozen in debug mode.

The SysTick timer is dedicated for OS, but could also be used as a standard down counter. It features:

 A 24-bit down counter

 Auto reload capability

 Maskable system interrupt generation when the counter reaches 0

 Programmable clock source

3.11.

Real time clock (RTC) and backup registers

 Independent binary-coded decimal (BCD) format timer/counter with twenty 32-bit backup registers.

 Calendar with sub-second, seconds, minutes, hours, week day, date, year and month automatically correction

 Alarm function with wake up from deep-sleep and standby mode capability

 On-the-fly correction for synchronization with master clock. Digital calibration with 1 ppm resolution for compensation of quartz crystal inaccuracy.

The real time clock is an independent timer which provides a set of continuously running counters in backup registers to provide a real calendar function, and provides an alarm interrupt or an expected interrupt. It is not reset by a system or power reset, or when the device wakes up from standby mode. A prescaler is used for the time base clock and is by default configured to generate a time base of 1 second from a clock at 32.768 KHz from external crystal oscillator.

67

GD32F470xx Datasheet

3.12.

Inter-integrated circuit (I2C)

 Up to three I2C bus interfaces can support both master and slave mode with a frequency up to 400 KHz (Fast mode)

 Provide  arbitration  function,  optional  PEC  (packet  error  checking)  generation  and checking

 Supports 7-bit and 10-bit addressing mode and general call addressing mode

The I2C interface is an internal circuit allowing communication with an external I2C interface which is an industry standard two line serial interface used for connection to external hardware. These two serial lines are known as a serial data line (SDA) and a serial clock line (SCL). The I2C module provides transfer rate of up to 100 KHz in standard mode and up to

400 KHz in fast mode. The I2C module also has an arbitration detect function to prevent the situation where more than one master attempts to transmit data to the I2C bus at the same time. A CRC-8 calculator is also provided in I2C interface to perform packet error checking for I2C data.

3.13.

Serial peripheral interface (SPI)

 Master or slave operation with full-duplex or simplex mode

 Support both master and slave mode

 Hardware CRC calculation and transmit automatic CRC error checking

 Quad wire configuration available in master mode (only in SPI5)

The SPI interface uses 4 pins, among which are the serial data input and output lines (MISO & MOSI), the clock line (SCK) and the slave select line (NSS). Both SPIs can be served by

the DMA controller. The SPI interface may be used for a variety of purposes, including simplex synchronous  transfers  on  two  lines  with  a  possible  bidirectional  data  line  or  reliable communication using CRC checking. Quad-SPI master mode is also supported in SPI5 (SPI5 is not available in GD32F470Vx series).

3.14.

Universal  synchronous/asynchronous  receiver  transmitter (USART/UART)

 Maximum speed up to 15MBit/s for USART0 and USART5

 Maximum speed up to 7.5MBit/s for USART1, USART2, UART3, UART4, UART6 and UART7

 Supports both asynchronous and clocked synchronous serial communication modes

 IrDA SIR encoder and decoder support

 LIN break generation and detection

 ISO 7816-3 compliant smart card interface

68

GD32F470xx Datasheet The USART (USART0, USART1, USART2, USART5) and UART (UART3, UART4, UART6, UART7) are used to transfer data between parallel and serial interfaces, provides a flexible full duplex data exchange using synchronous or asynchronous transfer. It is also commonly used for RS-232 standard communication. The USART/UART includes a programmable baud rate generator which is capable of dividing the system clock to produce a dedicated clock for the USART/UART transmitter and receiver. The USART/UART also supports DMA function for high speed data communication.

3.15.

Inter-IC sound (I2S)

 Two I2S bus Interfaces with sampling frequency from 8 KHz to 192 KHz, multiplexed with SPI1 and SPI2

 Support either master or slave mode Audio

 Sampling frequencies from 8 KHz up to 192 KHz are supported

The Inter-IC sound (I2S) bus provides a standard communication interface for digital audio applications by 4-wire serial lines. GD32F470xx contain an I2S-bus interface that can be operated with 16/32 bit resolution in master or slave mode, pin multiplexed with SPI1 and SPI2. The audio sampling frequencies from 8 KHz to 192 KHz is supported.

3.16.

Universal serial bus full-speed interface (USBFS)

 One USB device/host/OTG full-speed Interface with frequency up to 12 Mbit/s

 Internal 48 MHz oscillator support crystal-less operation

 Internal main PLL for USB CLK compliantly

 Internal USBFS PHY support

The Universal Serial Bus (USB) is a 4-wire bus with 4 bidirectional endpoints. The device controller  enables  12  Mbit/s  data  exchange  with  integrated  transceivers.  Transaction formatting is performed by the hardware, including CRC generation and checking. It supports both host and device modes, as well as OTG mode with Host Negotiation Protocol (HNP) and Session Request Protocol (SRP). The controller contains a full-speed USB PHY internal. For full-speed or low-speed operation, no more external PHY chip is needed. It supports all the four types of transfer (control, bulk, Interrupt and isochronous) defined in USB 2.0 protocol. The required precise 48 MHz clock which can be generated from the internal main PLL (the clock source must use an HXTAL crystal oscillator) or by the internal 48 MHz oscillator in automatic trimming mode that allows crystal-less operation.

3.17.

Universal serial bus high-speed interface (USBHS)

 One USB device/host/OTG high-speed Interface with frequency up to 480 Mbit/s

 An external PHY device connected to the ULPI is required when using in HS mode

69

GD32F470xx Datasheet USBHS supports both host and device modes, as well as OTG mode with Host Negotiation Protocol (HNP) and Session Request Protocol (SRP). The controller provides ULPI interface for external USB PHY integration and it also contains a full-speed USB PHY internal. For full- speed or low-speed operation, no more external PHY chip is needed. It supports all the four types of transfer (control, bulk, Interrupt and isochronous) defined in USB 2.0 protocol. HUB connection is supported when USBHS operates at high-speed in host mode. There is also a DMA engine operating as an AHB bus master in USBHS to speed up the data transfer between USBHS and system.

3.18.

Controller area network (CAN)

 Two CAN2.0B interface with communication frequency up to 1 Mbit/s

 Internal main PLL for CAN CLK compliantly

Controller area network (CAN) is a method for enabling serial communication in field bus. The CAN protocol has been used extensively in industrial automation and automotive applications. It can receive and transmit standard frames with 11-bit identifiers as well as extended frames with 29-bit identifiers. Each CAN has three mailboxes for transmission and two FIFOs of three message deep for reception. It also provides 28 scalable/configurable identifier filter banks for selecting the incoming messages needed and discarding the others.

3.19.

Ethernet (ENET)

 IEEE 802.3 compliant media access controller (MAC) for Ethernet LAN

 10/100 Mbit/s rates with dedicated DMA controller and SRAM

 Support hardware precision time protocol (PTP) with conformity to IEEE 1588

The Ethernet media access controller (MAC) conforms to IEEE 802.3 specifications and fully supports IEEE 1588 standards. The embedded MAC provides the interface to the required external network physical interface (PHY) for LAN bus connection via an internal media independent interface (MII) or a reduced media independent interface (RMII). The number of MII signals provided up to 16 with 25 MHz output and RMII up to 7 with 50 MHz output. The function of 32-bit CRC checking is also available.

3.20.

External memory controller (EXMC)

 Supported external memory: SRAM, PSRAM, ROM and NOR-Flash, NAND Flash and CF card, SDRAM with up to 32-bit data bus

 Provide ECC calculating hardware module for NAND Flash memory block

 Two SDRAM banks with independent configuration, up to 13-bits Row Address, 11-bits

Column Address, 2-bits internal banks address

 SDRAM Memory size: 4x16Mx32bit (256 MB), 4x16Mx16bit (128 MB), 4x16Mx8bit (64 MB)

70

GD32F470xx Datasheet External memory controller (EXMC) is an abbreviation of external memory controller. It is divided in to several sub-banks for external device support, each sub-bank has its own chip selection signal but at one time, only one bank can be accessed. The EXMC supports code execution from external memory except NAND Flash and CF card. The EXMC also can be configured to interface with the most common LCD module of Motorola 6800 and Intel 8080 series and reduce the system cost and complexity.

The EXMC of GD32F470xx in LQFP144 & BGA176 package also supports synchronous dynamic  random  access  memory  (SDRAM).  It  translates  AHB  transactions  into  the appropriate SDRAM protocol, and meanwhile, makes sure the access time requirements of the external SDRAM devices are satisfied.

3.21.

Secure digital input and output card interface (SDIO)

 Support SD2.0/SDIO2.0/MMC4.2 host interface

The Secure Digital Input and Output Card Interface (SDIO) provides access to external SD memory cards specifications version 2.0, SDIO card specification version 2.0 and multi-media card system specification version 4.2 with DMA supported. In addition, this interface is also compliant with CE-ATA digital protocol rev1.1.

3.22.

TFT LCD interface (TLI)

 24-bit RGB Parallel Pixel Output; 8 bits-per-pixel (RGB888)

 Supports up to XVGA (1024x768) resolution

 2 display layers with dedicated FIFO (64x32-bit)

The TFT LCD interface provides a parallel digital RGB (Red, Green and Blue) and signals for horizontal, vertical synchronization, Pixel Clock and Data Enable as output to interface directly to a variety of LCD (Liquid Crystal Display) and TFT (Thin Film Transistor) panels. A built-in DMA engine continuously move data from system memory to TLI and then, output to an external LCD display. Two separate layers are supported in TLI, as well as layer window and blending function.

3.23.

Image processing accelerator (IPA)

 Copy one source image to the destination image

 Convert one source image to the destination image with specific pixel format

 Convert and blend two source images to the destination image with specific pixel format

 Fill up the destination image with a specific color

The Image processing accelerator (IPA) provides a configurable and flexible image format conversion from one or two source image to the destination image. Eleven pixel formats from 4-bit up to 32-bit per pixel independently for the two source images and five pixel formats from

71

GD32F470xx Datasheet 16-bit up to 32-bit per pixel for the destination image are supported. Two 256*32 bits Look- Up Tables (LUT) separately for the two source images are implemented for the indirect pixel formats.

3.24.

Digital camera interface (DCI)

 Digital video/picture capture

 8/10/12/14 data width supported

 High transfer efficiency with DMA interface

 Video/picture crop supported

 Various pixel formats supported including JPEG/YCrCb/RGB

 Hard/embedded synchronous signals supported

DCI is an 8-bit to 14-bit parallel interface that able to capture video or picture from a camera via Digital Camera Interface. It supports 8/10/12/14 bits data width through DMA operation.

3.25.

Debug mode

 Serial wire JTAG debug port (SWJ-DP)

The Arm®  SWJ-DP Interface is embedded and is a combined JTAG and serial wire debug port that enables either a serial wire debug or a JTAG probe to be connected to the target.

3.26.

Package and operation temperature

 BGA176 (GD32F470Ix), LQFP144 (GD32F470Zx), BGA100 (GD32F470Vx) and LQFP100 (GD32F470Vx)

 Operation temperature range: -40°C to +85°C (industrial level) for grade 6 devices, and -40°C to +105°C (industrial level) for grade 7 devices

72

GD32F470xx Datasheet

4.	Electrical characteristics

4.1.

Absolute maximum ratings

The maximum ratings are the limits to which the device can be subjected without permanently damaging the device. Note that the device is not guaranteed to operate properly at the maximum ratings. Exposure to the absolute maximum rating conditions for extended periods may affect device reliability.

Table 4-1. Absolute maximum ratings(1)(4)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| VDD | External voltage range(2) | VSS - 0.3 | VSS + 3.6 | V |
| VDDA | External analog supply voltage | VSSA - 0.3 | VSSA + 3.6 | V |
| VBAT | External battery supply voltage | VSS - 0.3 | VSS + 3.6 | V |
| VIN | Input voltage on 5V tolerant pin(3) | VSS - 0.3 | VDD + 3.6 | V |
|  | Input voltage on other I/O | VSS - 0.3 | 3.6 | V |
| ¦ΔVDDX¦ | Variations between different VDD power pins | — | 50 | mV |
| ¦VSSX −VSS¦ | Variations between different ground pins | — | 50 | mV |
| IIO | Maximum current for GPIO pins | — | ±25 | mA |
| TA | Operating temperature range for grade 6 device | -40 | +85 | °C |
|  | Operating temperature range for grade 7 device | -40 | +105 |  |
| PD | Power dissipation at TA = 85°C of BGA176(5) | — | 888 | mW |
|  | Power dissipation at TA = 85°C of LQFP144(5) | — | 820 |  |
|  | Power dissipation at TA = 85°C of BGA100(5) | — | 511 |  |
|  | Power dissipation at TA = 85°C of LQFP100(5) | — | 697 |  |
|  | Power dissipation at TA = 105°C of LQFP144(5) | — | 410 |  |
|  | Power dissipation at TA = 105°C of LQFP100(5) | — | 348 |  |
| TSTG | Storage temperature range | -65 | +150 | °C |
| TJ | Maximum junction temperature | — | 125 | °C |

(1)	Guaranteed by design, not tested in production.

(2)	All main power and ground pins should be connected to an external power source within the allowable range.

(3)	VIN maximum value cannot exceed 5.5 V.

(4)	It is recommended that VDD  and VDDA  are powered by the same source. The maximum difference between

VDD and VDDA does not exceed 300 mV during power-up and operation.

(5)	For grade 6 devices, the parameter of TA=85°C, For grade 7 devices, the parameter of TA=105°C;

4.2.

Recommended DC characteristics

Table 4-2. DC operating conditions

| Symbol | Parameter | Conditions | Min(1) | Typ | Max(1) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VDD | Supply voltage | — | 2.6 | 3.3 | 3.6 | V |
| VDDA | Analog supply voltage | Same as VDD | 2.6 | 3.3 | 3.6 | V |

73

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min(1) | Typ | Max(1) | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VBAT | Battery supply voltage | — | 1.8(2) | — | 3.6 | V |  |

(1)	Based on characterization, not tested in production.

(2)	In the application which VBAT supply the backup domains, if the VBAT voltage drops below the minimum value,

when VDD is powered on again, it is necessary to refresh the registers of backup domains and enable LXTAL again.

Figure 4-1. Recommended power supply decoupling capacitors(1)(2)

|  |  | VBAT VSS N * VDD VSS VDDA VSSA VREFP VREFN |  |
| --- | --- | --- | --- |
| 100 nF |  |  |  |
| 4.7 μF + N * 100 nF |  |  |  |
| 1 μF 10 nF |  |  |  |
| 1 μF 10 nF |  |  |  |

(1)	The VREFP and VREFN pins are only available on no less than 100-pin packages, or else the VREFP and

VREFN pins are not available and internally connected to VDDA and VSSA pins. More details refer to AN056 GD32F4xx Hardware Development Guide.

(2)	All decoupling capacitors need to be as close as possible to the pins on the PCB board.

Table 4-3. Clock frequency(1)

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| fHCLK | AHB clock frequency | — | — | 240 | MHz |
| fAPB1 | APB1 clock frequency | — | — | 60 | MHz |
| fAPB2 | APB2 clock frequency | — | — | 120 | MHz |

(1)	Guaranteed by design, not tested in production.

Table 4-4. Operating conditions at Power up / Power down(1)

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| tVDD | VDD rise time rate | — | 0 | ∞ | μs/V |
|  | VDD fall time rate |  | 20 | ∞ |  |

(1)	Guaranteed by design, not tested in production.

Table 4-5. Start-up timings of Operating conditions(1)(2)(3)

| Symbol | Parameter | Conditions | Typ | Unit |
| --- | --- | --- | --- | --- |
| tstart-up | Start-up time | Code area in FLASH = 512 KB | 138 | ms |
|  |  | Code area in FLASH = 768 KB | 203 |  |

74

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Typ | Unit |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Code area in FLASH = 1024 KB | 270 |  |  |

(1)	Based on characterization, not tested in production.

(2)	After power-up, the start-up time is the time between the rising edge of NRST high and the main function.

(3)	PLL is off.

Table 4-6. Power saving mode wakeup timings characteristics(1)(2)

| Symbol | Parameter | Typ | Unit |
| --- | --- | --- | --- |
| tSleep | Wakeup from Sleep mode | 0.623 | μs |
| tDeep-sleep | Wakeup from Deep-sleep mode（LDO On） | 1.57 |  |
|  | Wakeup from Deep-sleep mode （LDO in low power mode） | 1.57 |  |
| tStandby | Wakeup from Standby mode | 140 | ms |

(1)	Based on characterization, not tested in production.

(2)	The wakeup time is measured from the wakeup event to the point at which the application code reads the first

instruction under the below conditions: VDD = VDDA = 3.3 V, IRC16M = System clock = 16 MHz.

4.3.

Power consumption

The power measurements specified in the tables represent that code with data executing from on-chip Flash with the following specifications.

Table 4-7. Power consumption characteristics(2)(3)(4)(5)(6)

| Symbol | Parameter | Conditions | Min | Typ(1) | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| IDD+IDDA | Supply current (Run mode) | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 240 MHz, All peripherals enabled | — | 73.5 | — | mA |
|  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 240 MHz, All peripherals disabled | — | 44.1 | — | mA |
|  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 200 MHz, All peripherals enabled | — | 61.5 | — | mA |
|  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 200 MHz, All peripherals disabled | — | 37.1 | — | mA |
|  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 180 MHz, All peripherals enabled | — | 55.9 | — | mA |
|  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 180 MHz, All peripherals disabled | — | 33.9 | — | mA |
|  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 168 MHz, All peripherals enabled | — | 52.6 | — | mA |

75

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ(1) | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 168 MHz, All peripherals disabled | — | 32.0 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 120 MHz, All peripherals enabled | — | 38.6 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 120 MHz, All peripherals disabled | — | 23.9 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 108 MHz, All peripherals enabled | — | 35.2 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 108 MHz, All peripherals disabled | — | 22.0 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 90 MHz, All peripherals enabled | — | 29.9 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 90 MHz, All peripherals disabled | — | 19.0 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 60 MHz, All peripherals enabled | — | 21.2 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 60 MHz, All peripherals disabled | — | 13.9 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 30 MHz, All peripherals enabled | — | 13.3 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 30 MHz, All peripherals disabled | — | 9.5 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 25 MHz, All peripherals enabled | — | 11.7 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 25 MHz, All peripherals disabled | — | 8.5 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 16 MHz, All peripherals enabled | — | 8.9 | — | mA |

76

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ(1) | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 16 MHz, All peripherals disabled | — | 6.9 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 8 MHz, All peripherals enabled | — | 6.4 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 8 MHz, All peripherals disabled | — | 5.3 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 4 MHz, All peripherals enabled | — | 5.0 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 4 MHz, All peripherals disabled | — | 4.5 | — | mA |
|  |  | Supply current (Sleep mode) | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 240 MHz,CPU clock off, All peripherals enabled | — | 50.0 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 240 MHz, CPU clock off, All peripherals disabled | — | 21.7 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 200 MHz,CPU clock off, All peripherals enabled | — | 42.2 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 200 MHz, CPU clock off, All peripherals disabled | — | 18.7 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 180 MHz, CPU clock off, All peripherals enabled | — | 38.5 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 180 MHz, CPU clock off, All peripherals disabled | — | 17.2 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 168 MHz, CPU clock off, All peripherals enabled | — | 36.2 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 168 MHz, CPU clock off, All peripherals disabled | — | 16.4 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 120 MHz, CPU clock off, All peripherals enabled | — | 27.0 | — | mA |

77

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ(1) | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 120 MHz, CPU clock off, All peripherals disabled | — | 12.8 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 108 MHz, CPU clock off, All peripherals enabled | — | 24.7 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 108 MHz, CPU clock off, All peripherals disabled | — | 11.9 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 90 MHz, CPU clock off, All peripherals enabled | — | 21.2 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 90 MHz, CPU clock off, All peripherals disabled | — | 10.5 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 60 MHz, CPU clock off, All peripherals enabled | — | 15.5 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 60 MHz, CPU clock off, All peripherals disabled | — | 8.4 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 30 MHz, CPU clock off, All peripherals enabled | — | 10.5 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 30 MHz, CPU clock off, All peripherals disabled | — | 6.7 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 25 MHz, CPU clock off, All peripherals enabled | — | 9.4 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 25 MHz, CPU clock off, All peripherals disabled | — | 6.2 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 16 MHz, CPU clock off, All peripherals enabled | — | 7.4 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 16 MHz, CPU clock off, All peripherals disabled | — | 5.4 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 8 MHz, CPU clock off, All peripherals enabled | — | 5.7 | — | mA |

78

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ(1) | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 8 MHz, CPU clock off, All peripherals disabled | — | 4.7 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 4 MHz, CPU clock off, All peripherals enabled | — | 4.8 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, HXTAL = 25 MHz, System clock = 4 MHz, CPU clock off, All peripherals disabled | — | 4.3 | — | mA |
|  |  | Supply current (Deep-Sleep mode) | VDD = VDDA = 3.3 V, LDO in run mode and normal driver mode, IRC32K off, RTC off | — | 1.39 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, LDO in low power mode and normal driver mode, IRC32K off | — | 1.36 | 11 | mA |
|  |  |  | VDD = VDDA = 3.3 V, LDO in run mode and low driver mode, IRC32K off, RTC off | — | 1.33 | — | mA |
|  |  |  | VDD = VDDA = 3.3 V, LDO in low power mode and low driver mode, IRC32K off | — | 1.30 | — | mA |
|  |  | Supply current (Standby mode) | VDD = VDDA = 3.3 V, LXTAL off, IRC32K on, RTC on, backup SRAM LDO ON | — | 9.90 | 17.5 | μA |
|  |  |  | VDD = VDDA = 3.3 V, LXTAL off, IRC32K on, RTC off, backup SRAM LDO ON | — | 9.67 | 17.3 | μA |
|  |  |  | VDD = VDDA = 3.3 V, LXTAL off, IRC32K off, RTC off, backup SRAM LDO ON | — | 9.19 | 16.8 | μA |
|  |  |  | VDD = VDDA = 3.3 V, LXTAL off, IRC32K off, RTC off, backup SRAM LDO OFF | — | 3.379 | 11 | μA |
|  | IBAT | Battery supply current (Backup mode) | VDD off, VDDA off, VBAT=3.6V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO ON | — | 9.09 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =3.3V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO ON | — | 8.93 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =2.6V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO ON | — | 8.74 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =1.8V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO ON | — | 7.47 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =3.6V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO OFF | — | 2.23 | — | μA |

79

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ(1) | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | VDD off, VDDA off, VBAT =3.3V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO OFF | — | 2.13 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =2.6V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO OFF | — | 2 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =1.8V, LXTAL on with external crystal, RTC on, LXTAL High driving, backup SRAM LDO OFF | — | 1.89 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =3.6V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO ON | — | 8.16 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =3.3V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO ON | — | 8 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =2.6V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO ON | — | 7.8 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =1.8V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO ON | — | 6.7 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =3.6V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO OFF | — | 1.27 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =3.3V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO OFF | — | 1.18 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =2.6V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO OFF | — | 1.06 | — | μA |
|  |  |  | VDD off, VDDA off, VBAT =1.8V, LXTAL on with external crystal, RTC on, LXTAL Low driving, backup SRAM LDO OFF | — | 0.96 | — | μA |

(1)	Based on characterization, not tested in production.

(2)	Unless otherwise specified, all values given for TA = 25 ℃ and test result is mean value.

(3)	When System Clock is less than 4 MHz, an external source is used, and the HXTAL bypass function is

needed, no PLL.

(4)	When System Clock is greater than 8 MHz, a crystal 8 MHz is used, and the HXTAL bypass function is

closed, using PLL.

(5)	When analog peripheral blocks such as ADCs, DACs, HXTAL, LXTAL, IRC16M, or IRC32K are ON, an

additional power consumption should be considered.

(6)	All GPIOs are configured as analog mode except standby mode.

80

GD32F470xx Datasheet

Figure 4-2. Typical supply current consumption in Run mode

Figure 4-3. Typical supply current consumption in Sleep mode

4.4.

EMC characteristics

EMS (electromagnetic susceptibility) includes ESD (Electrostatic discharge, positive and negative) and FTB (Burst of Fast Transient voltage, positive and negative) testing result is given in Table 4-8. EMS characteristics(1), based on the EMS levels and classes compliant with IEC 61000 series standard.

81

GD32F470xx Datasheet

Table 4-8. EMS characteristics(1)

| Symbol | Parameter | Conditions | Level/Class |
| --- | --- | --- | --- |
| VESD | Voltage applied to all device pins to induce a functional disturbance | VDD = 3.3 V, TA = 25 °C BGA176, fHCLK = 240 MHz conforms to IEC 61000-4-2 | 3A |
| VFTB | Fast transient voltage burst applied to induce a functional disturbance through 100 pF on VDD and VSS pins | VDD = 3.3 V, TA = 25 °C BGA176, fHCLK = 240 MHz conforms to IEC 61000-4-4 | 3A |

(1)	Based on characterization, not tested in production.

EMI (Electromagnetic Interference) emission test result is given in the Table 4-9. EMI characteristics(1), The electromagnetic field emitted by the device are monitored while an application, executing EEMBC code, is running. The test is compliant with SAE J1752-3:2017 standard which specifies the test board and the pin loading.

Table 4-9. EMI characteristics(1)

| Symbol | Parameter | Conditions | Tested frequency band | Max vs. [fHXTAL/fHCLK] | Unit |
| --- | --- | --- | --- | --- | --- |
|  |  |  |  | 25/240 MHz |  |
| SEMI | Peak level | VDD = 3.6 V, TA = +25 °C, LQFP144, fHCLK = 240 MHz, conforms to SAE J1752-3:2017 | 0.15 MHz to 30 MHz | 3.15 | dBμV |
|  |  |  | 30 MHz to 130 MHz | 11.54 |  |
|  |  |  | 130 MHz to 1 GHz | 9.55 |  |

(1)	Based on characterization, not tested in production.

4.5.

Power supply supervisor characteristics

Table 4-10. Power supply supervisor characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VLVD(1) | Low voltage Detector level selection | LVDT<2:0> = 000(rising edge) | — | 2.1 | — | V |
|  |  | LVDT<2:0> = 000(falling edge) | — | 1.98 | — |  |
|  |  | LVDT<2:0> = 001(rising edge) | — | 2.23 | — |  |
|  |  | LVDT<2:0> = 001(falling edge) | — | 2.12 | — |  |
|  |  | LVDT<2:0> = 010(rising edge) | — | 2.36 | — |  |
|  |  | LVDT<2:0> = 010(falling edge) | — | 2.25 | — |  |
|  |  | LVDT<2:0> = 011(rising edge) | — | 2.50 | — |  |
|  |  | LVDT<2:0> = 011(falling edge) | — | 2.38 | — |  |
|  |  | LVDT<2:0> = 100(rising edge) | — | 2.62 | — |  |
|  |  | LVDT<2:0> = 100(falling edge) | — | 2.52 | — |  |

82

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LVDT<2:0> = 101(rising edge) | — | 2.74 | — |  |
|  |  |  | LVDT<2:0> = 101(falling edge) | — | 2.66 | — |  |
|  |  |  | LVDT<2:0> = 110(rising edge) | — | 2.90 | — |  |
|  |  |  | LVDT<2:0> = 110(falling edge) | — | 2.80 | — |  |
|  |  |  | LVDT<2:0> = 111(rising edge) | — | 3.03 | — |  |
|  |  |  | LVDT<2:0> = 111(falling edge) | — | 2.93 | — |  |
|  | VLVDhyst(2) | LVD hystersis | — | — | 100 | — | mV |
|  | VPOR(1) | Power on reset threshold | — | — | 2.45 | — | V |
|  | VPDR(1) | Power down reset threshold | — | — | 1.82 | — | V |
|  | VPDRhyst(2) | PDR hysteresis | — | — | 600 | — | mV |
|  | VBOR3(1) | Brownout level 3 threshold | Falling edge | — | 2.80 | — | V |
|  |  |  | Rising edge | — | 2.89 | — | V |
|  | VBOR2(1) | Brownout level 2 threshold | Falling edge | — | 2.51 | — | V |
|  |  |  | Rising edge | — | 2.59 | — | V |
|  | VBOR1(1) | Brownout level 1 threshold | Falling edge | — | 2.20 | — | V |
|  |  |  | Rising edge | — | 2.30 | — | V |
|  | VBORhyst(2) | BOR hysteresis | — | — | 100 | — | mV |
|  | tRSTTEMPO(2) | Reset temporization | — | — | 2 | — | ms |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

Figure 4-4. Recommended PDR_ON pin circuit

| VDD |  |  |  |  |
| --- | --- | --- | --- | --- |
|  |  |  | PDR_ON |  |
| R External |  |  |  |  |
|  | 10 kΩ |  |  |  |
|  | circuit |  |  |  |

(1)	The PDR supervisor can be enabled/disabled through PDR_ON pin.

83

GD32F470xx Datasheet

(2)	When PDR_ON pin is connected to VSS (Internal Reset OFF), the VBAT functionality is no more available and

VBAT pin should be connected to VDD.

(3)	The PDR_ON pin must be kept at high level. The user can flexibly adjust the value of the pull-up resistor R

according to the specific scenario for a better performance.

4.6.

Electrical sensitivity

The device is strained in order to determine its performance in terms of electrical sensitivity. Electrostatic discharges (ESD) are applied directly to the pins of the sample. Static latch-up (LU) test is based on the two measurement methods.

Table 4-11. ESD characteristics(1)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VESD(HBM) | Electrostatic discharge voltage (human body model) | TA = 25 °C; JS-001-2017 | — | — | 5000 | V |
| VESD(CDM) | Electrostatic discharge voltage (charge device model) | TA = 25 °C; JS-002-2018 | — | — | 1000 | V |

(1)	Based on characterization, not tested in production.

Table 4-12. Static latch-up characteristics(1)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| LU | I-test | TA = 105 °C; JESD78 | — | — | ±200 | mA |
|  | Vsupply over voltage |  | — | — | 5.4 | V |

(1)	Based on characterization, not tested in production.

4.7.

External clock characteristics

Table 4-13. High speed  external  clock (HXTAL)  generated from a crystal/ceramic

characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fHXTAL(1) | Crystal or ceramic frequency | 2.6 V ≤VDD ≤ 3.6 V | 4 | 25 | 32 | MHz |
| RF(2) | Feedback resistor | VDD = 3.3 V | — | 400 | — | kΩ |
| CHXTAL(2) (3) | Recommended matching capacitance on OSCIN and OSCOUT | — | — | 20 | 30 | pF |
| Ducy(HXTAL)(2) | Crystal or ceramic duty cycle | — | 30 | 50 | 70 | % |
| gm(2) | Oscillator transconductance | Startup | — | 25 | — | mA/V |
| IDDHXTAL(1) | Crystal or ceramic operating current | VDD = 3.3 V | — | 1.2 | — | mA |
| tSUHXTAL(1) | Crystal or ceramic startup time | VDD = 3.3 V | — | 0.42 | — | ms |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

84

GD32F470xx Datasheet

(3)	CHXTAL1 = CHXTAL2 = 2*(CLOAD - CS), For CHXTAL1 and CHXTAL2, it is recommended matching capacitance on OSCIN

and  OSCOUT.  For  CLOAD,  it  is  crystal/ceramic  load  capacitance,  provided  by  the  crystal  or  ceramic manufacturer. For CS, it is PCB and MCU pin stray capacitance.

Table 4-14. High speed external clock characteristics (HXTAL in bypass mode)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fHXTAL_ext(1) | External clock source or oscillator frequency | 2.6 V ≤VDD ≤ 3.6 V | 1 | — | 50 | MHz |
| VHXTALH(2) | OSCIN input pin high level voltage | VDD = 3.3 V | 0.7 VDD | — | VDD | V |
| VHXTALL(2) | OSCIN input pin low level voltage |  | VSS | — | 0.3 VDD | V |
| tH/L(HXTAL) (2) | OSCIN high or low time | — | 5 | — | — | ns |
| tR/F(HXTAL) (2) | OSCIN rise or fall time | — | — | — | 10 | ns |
| CIN(2) | OSCIN input capacitance | — | — | 5 | — | pF |
| Ducy(HXTAL) (2) | Duty cycle | — | 40 | — | 60 | % |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

85

GD32F470xx Datasheet Table  4-15.  Low  speed  external  clock  (LXTAL)  generated  from  a  crystal/ceramic characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fLXTAL(1) | Crystal or ceramic frequency | VDD = 3.3 V | — | 32.768 | — | kHz |
| CLXTAL(2) (3) | Recommended matching capacitance on OSC32IN and OSC32OUT | — | — | 15 | — | pF |
| Ducy(LXTAL)(2) | Crystal or ceramic duty cycle | — | 30 | — | 70 | % |
| gm(2) | Oscillator transconductance | Medium low driving capability | — | 6 | — | μA/V |
|  |  | Higher driving capability | — | 18 | — |  |
| IDDLXTAL (1) | Crystal or ceramic operating current | LXTALDRI= 0 | — | 0.8 | — | μA |
|  |  | LXTALDRI= 1 | — | 1.6 | — |  |
| tSULXTAL(1) (4) | Crystal or ceramic startup time | LXTALDRI= 0 | — | 369 | — | ms |
|  |  | LXTALDRI= 1 | — | 175 | — | ms |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

(3)	CLXTAL1 = CLXTAL2 = 2*(CLOAD - CS), For CLXTAL1 and CLXTAL2, it is recommended matching capacitance on OSC32IN

and  OSC32OUT.  For  CLOAD,  it  is  crystal/ceramic  load capacitance,  provided  by  the crystal or ceramic

manufacturer. For CS, it is PCB and MCU pin stray capacitance.

(4)	tSULXTAL is the startup time measured from the moment it is enabled (by software) to the 32.768 kHz oscillator

stabilization flags is SET. This value varies significantly with the crystal manufacturer.

Table 4-16. Low speed external user clock characteristics (LXTAL in bypass mode)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fLXTAL_ext(1) | External clock source or oscillator frequency | VDD = 3.3 V | — | 32.768 | 1000 | kHz |
| VLXTALH(2) | OSC32IN input pin high level voltage | — | 0.7 VDD | — | VDD | V |
| VLXTALL(2) | OSC32IN input pin low level voltage | — | VSS | — | 0.3 VDD |  |
| tH/L(LXTAL) (2) | OSC32IN high or low time | — | 450 | — | — | ns |
| tR/F(LXTAL) (2) | OSC32IN rise or fall time | — | — | — | 50 |  |
| CIN(2) | OSC32IN input capacitance | — | — | 5 | — | pF |
| Ducy(LXTAL) (2) | Duty cycle | — | 30 | 50 | 70 | % |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

86

GD32F470xx Datasheet

4.8.

Internal clock characteristics

Table 4-17. High speed internal clock (IRC16M) characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fIRC16M | High Speed Internal Oscillator (IRC16M) frequency | VDD = VDDA = 3.3 V | — | 16 | — | MHz |
| ACCIRC16M | IRC16M oscillator Frequency accuracy, Factory-trimmed | VDD = VDDA = 3.3 V, TA = -40 °C ~ +85 °C for grade 6 devices | — | -1.73 to 1.1 (1) | — | % |
|  |  | VDD = VDDA = 3.3 V, TA = -40 °C ~ +105 °C for grade 7 devices | — | -1.8 to 1.1 (1) | — |  |
|  |  | VDD = VDDA = 3.3 V, TA = 25 °C | -1.0 | — | +1.0 |  |
|  | IRC16M oscillator Frequency accuracy, User trimming step(1) | — | — | 0.5 | — | % |
| DucyIRC16M(2) | IRC16M oscillator duty cycle | VDD = VDDA = 3.3 V | 45 | 50 | 55 | % |
| IDDIRC16M+ IDDAIRC16M(1) | IRC16M oscillator operating current | VDD = VDDA = 3.3 V, fHCLK =fHXTAL = 25 MHz | — | 47 | — | μA |
| tSUIRC16M(1) | IRC16M oscillator startup time | VDD = VDDA = 3.3 V, fHCLK =fHXTAL_PLL = 200 MHz | — | 1.18 | — | μs |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

87

GD32F470xx Datasheet

Table 4-18. High speed internal clock (IRC48M) characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fIRC48M | High Speed Internal Oscillator (IRC48M) frequency | VDD = 3.3 V | — | 48 | — | MHz |
| ACCIRC48M | IRC48M oscillator Frequency accuracy, Factory-trimmed | VDD = VDDA = 3.3 V, TA = -40 °C ~ +85 °C for grade 6 devices | — | -1.31 to 0.39 (1) | — | % |
|  |  | VDD = VDDA = 3.3 V, TA = -40 °C ~ +105 °C for grade 7 devices | — | -1.48 to 0.39 (1) | — |  |
|  |  | VDD = VDDA = 3.3 V, TA = 25 °C | -2.0 | — | +2.0 |  |
|  | IRC48M oscillator Frequency accuracy, User trimming step(1) | — | — | 0.12 | — | % |
| DIRC48M(2) | IRC48M oscillator duty cycle | VDD = VDDA = 3.3 V | 45 | 50 | 55 | % |
| IDDIRC48M+ IDDAIRC48M(1) | IRC48M oscillator operating current | VDD = VDDA = 3.3 V, fHCLK = fIRC16M = 16 MHz | — | 358 | — | μA |
| tSUIRC48M(1) | IRC48M oscillator startup time | VDD = VDDA = 3.3 V, fHCLK = fHXTAL_PLL = 200 MHz | — | 1.23 | — | μs |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

Table 4-19. Low speed internal clock (IRC32K) characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fIRC32K(1) | Low Speed Internal oscillator (IRC32K) frequency | VDD = VDDA = 3.3 V | — | 32 | — | kHz |
| IDDAIRC32K(2) | IRC32K oscillator operating current | VDD = VDDA = 3.3 V, fHCLK = fIRC16M = 16 MHz | — | 0.43 | — | μA |
| tSUIRC32K(2) | IRC32K oscillator startup time | VDD = VDDA = 3.3 V, fHCLK = fHXTAL_PLL = 200 MHz | — | 22.1 | — | μs |

(1)	Guaranteed by design, not tested in production.

(2)	Based on characterization, not tested in production.

4.9.

PLL characteristics

Table 4-20. PLL characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fPLLIN(1) | PLL input clock frequency | — | 1 | — | 4 | MHz |
| fPLLOUT(2) | PLL output clock frequency | — | 32 | — | 250 | MHz |
| fVCO(2) | PLL VCO output clock frequency | — | 64 | — | 500 | MHz |

88

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | tLOCK(2) | PLL lock time | — | — | — | 400 | μs |
|  | IDDA(1)(3) | Current consumption on VDDA | VCO freq = 400 MHz | — | 797 | — | μA |
|  | JitterPLL | Cycle to cycle Jitter(rms) | System clock | — | 40 | — | ps |
|  |  | Cycle to cycle Jitter （peak to peak） |  | — | 400 | — |  |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

(3)	System clock = HXTAL = 25 MHz, PLL clock source = HXTAL/25 = 1 MHz, fPLLOUT = 100 MHz.

(4)	Value given with main PLL running.

Table 4-21. PLLI2S characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fPLLIN(1) | PLLI2S input clock frequency | — | 1 | — | 4 | MHz |
| fPLLOUT(2) | PLLI2S output clock frequency | — | 32 | — | 250 | MHz |
| fVCO(2) | PLLI2S VCO output clock frequency | — | 64 | — | 500 | MHz |
| tLOCK(2) | PLLI2S lock time | — | — | — | 400 | μs |
| IDDA(1)(3) | Current consumption on VDDA | VCO freq = 400 MHz | — | 814 | — | μA |
| JitterPLL | Cycle to cycle Jitter(rms) | System clock | — | 40 | — | ps |
|  | Cycle to cycle Jitter （peak to peak） |  | — | 400 | — |  |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

(3)	System clock = HXTAL = 25 MHz, PLL clock source = HXTAL/25 = 1 MHz, fPLLOUT = 100 MHz.

(4)	Value given with main PLLI2S running.

Table 4-22. PLLSAI characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fPLLIN(1) | PLLSAI input clock frequency | — | 1 | — | 4 | MHz |
| fPLLOUT(2) | PLLSAI output clock frequency | — | 32 | — | 250 | MHz |
| fVCO(2) | PLLSAI VCO output clock frequency | — | 64 | — | 500 | MHz |
| tLOCK(2) | PLLSAI lock time | — | — | — | 400 | μs |
| IDDA(1)(3) | Current consumption on VDDA | VCO freq = 400 MHz | — | 796 | — | μA |
| JitterPLL | Cycle to cycle Jitter(rms) | System clock | — | 40 | — | ps |
|  | Cycle to cycle Jitter （peak to peak） |  | — | 400 | — |  |

(1)	Based on characterization, not tested in production.

89

GD32F470xx Datasheet

(2)	Guaranteed by design, not tested in production.

(3)	System clock = HXTAL = 25 MHz, PLL clock source = HXTAL/25 = 1 MHz, fPLLOUT = 100 MHz.

(4)	Value given with main PLLSAI running.

Table 4-23. PLL spread spectrum clock generation (SSCG) characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| FMOD | Modulation frequency | — | — | — | 10 | KHz |
| Mdamp | Peak modulation amplitude | — | — | — | 2 | % |
| MODCNT* MODSTEP | — | — | — | — | 215-1 | — |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

Equation 1: SSCG configuration equation:

MODCNT = round(fPLLIN/4/fmod)

MODSTEP   =   round(mdamp ∗ PLLN ∗ 214/(MODCNT ∗ 100)) The formula above (Equation 1) is SSCG configuration equation.

4.10.

Memory characteristics

Table 4-24. Flash memory characteristics(1)

| Symbol | Parameter | Conditions | Min(1) | Typ(1) | Max(1) | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| PECYC | Number of guaranteed program /erase cycles before failure (Endurance) | — | 100 | — | — | kcycles |
| tREAD | Read time at code flash area |  | — | 1 | — | hclks |
|  | Read time at data flash area |  | 56 | — | 4176 |  |
| tRET | Data retention time | — | — | 20 | — | years |
| tPROG | Word programming time | TA range(2) | — | 37.5 | 180 | μs |
| tERASE4kB | Page(4kB) erase time |  | — | 45 | — | ms |
| tERASE16kB | Sector(16kB) erase time |  | — | 200 | 2000 |  |
| tERASE64kB | Sector(64kB) erase time |  | — | 300 | 4000 |  |
| tERASE128kB | Sector(128kB) erase time |  | — | 600 | 8000 |  |
| tMERASE(512K) | Mass erase time |  | — | 2.4 | 32 | s |
| tMERASE(1MB) | Mass erase time |  | — | 4.8 | 64 | s |
| tMERASE(2MB) | Mass erase time |  | — | 9.6 | 128 | s |
| tMERASE(3MB) | Mass erase time |  | — | 14.4 | 192 | s |

(1)	Guaranteed by design and/or characterization, not 100% tested in production.

(2)	For grade 6 devices, TA range= -40°C ~ +85°C. For grade 7 devices, TA range= -40°C ~ +105°C.

4.11.

NRST pin characteristics

Table 4-25. NRST pin characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VIL(NRST)(1) | NRST Input low level voltage | VDD = VDDA = 2.6 V | -0.3 | — | 0.3 VDD | V |

90

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | VIH(NRST)(1) | NRST Input high level voltage |  | 0.7 VDD | — | VDD + 0.3 |  |
|  | Vhyst(1) | Schmidt trigger Voltage hysteresis |  | — | 440 | — | mV |
|  | VIL(NRST)(1) | NRST Input low level voltage | VDD = VDDA = 3.3 V | -0.3 | — | 0.3 VDD | V |
|  | VIH(NRST)(1) | NRST Input high level voltage |  | 0.7 VDD | — | VDD + 0.3 |  |
|  | Vhyst(1) | Schmidt trigger Voltage hysteresis |  | — | 490 | — | mV |
|  | VIL(NRST)(1) | NRST Input low level voltage | VDD = VDDA = 3.6 V | -0.3 | — | 0.3 VDD | V |
|  | VIH(NRST)(1) | NRST Input high level voltage |  | 0.7 VDD | — | VDD + 0.3 |  |
|  | Vhyst(1) | Schmidt trigger Voltage hysteresis |  | — | 510 | — | mV |
|  | Rpu(2) | Pull-up equivalent resistor | — | — | 40 | — | kΩ |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

Figure 4-5. Recommended external NRST pin circuit(1)

VDD	VDD

External reset circuit	10 kΩ	RPU

NRST

K	100 nF

GND

(1)	Unless the voltage on NRST pin go below VIL(NRST) level, the device would not generate a reliable reset.

4.12.

GPIO characteristics

Table 4-26. I/O port DC characteristics(1)(3)

| Symbol | Parameter |  | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| VIL | Standard IO Low level input voltage |  | 2.6 V ≤VDD = VDDA ≤ 3.6 V | — | — | 0.3 VDD | V |
|  | 5V-tolerant IO Low level input voltage |  | 2.6 V ≤VDD = VDDA ≤ 3.6 V | — | — | 0.3 VDD | V |
| VIH | Standard IO High level input voltage |  | 2.6 V ≤VDD = VDDA ≤ 3.6 V | 0.7 VDD | — | — | V |
|  | 5V-tolerant IO High level input voltage |  | 2.6 V ≤VDD = VDDA ≤ 3.6 V | 0.7 VDD | — | — | V |
| RPU(2) | Internal pull- up resistor | All pins | VIN = VSS | — | 40 | — | kΩ |
|  |  | PA10 | — | — | 10 | — |  |
| RPD(2) | Internal pull- down resistor | All pins | VIN = VDD | — | 40 | — | kΩ |
|  |  | PA10 | — | — | 10 | — |  |

91

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ | Max | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO_Speed:level 3 |  |  |  |  |  |  |  |
|  | VOL | Low level output voltage for an IO Pin (IIO = +8 mA) | VDD = 2.6 V | — | — | 0.2 | V |  |
|  |  |  | VDD = 3.3 V | — | — | 0.2 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.2 |  |  |
|  |  | Low level output voltage for an IO Pin (IIO = +20 mA) | VDD = 2.6 V | — | — | 0.29 |  |  |
|  |  |  | VDD = 3.3 V | — | — | 0.27 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.26 |  |  |
|  | VOH | High level output voltage for an IO Pin (IIO = +8 mA) | VDD = 2.6 V | 2.38 | — | — |  |  |
|  |  |  | VDD = 3.3 V | 3.1 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 3.4 | — | — |  |  |
|  |  | High level output voltage for an IO Pin (IIO = +20 mA) | VDD = 2.6 V | 2.22 | — | — |  |  |
|  |  |  | VDD = 3.3 V | 2.98 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 3.29 | — | — |  |  |
|  | IO_Speed:level 2 |  |  |  |  |  |  |  |
|  | VOL | Low level output voltage for an IO Pin (IIO = +8 mA) | VDD = 2.6 V | — | — | 0.25 | V |  |
|  |  |  | VDD = 3.3 V | — | — | 0.24 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.24 |  |  |
|  |  | Low level output voltage for an IO Pin (IIO = +20 mA) | VDD = 2.6 V | — | — | 0.43 |  |  |
|  |  |  | VDD = 3.3 V | — | — | 0.37 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.36 |  |  |
|  | VOH | High level output voltage for an IO Pin (IIO = +8 mA) | VDD = 2.6 V | 2.32 | — | — |  |  |
|  |  |  | VDD = 3.3 V | 3.04 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 3.36 | — | — |  |  |
|  |  | High level output voltage for an IO Pin (IIO = +20 mA) | VDD = 2.6 V | 2.05 | — | — |  |  |
|  |  |  | VDD = 3.3 V | 2.84 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 3.17 | — | — |  |  |
|  | IO_Speed:level 1 |  |  |  |  |  |  |  |
|  | VOL | Low level output voltage for an IO Pin (IIO = +8 mA) | VDD = 2.6 V | — | — | 0.37 | V |  |
|  |  |  | VDD = 3.3 V | — | — | 0.38 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.34 |  |  |
|  |  | (IIO = +15 mA) | VDD = 2.6 V | — | — | 0.57 |  |  |
|  |  | Low level output voltage for an IO Pin (IIO = +20 mA) | VDD = 3.3 V | — | — | 0.66 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.64 |  |  |
|  | VOH | High level output voltage for an IO Pin (IIO = +8 mA) | VDD = 2.6 V | 2.15 | — | — |  |  |
|  |  |  | VDD = 3.3 V | 2.92 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 3.23 | — | — |  |  |
|  |  | (IIO = +15 mA) | VDD = 2.6 V | 1.83 | — | — |  |  |
|  |  | High level output voltage for an IO Pin (IIO = +20 mA) | VDD = 3.3 V | 2.45 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 2.81 | — | — |  |  |

92

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ | Max | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | IO_Speed:level 0 |  |  |  |  |  |  |  |
|  | VOL | Low level output voltage for an IO Pin (IIO = +1 mA) | VDD = 2.6 V | — | — | 0.17 | V |  |
|  |  |  | VDD = 3.3 V | — | — | 0.15 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.15 |  |  |
|  |  | Low level output voltage for an IO Pin (IIO = +4 mA) | VDD = 2.6 V | — | — | 0.80 |  |  |
|  |  |  | VDD = 3.3 V | — | — | 0.63 |  |  |
|  |  |  | VDD = 3.6 V | — | — | 0.60 |  |  |
|  | VOH | High level output voltage for an IO Pin (IIO = +1 mA) | VDD = 2.6 V | 2.38 | — | — |  |  |
|  |  |  | VDD = 3.3 V | 3.12 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 3.42 | — | — |  |  |
|  |  | High level output voltage for an IO Pin (IIO = +4 mA) | VDD = 2.6 V | 1.45 | — | — |  |  |
|  |  |  | VDD = 3.3 V | 2.48 | — | — |  |  |
|  |  |  | VDD = 3.6 V | 2.83 | — | — |  |  |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

(3)	All pins except PC13 / PC14 / PC15 / PI8. Since PC13 to PC15 and PI8 are supplied through the Power Switch,

which can only be obtained by a small current( typical source capability:3mA shared between these IOs, but sink capability is same as other IO) , the speed of GPIOs PC13 to PC15 and PI8 should not exceed 2 MHz when they are in output mode (maximum load: 30 pF).

Table 4-27. I/O port AC characteristics(1)(2)(4)

| GPIOx_OSPD[1:0] bit value(3) | Parameter | Conditions | Max | Unit |
| --- | --- | --- | --- | --- |
| GPIOx_OSPD0->OSPDy[1:0] = 00 （IO_Speed:level 0） | TRise/TFall | 2.6 ≤ VDD ≤ 3.6 V, CL = 10 pF | 51 | ns |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 30 pF | 63.2 |  |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 50 pF | 74.2 |  |
| GPIOx_OSPD0->OSPDy[1:0] = 01 （IO_Speed:level 1） | TRise/TFall | 2.6 ≤ VDD ≤ 3.6 V, CL = 10 pF | 3.6 | ns |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 30 pF | 9.6 |  |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 50 pF | 12.2 |  |
| GPIOx_OSPD0->OSPDy[1:0] = 10 （IO_Speed: level 2） | TRise/TFall | 2.6 ≤ VDD ≤ 3.6 V, CL = 10 pF | 2.2 | ns |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 30 pF | 3 |  |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 50 pF | 3.8 |  |
| GPIOx_OSPD0->OSPDy[1:0] = 11 （IO_Speed:level 3） | TRise/TFall | 2.6 ≤ VDD ≤ 3.6 V, CL = 10 pF | 2 | ns |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 30 pF | 2.8 |  |
|  |  | 2.6 ≤VDD≤ 3.6 V, CL = 50 pF | 3.4 |  |

(1)	Based on characterization, not tested in production.

(2)	Unless otherwise specified, all test results given for TA = 25 °C.

(3)	The I/O speed is configured using the GPIOx_OSPD -> OSPDy[1:0]bits.

(4)	Only for reference, Depending on user’s design.

(5)	Max frequency is defined when the sum of rise time plus the fall time is less than 2/3 cycle.

93

GD32F470xx Datasheet

4.13.

ADC characteristics

Table 4-28. ADC characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VDDA(1) | Operating voltage | — | 2.6 | 3.3 | 3.6 | V |
| VIN(1) | ADC input voltage range | — | 0 | — | VREFP | V |
| VREFP(2)(3) | Positive Reference Voltage | — | 2.6 | — | VDDA | V |
| VREFN(2) | Negative Reference Voltage | — | — | VSSA | — | V |
| fADC(1) | ADC clock | — | 0.1 | — | 40 | MHz |
| fS(1) | Sampling rate | 12-bit | 0.007 | — | 2.6 | MSP S |
|  |  | 10-bit | 0.008 | — | 3.1 |  |
|  |  | 8-bit | 0.01 | — | 3.6 |  |
|  |  | 6-bit | 0.011 | — | 4.4 |  |
| VAIN(1) | Analog input voltage | 16 external; 3 internal | 0 | — | VREFP | V |
| RAIN(2) | External input impedance | See Equation 2 | — | — | 308.6 | kΩ |
| RADC(2) | Input sampling switch resistance | — | — | — | 0.55 | kΩ |
| CADC(2) | Input sampling capacitance | No pin/pad capacitance included | — | — | 4.0 | pF |
| tCAL(2) | Calibration time | fADC = 40 MHz | — | 3.275 | — | μs |
| ts(2) | Sampling time | fADC = 40 MHz | 0.075 | — | 12 | μs |
| tCONV(2) | Total conversion time (including sampling time) | 12-bit | — | 15 | — | 1/ fADC |
|  |  | 10-bit | — | 13 | — |  |
|  |  | 8-bit | — | 11 | — |  |
|  |  | 6-bit | — | 9 | — |  |
| tSU(2) | Startup time | — | — | — | 1 | μs |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

(3)	VREFP should always be equal to or less than VDDA, especially during power up.

Equation 2: RAIN max formula   RAIN  <

Ts fADC∗CADC∗ln(2N+2) − RADC

The formula above (Equation 2) is used to determine the maximum external impedance allowed for an error below 1/4 of LSB. Here N = 12 (from 12-bit resolution).

Table 4-29. ADC RAIN max for fADC = 40 MHz(2)

| Ts (cycles) | ts (us) | RAIN max (KΩ) |
| --- | --- | --- |
| 3 | 0.075 | 1.3 |
| 15 | 0.375 | 9.1 |
| 28 | 0.7 | 17.4 |
| 55 | 1.375 | 34.8 |
| 84 | 2.1 | 53.5 |
| 112 | 2.8 | 71.5 |
| 144 | 3.6 | 92.4 |

94

GD32F470xx Datasheet

|  | Ts (cycles) | ts (us) | RAIN max (KΩ) |
| --- | --- | --- | --- |
|  | 480 | 12 | 308.6 |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

Table 4-30. ADC dynamic accuracy at fADC = 40 MHz(1)

| Symbol | Parameter | Test conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| ENOB | Effective number of bits | fADC = 40 MHz VDDA = VREFP = 3.3 V Input Frequency = 110 kHz Temperature = 25 ℃ | — | 10.9 | — | bits |
| SNDR | Signal-to-noise and distortion ratio |  | — | 67.3 | — | dB |
| SNR | Signal-to-noise ratio |  | — | 67.7 | — |  |
| THD | Total harmonic distortion |  | — | -75 | — |  |

(1)	Based on characterization, not tested in production.

Table 4-31. ADC static accuracy at fADC = 40 MHz(1)

| Symbol | Parameter | Test conditions | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| Offset | Offset error | fADC = 40 MHz VDDA = VREFP = 3.3 V | ±1 | — | LSB |
| DNL | Differential linearity error |  | ±1 | — |  |
| INL | Integral linearity error |  | ±1.5 | — |  |

(1)	Based on characterization, not tested in production.

4.14.

Temperature sensor characteristics

Table 4-32. Temperature sensor characteristics(1)

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| TL | VSENSE linearity with temperature | — | ±1.5 | — | ℃ |
| Avg_Slope | Average slope | — | 4.4 | — | mV/℃ |
| V25 | Voltage at 25 °C | — | 1.4 | — | V |
| tS_temp (2) | ADC sampling time when reading the temperature | — | 17.1 | — | μs |

(1)	Based on characterization, not tested in production.

(2)	Shortest sampling time can be determined in the application by multiple iterations.

4.15.

DAC characteristics

Table 4-33. DAC characteristics

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| VDDA(1) | Operating voltage | — | 2.6 | 3.3 | 3.6 | V |
| VREFP(2) | Positive Reference Voltage | — | 2.6 | — | VDDA | V |
| VREFN(2) | Negative Reference Voltage | — | — | VSSA | — | V |
| RLOAD(2) | Resistive load | Resistive load with buffer ON | 5 | — | — | kΩ |
| Ro(2) | Impedance output | Impedance output with buffer OFF | — | — | 15 | kΩ |
| CLOAD(2) | Capacitive load | Capacitive load with buffer ON | — | — | 50 | pF |

95

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | DAC_OUT min(2) | Lower DAC_OUT voltage | Lower DAC_OUT voltage with buffer ON | 0.2 | — | — | V |
|  |  |  | Lower DAC_OUT voltage with buffer OFF | 0.5 | — | — | mV |
|  | DAC_OUT max (2) | Higher DAC_OUT voltage | Higher DAC_OUT voltage with buffer ON | — | — | VDDA - 0.2 | V |
|  |  |  | Higher DAC_OUT voltage with buffer OFF | — | — | VDDA - 1LSB | V |
|  | IDDA(1) | DAC current consumption in quiescent mode | With no load, middle code(0x800) on the input, VREFP = 3.6 V | — | 350 | — | μA |
|  |  |  | With no load, worst code(0xF1C) on the input, VREFP = 3.6 V | — | 430 | — |  |
|  | IDDVREFP(1) | DAC current consumption in quiescent mode | With no load, middle code(0x800) on the input, VREFP = 3.6 V | — | 115 | — | μA |
|  |  |  | With no load, worst code(0xF1C) on the input, VREFP = 3.6 V | — | 298 | — |  |
|  | DNL(1) | Differential non linearity | 10-bit configuration | — | — | ±0.75 | LSB |
|  |  |  | 12-bit configuration | — | — | ±3 |  |
|  | INL(1) | Integral non linearity | 10-bit configuration | — | — | ±1.25 | LSB |
|  |  |  | 12-bit configuration | — | — | ±5 |  |
|  | Offset(1) | Offset error | DAC in 12-bit mode | — | — | ±24 | LSB |
|  | GE(1) | Gain error | DAC in 12-bit mode | — | — | ±1.5 | % |
|  | Tsetting(1) | Settling time | CLOAD ≤ 50 pF, RLOAD ≥ 5 kΩ | — | 0.5 | 1 | μs |
|  | Twakeup(2) | Wakeup from off state | — | — | 5 | 10 | μs |
|  | Update rate(2) | Max frequency for a correct DAC_OUT change from code i to i±1LSB | CLOAD ≤ 50 pF, RLOAD ≥ 5 kΩ | — | — | 4 | MS/s |
|  | PSRR(2) | Power supply rejection ratio(to VDDA) | No RLoad, CLOAD=50 pF | — | -90 | — | dB |

(1)	Based on characterization, not tested in production.

(2)	Guaranteed by design, not tested in production.

4.16.

I2C characteristics

Table 4-34. I2C characteristics(1)(2)

| Symbol | Parameter | Conditions | Standard mode |  | Fast mode |  | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Min | Max | Min | Max |  |
| tSCL(H) | SCL clock high time | — | 4.0 | — | 0.6 | — | μs |

96

GD32F470xx Datasheet

|  | Symbol | Parameter | Conditions | Standard mode |  | Fast mode |  | Unit |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | Min | Max | Min | Max |  |  |
|  | tSCL(L) | SCL clock low time | — | 4.7 | — | 1.3 | — | μs |  |
|  | tSU(SDA) | SDA setup time | — | 250 | — | 100 | — | ns |  |
|  | th(SDA) | SDA data hold time | — | 0(3) | 3450 | 0 | 900 | ns |  |
|  | tr(SDA/SCL) | SDA and SCL rise time | — | — | 1000 | — | 300 | ns |  |
|  | tf(SDA/SCL) | SDA and SCL fall time | — | — | 300 | — | 300 | ns |  |
|  | th(STA) | Start condition hold time | — | 4.0 | — | 0.6 | — | μs |  |
|  | tSU(STA) | Repeated Start condition setup time | — | 4.7 | — | 0.6 | — | μs |  |
|  | tSU(STO) | Stop condition setup time | — | 4.0 | — | 0.6 | — | μs |  |
|  | tbuff | Stop to Start condition time (bus free) | — | 4.7 | — | 1.3 | — | μs |  |

(1)	Guaranteed by design, not tested in production.

(2)	To ensure the standard mode I2C frequency, fPCLK1  must be at least 2 MHz. To ensure the fast mode I2C

frequency, fPCLK1 must be at least 4 MHz.

(3)	The device should provide a data hold time of 300 ns at least in order to bridge the undefined region of the

falling edge of SCL.

Figure 4-6. I2C bus timing diagram

70%

tSU(STA)

SDA	70%	70%	70%

30%	30%	tBUFF

tF(SDA)	tR(SDA)	tH(SDA)

SCL

tH(STA)

70%

tSCL(H)

tSU(SDA)

70%

30%

tSCL(L)	tR(SCL)	tF(SCL)	tSU(STO)

97

GD32F470xx Datasheet

4.17.

SPI characteristics

Table 4-35. Standard SPI characteristics(1)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fSCK | SCK clock frequency | — | — | — | 30 | MHz |
| tSCK(H) | SCK clock high time | Master mode, fPCLKx = 120 MHz, presc = 4 | 14.67 | 16.67 | 18.67 | ns |
| tSCK(L) | SCK clock low time | Master mode, fPCLKx = 120 MHz, presc = 4 | 14.67 | 16.67 | 18.67 | ns |
| SPI master mode |  |  |  |  |  |  |
| tV(MO) | Data output valid time | — | — | — | 8 | ns |
| tSU(MI) | Data input setup time | — | 6 | — | — | ns |
| tH(MI) | Data input hold time | — | 0 | — | — | ns |
| SPI slave mode |  |  |  |  |  |  |
| tSU(NSS) | NSS enable setup time | — | 0 | — | — | ns |
| tH(NSS) | NSS enable hold time | — | 3.3 | — | — | ns |
| tA(SO) | Data output access time | — | — | 9 | — | ns |
| tDIS(SO) | Data output disable time | — | — | 10 | — | ns |
| tV(SO) | Data output valid time | — | — | 11 | — | ns |
| tSU(SI) | Data input setup time | — | 0 | — | — | ns |
| tH(SI) | Data input hold time | — | 2.2 | — | — | ns |

(1)	Based on characterization, not tested in production.

Figure 4-7. SPI timing diagram - master mode

tSCK

SCK (CKPH=0 CKPL=0) SCK (CKPH=0 CKPL=1)

SCK (CKPH=1 CKPL=0)

SCK (CKPH=1 CKPL=1)

tSCK(H) tSCK(L)

tSU(MI)

MISO	D[0]	D[7]

LF=1,FF16=0	tH(MI)

MOSI	D[0]	D[7]

tV(MO)	tH(MO)

98

GD32F470xx Datasheet

Figure 4-8. SPI timing diagram - slave mode

NSS

tSU(NSS)	tSCK	tH(NSS)

SCK (CKPH=0 CKPL=0) SCK (CKPH=0 CKPL=1)

tSCK(H) tSCK(L)

tH(SO)

tA(SO)	tV(SO)	tDIS(SO)

MISO	D[0]	D[7]

tSU(SI)

MOSI

D[0]

tH(SI)

D[7]

99

GD32F470xx Datasheet

4.18.

I2S characteristics

Table 4-36. I2S characteristics(1)(2)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fCK | Clock frequency | Master mode (data: 32 bits, Audio frequency = 96 kHz) | — | 6.25 | — | MHz |
|  |  | Slave mode | — | — | 12.5 |  |
| tH | Clock high time | — | — | 80 | — | ns |
| tL | Clock low time |  | — | 80 | — | ns |
| tV(WS) | WS valid time | Master mode | — | 3 | — | ns |
| tH(WS) | WS hold time | Master mode | — | 3 | — | ns |
| tSU(WS) | WS setup time | Slave mode | 0 | — | — | ns |
| tH(WS) | WS hold time | Slave mode | 3 | — | — | ns |
| Ducy(SCK) | I2S slave input clock duty cycle | Slave mode | — | 50 | — | % |
| tSU(SD_MR) | Data input setup time | Master mode | 0 | — | — | ns |
| tsu(SD_SR) | Data input setup time | Slave mode | 0 | — | — | ns |
| tH(SD_MR) | Data input hold time | Master receiver | 1 | — | — | ns |
| tH(SD_SR) |  | Slave receiver | 3 | — | — | ns |
| tV(SD_ST) | Data output valid time | Slave transmitter (after enable edge) | — | — | 9 | ns |
| tH(SD_ST) | Data output hold time | Slave transmitter (after enable edge) | 6 | — | — | ns |
| tV(SD_MT) | Data output valid time | Master transmitter (after enable edge) | — | — | 6 | ns |
| tH(SD_MT) | Data output hold time | Master transmitter (after enable edge) | 0 | — | — | ns |

(1)	Guaranteed by design, not tested in production.

(2)	Based on characterization, not tested in production.

100

GD32F470xx Datasheet

Figure 4-9. I2S timing diagram - master mode

tCK

CPOL=0

CPOL=1

tL

WS output	tV(WS)	tH	tH(WS)

tV(SD_MT)	tH(SD_MT)

SD transmit

SD receive

D[0]

D[0]

tSU(SD_MR)

tH(SD_MR)

Figure 4-10. I2S timing diagram - slave mode

tCK

CPOL=0

tL

CPOL=1

tH

WS input

tH(WS)

tSU(WS)	tV(SD_ST)	tH(SD_ST)

SD transmit

SD receive

D[0]

D[0]

tSU(SD_SR)

tH(SD_SR)

101

GD32F470xx Datasheet

4.19.

USART characteristics

Table 4-37. USART characteristics(1)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fSCK | SCK clock frequency | fPCLKx = 120 MHz | — | — | 60 | MHz |
| tSCK(H) | SCK clock high time | fPCLKx = 120 MHz | 8.33 | — | — | ns |
| tSCK(L) | SCK clock low time | fPCLKx = 120 MHz | 8.33 | — | — | ns |

(1)	Guaranteed by design, not tested in production.

4.20.

SDIO characteristics

Table 4-38. SDIO characteristics(1)(2)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| fPP(3) | Clock frequency in data transfer mode | — | 0 | — | 48 | MHz |
| tW(CKL) (3) | Clock low time | fpp = 48 MHz | 9.5 | 10.5 | — | ns |
| tW(CKH) (3) | Clock high time | fpp = 48 MHz | 9.3 | 10.3 | — | ns |
| CMD, D inputs (referenced to CK) in MMC and SD HS mode |  |  |  |  |  |  |
| tISU(4) | Input setup time HS | fpp = 48 MHz | 4 | — | — | ns |
| tIH(4) | Input hold time HS | fpp = 48 MHz | 3 | — | — | ns |
| CMD, D outputs (referenced to CK) in MMC and SD HS mode |  |  |  |  |  |  |
| tOV(3) | Output valid time HS | fpp = 48 MHz | — | — | 13.8 | ns |
| tOH(3) | Output hold time HS | fpp = 48 MHz | 12 | — | — | ns |
| CMD, D inputs (referenced to CK) in SD default mode |  |  |  |  |  |  |
| tISUD(4) | Input setup time SD | fpp = 24 MHz | 3 | — | — | ns |
| tIHD(4) | Input hold time SD | fpp = 24 MHz | 3 | — | — | ns |
| CMD, D outputs (referenced to CK) in SD default mode |  |  |  |  |  |  |
| tOVD(3) | Output valid default time SD | fpp = 24 MHz | — | 2.4 | 2.8 | ns |
| tOHD(3) | Output hold default time SD | fpp = 24 MHz | 2 | — | — | ns |

(1)	CLK timing is measured at 50% of VDD.

(2)	Capacitive load CL = 30 pF.

(3)	Based on characterization, not tested in production.

(4)	Guaranteed by design, not tested in production.

4.21.

CAN characteristics

Refer to Table 4-26. I/O port DC characteristics(1) for more details on the input/output alternate function characteristics (CANTX and CANRX).

102

|  | tf |  | tr |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

GD32F470xx Datasheet

4.22.

USBFS characteristics

Table 4-39. USBFS start up time

| Symbol | Parameter | Max | Unit |
| --- | --- | --- | --- |
| tSTARTUP(1) | USBFS startup time | 1 | μs |

(1)	Guaranteed by design, not tested in production.

Table 4-40. USBFS DC electrical characteristics

| Symbol |  | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Input levels(1) | VDD | USBFS operating voltage | — | 3 | — | 3.6 | V |
|  | VDI | Differential input sensitivity | — | 0.2 | — | — |  |
|  | VCM | Differential common mode range | Includes VDI range | 0.8 | — | 2.5 |  |
|  | VSE | Single ended receiver threshold | — | 1.3 | — | 2.0 |  |
| Output levels (2) | VOL | Static output level low | RL of 1.0 kΩ to 3.6 V | — | 0.06 | 0.3 | V |
|  | VOH | Static output level high | RL of 15 kΩ to VSS | 2.8 | 3.3 | 3.6 |  |
| RPD(2) |  | PA11, PA12(USBFS_DM/DP) PB14, PB15(USBHS_ DM/DP) | VIN = VDD | 17 | 21 | 25 | kΩ |
|  |  | PA9(USBFS_VBUS) PB13(USBHS_VBUS) |  | 0.72 | 0.9 | 1.1 |  |
| RPU(2) |  | PA11, PA12(USBFS_DM/DP) PB14, PB15(USBHS_ DM/DP) | VIN = VSS | 1.2 | 1.5 | 1.8 |  |
|  |  | PA9(USBFS_VBUS) PB13(USBHS_VBUS) |  | 0.24 | 0.3 | 0.33 |  |

(1)	Guaranteed by design, not tested in production.

(2)	Based on characterization, not tested in production.

Table 4-41. USBFS full speed-electrical characteristics(1)

| Symbol | Parameter | Conditions | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- | --- |
| tR | Rise time | CL = 50 pF | 4 | — | 20 | ns |
| tF | Fall time | CL = 50 pF | 4 | — | 20 | ns |
| tRFM | Rise/ fall time matching | tR / tF | 90 | — | 110 | % |
| vCRS | Output signal crossover voltage | — | 1.3 | — | 2.0 | V |

(1)	Guaranteed by design, not tested in production.

Figure 4-11. USBFS timings: definition of data signal rise and fall time

Crossover

points

Differential

data lines VCRS

VSS

103

Table 4-42. USBHS clock timing parameters

GD32F470xx Datasheet

4.23.	USBHS characteristics

(1)

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| VDD | USBHS operating voltage | 3.0 | — | 3.6 | V |
| fHCLK | fHCLK value to guarantee proper operation of USBHS interface | 30 | — | — | MHz |
| FSTART_8BIT | Frequency (first transition) 8-bit ± 10% | 54 | 60 | 66 | MHz |
| FSTEADY | Frequency (steady state) ±500 ppm | 59.97 | 60 | 60.63 | MHz |
| DSTART_8BIT | Duty cycle (first transition) 8-bit ± 10% | 40 | 50 | 60 | % |
| DSTEADY | Duty cycle (steady state) ±500 ppm | 49.975 | 50 | 50.025 | % |

(1)	Guaranteed by design, not tested in production.

Table 4-43. USB-ULPI Dynammic characteristics

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| tSC | Control in (ULPI_DIR, ULPI_NXT) setup time | — | — | 2 | ns |
| tHC | Control in (ULPI_DIR, ULPI_NXT) hold time | 0.5 | — | — | ns |
| tSD | Data in setup time | — | — | 2 | ns |
| tHD | Data in hold time | 0 | — | — | ns |

(1)	Guaranteed by design, not tested in production.

4.24.

Ethernet (ENET) characteristics

Table 4-44. Dynamics characteristics: Ethernet MAC signals for SMI(1)

| Symbol | Parameter | Min | Typ | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| tMDC | MDC cycle time(2.38 MHz) | 411 | 420 | 425 | ns |
| TD(MDIO） | Write data valid delay time | 1/4 tmdc | 1/4 tmdc | 1/4 tmdc |  |
| tSU(MDIO) | Read data setup time | 12 | — | — |  |
| tH(SD_MR) | Read data hold time | 0 | — | — |  |

(1)	Guaranteed by design, not tested in production.

104

GD32F470xx Datasheet

Figure 4-12. Ethernet SMI timing diagram

tMDC

ETH_MDC	TD

Th(MDIOI)

ETH_MDIO(O)

ETH_MDIO(I)

4.25.

EXMC characteristics

Table 4-45. Asynchronous non-multiplexed SRAM/PSRAM/NOR read timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | EXMC_NE low time | 19.85 | 21.85 | ns |
| tV(NOE_NE) | EXMC_NEx low to EXMC_NOE low | 0 | — | ns |
| tw(NOE) | EXMC_NOE low time | 19.85 | 21.85 | ns |
| th(NE_NOE) | EXMC_NOE high to EXMC_NE high hold time | 0 | — | ns |
| tv(A_NE) | EXMC_NEx low to EXMC_A valid | 0 | — | ns |
| tv(BL_NE) | EXMC_NEx low to EXMC_BL valid | 0 | — | ns |
| tsu(DATA_NE) | Data to EXMC_NEx high setup time | 15.68 | — | ns |
| tsu(DATA_NOE) | Data to EXMC_NOEx high setup time | 15.68 | — | ns |
| th(DATA_NOE) | Data hold time after EXMC_NOE high | 0 | — | ns |
| th(DATA_NE) | Data hold time after EXMC_NEx high | 0 | — | ns |
| tv(NADV_NE) | EXMC_NEx low to EXMC_NADV low | 0 | — | ns |
| tw(NADV) | EXMC_NADV low time | 3.17 | 5.17 | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	Based on configure: fHCLK = 240 MHz, AddressSetupTime = 0, AddressHoldTime = 1, DataSetupTime = 1.

105

GD32F470xx Datasheet

Table 4-46. Asynchronous non-multiplexed SRAM/PSRAM/NOR write timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | EXMC_NE low time | 11.51 | 13.51 | ns |
| tV(NWE_NE) | EXMC_NEx low to EXMC_NWE low | 3.17 | — | ns |
| tw(NWE) | EXMC_NWE low time | 3.17 | 5.17 | ns |
| th(NE_NWE) | EXMC_NWE high to EXMC_NE high hold time | 3.17 | 5.17 | ns |
| tv(A_NE) | EXMC_NEx low to EXMC_A valid | 0 | — | ns |
| tV(NADV_NE) | EXMC_NEx low to EXMC_NADV low | 0 | — | ns |
| tw(NADV) | EXMC_NADV low time | 3.17 | 5.17 | ns |
| th(AD_NADV) | EXMC_AD(address) valid hold time after EXMC_NADV high | 7.34 | — | ns |
| th(A_NWE) | Address hold time after EXMC_NWE high | 3.17 | — | ns |
| th(BL_NWE) | EXMC_BL hold time after EXMC_NWE high | 3.17 | — | ns |
| tv(BL_NE) | EXMC_NEx low to EXMC_BL valid | 0 | — | ns |
| tv(DATA_NADV) | EXMC_NADV high to DATA valid | 0 | — | ns |
| th(DATA_NWE) | Data hold time after EXMC_NWE high | 3.17 | — | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	Based on configure: fHCLK  = 240 MHz, AddressSetupTime = 0, AddressHoldTime = 1, DataSetupTime = 1.

Table 4-47. Asynchronous multiplexed PSRAM/NOR read timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | EXMC_NE low time | 28.19 | 30.19 | ns |
| tV(NOE_NE) | EXMC_NEx low to EXMC_NOE low | 11.51 | — | ns |
| tw(NOE) | EXMC_NOE low time | 15.68 | 17.68 | ns |
| th(NE_NOE) | EXMC_NOE high to EXMC_NE high hold time | 0 | — | ns |
| tv(A_NE) | EXMC_NEx low to EXMC_A valid | 0 | — | ns |
| tv(A_NOE) | Address hold time after EXMC_NOE high | 0 | — | ns |
| tv(BL_NE) | EXMC_NEx low to EXMC_BL valid | 0 | — | ns |
| th(BL_NOE) | EXMC_BL hold time after EXMC_NOE high | 0 | — | ns |
| tsu(DATA_NE) | Data to EXMC_NEx high setup time | 15.68 | — | ns |
| tsu(DATA_NOE) | Data to EXMC_NOEx high setup time | 15.68 | — | ns |
| th(DATA_NOE) | Data hold time after EXMC_NOE high | 0 | — | ns |
| th(DATA_NE) | Data hold time after EXMC_NEx high | 0 | — | ns |
| tv(NADV_NE) | EXMC_NEx low to EXMC_NADV low | 0 | — | ns |
| tw(NADV) | EXMC_NADV low time | 3.17 | 5.17 | ns |
| Th(AD_NADV) | EXMC_AD(adress) valid hold time after EXMC_NADV high | 3.17 | 5.17 | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	Based on configure: fHCLK= 240 MHz, AddressSetupTime = 0, AddressHoldTime = 1, DataSetupTime = 1.

Table 4-48. Asynchronous multiplexed PSRAM/NOR write timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(NE) | EXMC_NE low time | 19.85 | 21.85 | ns |

106

GD32F470xx Datasheet

|  | Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
|  | tV(NWE_NE) | EXMC_NEx low to EXMC_NWE low | 3.17 | — | ns |
|  | tw(NWE) | EXMC_NWE low time | 11.51 | 13.51 | ns |
|  | th(NE_NWE) | EXMC_NWE high to EXMC_NE high hold time | 3.17 | — | ns |
|  | tv(A_NE) | EXMC_NEx low to EXMC_A valid | 0 | — | ns |
|  | tV(NADV_NE) | EXMC_NEx low to EXMC_NADV low | 0 | — | ns |
|  | tw(NADV) | EXMC_NADV low time | 3.17 | 5.17 | ns |
|  | th(AD_NADV) | EXMC_AD(address) valid hold time after EXMC_NADV high | 3.17 | — | ns |
|  | th(A_NWE) | Address hold time after EXMC_NWE high | 3.17 | — | ns |
|  | th(BL_NWE) | EXMC_BL hold time after EXMC_NWE high | 3.17 | — | ns |
|  | tv(BL_NE) | EXMC_NEx low to EXMC_BL valid | 0 | — | ns |
|  | tv(DATA_NADV) | EXMC_NADV high to DATA valid | 3.17 | — | ns |
|  | th(DATA_NWE) | Data hold time after EXMC_NWE high | 3.17 | — | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	Based on configure: fHCLK = 240 MHz, AddressSetupTime = 0, AddressHoldTime = 1, DataSetupTime = 1.

Table 4-49. Synchronous multiplexed PSRAM/NOR read timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | EXMC_CLK period | 16.67 | — | ns |
| td(CLKL-NExL) | EXMC_CLK low to EXMC_NEx low | 0 | — | ns |
| td(CLKH-NExH) | EXMC_CLK high to EXMC_NEx high | 7.34 | — | ns |
| td(CLKL-NADVL) | EXMC_CLK low to EXMC_NADV low | 0 | — | ns |
| td(CLKL-NADVH) | EXMC_CLK low to EXMC_NADV high | 0 | — | ns |
| td(CLKL-AV) | EXMC_CLK low to EXMC_Ax valid | 0 | — | ns |
| td(CLKH-AIV) | EXMC_CLK high to EXMC_Ax invalid | 7.34 | — | ns |
| td(CLKL-NOEL) | EXMC_CLK low to EXMC_NOE low | 0 | — | ns |
| td(CLKH-NOEH) | EXMC_CLK high to EXMC_NOE high | 7.34 | — | ns |
| td(CLKL-ADV) | EXMC_CLK low to EXMC_AD valid | 0 | — | ns |
| td(CLKL-ADIV) | EXMC_CLK low to EXMC_AD invalid | 0 | — | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	(Based on configure: fHCLK = 240 MHz, BurstAccessMode = Enable; Memory Type = PSRAM; WriteBurst =

Enable; CLKDivision = 3 (EXMC_CLK is 4 divided by HCLK); Data Latency = 1.

Table 4-50. Synchronous multiplexed PSRAM write timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | EXMC_CLK period | 16.67 | — | ns |
| td(CLKL-NExL) | EXMC_CLK low to EXMC_NEx low | 0 | — | ns |
| td(CLKH-NExH) | EXMC_CLK high to EXMC_NEx high | 7.34 | — | ns |
| td(CLKL-NADVL) | EXMC_CLK low to EXMC_NADV low | 0 | — | ns |
| td(CLKL-NADVH) | EXMC_CLK low to EXMC_NADV high | 0 | — | ns |
| td(CLKL-AV) | EXMC_CLK low to EXMC_Ax valid | 0 | — | ns |
| td(CLKH-AIV) | EXMC_CLK high to EXMC_Ax invalid | 7.34 | — | ns |

107

GD32F470xx Datasheet

|  | Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
|  | td(CLKL-NWEL) | EXMC_CLK low to EXMC_NWE low | 0 | — | ns |
|  | td(CLKH-NWEH) | EXMC_CLK high to EXMC_NWE high | 7.34 | — | ns |
|  | td(CLKL-ADIV) | EXMC_CLK low to EXMC_AD invalid | 0 | — | ns |
|  | td(CLKL-DATA) | EXMC_A/D valid data after EXMC_CLK low | 0 | — | ns |
|  | th(CLKL-NBLH) | EXMC_CLK low to EXMC_NBL high | 0 | — | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	Based on configure: fHCLK  = 240 MHz, BurstAccessMode = Enable; MemoryType = PSRAM; WriteBurst =

Enable; CLKDivision = 3 (EXMC_CLK is 4 divided by HCLK); DataLatency = 1.

Table 4-51. Synchronous non-multiplexed PSRAM/NOR read timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | EXMC_CLK period | 16.67 | — | ns |
| td(CLKL-NExL) | EXMC_CLK low to EXMC_NEx low | 0 | — | ns |
| td(CLKH-NExH) | EXMC_CLK high to EXMC_NEx high | 7.34 | — | ns |
| td(CLKL-NADVL) | EXMC_CLK low to EXMC_NADV low | 0 | — | ns |
| td(CLKL-NADVH) | EXMC_CLK low to EXMC_NADV high | 0 | — | ns |
| td(CLKL-AV) | EXMC_CLK low to EXMC_Ax valid | 0 | — | ns |
| td(CLKH-AIV) | EXMC_CLK high to EXMC_Ax invalid | 7.34 | — | ns |
| td(CLKL-NOEL) | EXMC_CLK low to EXMC_NOE low | 0 | — | ns |
| td(CLKH-NOEH) | EXMC_CLK high to EXMC_NOE high | 7.34 | — | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	Based on configure: fHCLK  = 240 MHz, BurstAccessMode = Enable; MemoryType = PSRAM; WriteBurst =

Enable; CLKDivision = 3 (EXMC_CLK is 4 divided by HCLK); DataLatency = 1.

Table 4-52. Synchronous non-multiplexed PSRAM write timings(1)(2)(3)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| tw(CLK) | EXMC_CLK period | 16.67 | — | ns |
| td(CLKL-NExL) | EXMC_CLK low to EXMC_NEx low | 0 | — | ns |
| td(CLKH-NExH) | EXMC_CLK high to EXMC_NEx high | 7.34 | — | ns |
| td(CLKL-NADVL) | EXMC_CLK low to EXMC_NADV low | 0 | — | ns |
| td(CLKL-NADVH) | EXMC_CLK low to EXMC_NADV high | 0 | — | ns |
| td(CLKL-AV) | EXMC_CLK low to EXMC_Ax valid | 0 | — | ns |
| td(CLKH-AIV) | EXMC_CLK high to EXMC_Ax invalid | 7.34 | — | ns |
| td(CLKL-NWEL) | EXMC_CLK low to EXMC_NWE low | 0 | — | ns |
| td(CLKH-NWEH) | EXMC_CLK high to EXMC_NWE high | 7.34 | — | ns |
| td(CLKL-DATA) | EXMC_A/D valid data after EXMC_CLK low | 0 | — | ns |
| th(CLKL-NBLH) | EXMC_CLK low to EXMC_NBL high | 0 | — | ns |

(1)	CL = 30 pF.

(2)	Guaranteed by design, not tested in production.

(3)	Based on configure: fHCLK  = 240 MHz, BurstAccessMode = Enable; MemoryType = PSRAM; WriteBurst =

Enable; CLKDivision = 3 (EXMC_CLK is 4 divided by HCLK); DataLatency = 1.

108

GD32F470xx Datasheet

4.26.

TIMER characteristics

Table 4-53. TIMER characteristics(1)

| Symbol | Parameter | Conditions | Min | Max | Unit |
| --- | --- | --- | --- | --- | --- |
| tres | Timer resolution time | — | 1 | — | tTIMERxCLK |
|  |  | fTIMERxCLK = 240 MHz | 4.17 | — | ns |
| fEXT | Timer external clock frequency | — | 0 | fTIMERxCLK/2 | MHz |
|  |  | fTIMERxCLK = 240 MHz | 0 | 120 | MHz |
| RES | Timer resolution | TIMERx (except TIMER1 & TIMER4) | — | 16 | bit |
|  |  | TIMER1 & TIMER4 | — | 32 | bit |
| tCOUNTER | 16-bit counter clock period when internal clock is selected | — | 1 | 65536 | tTIMERxCLK |
|  |  | fTIMERxCLK = 240 MHz | 0.004 | 273.07 | μs |
| tMAX_COUNT | Maximum possible count | — | — | 65536x65536 | tTIMERxCLK |
|  |  | fTIMERxCLK = 240 MHz | — | 17.90 | s |

(1)	Guaranteed by design, not tested in production.

4.27.

DCI characteristics

Table 4-54. DCI characteristics(1)

| Symbol | Parameter | Min | Max | Unit |
| --- | --- | --- | --- | --- |
| Frequency ratio | DCI_PIXCLK /fHCLK | — | 0.4 |  |
| DCI_PIXCLK | Pixel clock input | — | 96 | MHz |
| DPixel | Pixel clock input duty cycle | 30 | 70 | % |
| tsu(DATA) | Data input setup time | 2.5 | — | ns |
| th(DATA) | Data input hold time | 1 | — | ns |
| tsu(HSYNC) | DCI_HS input setup time | 2 | — | ns |
| tsu(VSYNC) | DCI_VS input setup time | 2 | — | ns |
| th(HSYNC) | DCI_HS input hold time | 0.5 | — | ns |
| th(VSYNC) | DCI_VS input hold time | 0.5 | — | ns |

(1)	Guaranteed by design, not tested in production.

109

GD32F470xx Datasheet

4.28.

WDGT characteristics

Table 4-55. FWDGT min/max timeout period at 32 kHz (IRC32K)(1)

| Prescaler divider | PSC[2:0] bits | Min timeout RLD[11:0] = 0x000 | Max timeout RLD[11:0] = 0xFFF | Unit |
| --- | --- | --- | --- | --- |
| 1/4 | 000 | 0.03125 | 511.90625 | ms |
| 1/8 | 001 | 0.03125 | 1023.7812 |  |
| 1/16 | 010 | 0.03125 | 2047.53125 |  |
| 1/32 | 011 | 0.03125 | 4095.03125 |  |
| 1/64 | 100 | 0.03125 | 8190.03125 |  |
| 1/128 | 101 | 0.03125 | 16380.03125 |  |
| 1/256 | 110 or 111 | 0.03125 | 32760.03125 |  |

(1)	Guaranteed by design, not tested in production.

Table 4-56. WWDGT min-max timeout value at 60 MHz (fPCLK1)(1)

| Prescaler divider | PSC[1:0] | Min timeout value CNT[6:0] = 0x40 | Unit | Max timeout value CNT[6:0] = 0x7F | Unit |
| --- | --- | --- | --- | --- | --- |
| 1/1 | 00 | 68.27 | μs | 4.37 | ms |
| 1/2 | 01 | 136.53 |  | 8.74 |  |
| 1/4 | 10 | 273.07 |  | 17.48 |  |
| 1/8 | 11 | 546.13 |  | 34.95 |  |

(1)	Guaranteed by design, not tested in production.

4.29.

Parameter conditions

Unless otherwise specified, all values given for VDD = VDDA = 3.3 V, TA = 25 ℃.

110

GD32F470xx Datasheet

5.	Package information

5.1.

BGA176 package outline dimensions

Figure 5-1. BGA176 package outline

aaa  B

2X

E

B	A

E1

eee	C  A  B

fff	C

PIN 1 CORNER	e	b

1   2   3   4   5   6   7   8   9 10  11 12 13 14 15	15 14 13 12 11 10 9   8   7   6   5   4   3   2   1	PIN 1 CORNER

A B C D E F G

LASER MARK

PIN 1

A B C D E F G

e

H J K L M

D

H J K L M

D1

N P R

L

N P R

aaa  A	L

2X

TOP VIEW	BOTTOM VIEW

DETAIL A	A2	A3

ccc  C

SEATING PLANE

SEATING PLANE	A	ddd  C   A1	c	C

C

SIDE VIEW	DETAIL A(3:1)

Table 5-1. BGA176 package dimensions

| Symbol | Min | Typ | Max |
| --- | --- | --- | --- |
| A | — | — | 0.89 |
| A1 | 0.13 | 0.18 | 0.23 |
| A2 | 0.58 | 0.63 | 0.68 |
| A3 | — | 0.45 | — |
| b | 0.20 | 0.25 | 0.30 |
| c | 0.15 | 0.18 | 0.21 |
| D | 9.90 | 10.00 | 10.10 |
| D1 | — | 9.10 | — |
| E | 9.90 | 10.00 | 10.10 |
| E1 | — | 9.10 | — |
| e | — | 0.65 | — |
| L | — | 0.325 | — |
| aaa | — | 0.10 | — |
| ccc | — | 0.20 | — |
| ddd | — | 0.08 | — |
| eee | — | 0.15 | — |
| fff | — | 0.08 | — |

(Original dimensions are in millimeters)

111

GD32F470xx Datasheet

Figure 5-2. BGA176 recommended footprint

Dimension	Recommended values

Pitch	0.65 mm

Dpad	0.30 mm

Dsm	0.40 mm

Dpad

Dsm

(Original dimensions are in millimeters)

112

GD32F470xx Datasheet

5.2.

LQFP144 package outline dimensions

Figure 5-3. LQFP144 package outline

A3

c

F	A1	A2A	θ	0.08

D

D1

108	73

109	72	0.25

L

L1

DETAIL:   F

E1  E

b

b1

c1 c

144	37	BASE METAL

1

b

e

B B

36

WITH PLATING

SECTION B-B

Table 5-2. LQFP144 package dimensions

| Symbol | Min | Typ | Max |
| --- | --- | --- | --- |
| A | — | — | 1.60 |
| A1 | 0.05 | — | 0.15 |
| A2 | 1.35 | 1.40 | 1.45 |
| A3 | 0.59 | 0.64 | 0.69 |
| b | 0.18 | — | 0.26 |
| b1 | 0.17 | 0.20 | 0.23 |
| c | 0.13 | — | 0.17 |
| c1 | 0.12 | 0.13 | 0.14 |
| D | 21.80 | 22.00 | 22.20 |
| D1 | 19.90 | 20.00 | 20.10 |
| E | 21.80 | 22.00 | 22.20 |
| E1 | 19.90 | 20.00 | 20.10 |
| e | — | 0.50 | — |
| L | 0.45 | — | 0.75 |
| L1 | — | 1.00 | — |
| θ | 0° | — | 7° |

(Original dimensions are in millimeters)

113

144

109

37

72

22.70

17.80

0.30

GD32F470xx Datasheet

Figure 5-4. LQFP144 recommended footprint

22.70

20.30

1

36

108

73

1.20

0.50

(Original dimensions are in millimeters)

114

GD32F470xx Datasheet

5.3.

BGA100 package outline dimensions

Figure 5-5. BGA100 package outline

aaa  B

2X	E	B  A	E1	eee	C  A  B

PIN 1 CORNER

e

b

fff	C

PIN 1 CORNER

1   2   3   4   5   6   7   8   9  10 11 12	12  11 10  9   8   7   6   5   4   3   2   1

A

B

LASER MARK

PIN 1

A

B

C D E

C D E

e

F G H J

D

F G H J

D1

K L M

L

K L M

aaa  A	L

2X

TOP VIEW	BOTTOM VIEW

A2

DETAIL A

ccc  C	A3

SEATING PLANE	A	c

C	100X	ddd  C	A1	SEATING PLANE	C

SIDE VIEW	DETAIL A(3:1)

Table 5-3. BGA100 package dimensions

| Symbol | Min | Typ | Max |
| --- | --- | --- | --- |
| A | — | — | 0.84 |
| A1 | 0.13 | 0.18 | 0.23 |
| A2 | 0.53 | 0.58 | 0.63 |
| A3 | — | 0.40 | — |
| b | 0.20 | 0.25 | 0.30 |
| c | 0.15 | 0.18 | 0.21 |
| D | 6.90 | 7.00 | 7.10 |
| D1 | — | 5.50 | — |
| E | 6.90 | 7.00 | 7.10 |
| E1 | — | 5.50 | — |
| e | — | 0.50 | — |
| L | — | 0.625 | — |
| aaa | — | 0.10 | — |
| ccc | — | 0.20 | — |
| ddd | — | 0.08 | — |
| eee | — | 0.15 | — |
| fff | — | 0.08 | — |

(Original dimensions are in millimeters)

115

GD32F470xx Datasheet

Figure 5-6. BGA100 recommended footprint

Dimension	Recommended values

Pitch	0.50 mm

Dpad	0.25 mm

Dsm	0.35 mm

Dpad

Dsm

(Original dimensions are in millimeters)

116

GD32F470xx Datasheet

5.4.

LQFP100 package outline dimensions

Figure 5-7. LQFP100 package outline

A3	A2  A	θ	c

F	A1	0.08

eB

D

D1

75	51

0.25

76	50

L

L1

DETAIL:   F

E1	E

b

b1

100	26	c1 c

BASE METAL

b

1

e

B

B

25

WITH PLATING

SECTION B-B

Table 5-4. LQFP100 package dimensions

| Symbol | Min | Typ | Max |
| --- | --- | --- | --- |
| A | — | — | 1.60 |
| A1 | 0.05 | — | 0.15 |
| A2 | 1.35 | 1.40 | 1.45 |
| A3 | 0.59 | 0.64 | 0.69 |
| b | 0.18 | — | 0.26 |
| b1 | 0.17 | 0.20 | 0.23 |
| c | 0.13 | — | 0.17 |
| c1 | 0.12 | 0.13 | 0.14 |
| D | 15.80 | 16.00 | 16.20 |
| D1 | 13.90 | 14.00 | 14.10 |
| E | 15.80 | 16.00 | 16.20 |
| E1 | 13.90 | 14.00 | 14.10 |
| e | — | 0.50 | — |
| eB | 15.05 | — | 15.35 |
| L | 0.45 | — | 0.75 |
| L1 | — | 1.00 | — |
| θ | 0° | — | 7° |

(Original dimensions are in millimeters)

117

100

76

26

50

16.70

12.30

0.30

GD32F470xx Datasheet

Figure 5-8. LQFP100 recommended footprint

16.70

14.30

1

25

75

51

1.20

0.50

(Original dimensions are in millimeters)

118

GD32F470xx Datasheet

5.5.

Thermal characteristics

Thermal resistance is used to characterize the thermal performance of the package device, which is represented by the Greek letter “θ”. For semiconductor devices, thermal resistance represents the steady-state temperature rise of the chip junction due to the heat dissipated on the chip surface.

θJA: Thermal resistance, junction-to-ambient. θJB: Thermal resistance, junction-to-board. θJC: Thermal resistance, junction-to-case.

ᴪJB: Thermal characterization parameter, junction-to-board.

ᴪJT: Thermal characterization parameter, junction-to-top center.

θJA=(TJ-TA)/PD	(5-1)

θJB=(TJ-TB)/PD	(5-2)

θJC=(TJ-TC)/PD	(5-3)

Where, TJ = Junction temperature. TA = Ambient temperature

TB = Board temperature

TC = Case temperature which is monitoring on package surface PD = Total power dissipation

θJA represents the resistance of the heat flows from the heating junction to ambient air. It is an indicator of package heat dissipation capability. Lower θJA  can be considerate as better overall thermal performance. θJA is generally used to estimate junction temperature.

θJB is used to measure the heat flow resistance between the chip surface and the PCB board.

θJC represents the thermal resistance between the chip surface and the package top case. θJC is mainly used to estimate the heat dissipation of the system (using heat sink or other heat dissipation methods outside the device package).

Table 5-5. Package thermal characteristics(1)

| Symbol | Condition | Package | Value | Unit |
| --- | --- | --- | --- | --- |
| θJA | Natural convection, 2S2P PCB | BGA176 | 45.02 | °C/W |
|  |  | LQFP144 | 48.76 |  |
|  |  | BGA100 | 78.32 |  |
|  |  | LQFP100 | 57.42 |  |
| θJB | Cold plate, 2S2P PCB | BGA176 | 26.55 | °C/W |
|  |  | LQFP144 | 35.00 |  |
|  |  | BGA100 | 55.27 |  |

119

GD32F470xx Datasheet

|  | Symbol | Condition | Package | Value | Unit |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | LQFP100 | 31.68 |  |  |
|  | θJC | Cold plate, 2S2P PCB | BGA176 | 9.93 | °C/W |  |
|  |  |  | LQFP144 | 12.03 |  |  |
|  |  |  | BGA100 | 20.15 |  |  |
|  |  |  | LQFP100 | 13.85 |  |  |
|  | ᴪJB | Natural convection, 2S2P PCB | BGA176 | 28.31 | °C/W |  |
|  |  |  | LQFP144 | 35.32 |  |  |
|  |  |  | BGA100 | 55.74 |  |  |
|  |  |  | LQFP100 | 41.28 |  |  |
|  | ᴪJT | Natural convection, 2S2P PCB | BGA176 | 0.69 | °C/W |  |
|  |  |  | LQFP144 | 1.86 |  |  |
|  |  |  | BGA100 | 1.74 |  |  |
|  |  |  | LQFP100 | 0.75 |  |  |

(1)	Thermal characteristics are based on simulation, and meet JEDEC specification.

120

GD32F470xx Datasheet

6.

Ordering information

Table 6-1. Part ordering code for GD32F470xx devices

| Ordering code | Flash (KB) | Package | Package type | Temperature operating range |
| --- | --- | --- | --- | --- |
| GD32F470IKH6 | 3072 | BGA176 | Green | Industrial -40°C to +85°C |
| GD32F470IIH6 | 2048 | BGA176 | Green | Industrial -40°C to +85°C |
| GD32F470IGH6 | 1024 | BGA176 | Green | Industrial -40°C to +85°C |
| GD32F470ZKT6 | 3072 | LQFP144 | Green | Industrial -40°C to +85°C |
| GD32F470ZIT6 | 2048 | LQFP144 | Green | Industrial -40°C to +85°C |
| GD32F470ZGT6 | 1024 | LQFP144 | Green | Industrial -40°C to +85°C |
| GD32F470ZGT7 | 1024 | LQFP144 | Green | Industrial -40°C to +105°C |
| GD32F470ZET6 | 512 | LQFP144 | Green | Industrial -40°C to +85°C |
| GD32F470VKH6 | 3072 | BGA100 | Green | Industrial -40°C to +85°C |
| GD32F470VIH6 | 2048 | BGA100 | Green | Industrial -40°C to +85°C |
| GD32F470VGH6 | 1024 | BGA100 | Green | Industrial -40°C to +85°C |
| GD32F470VGH7 | 1024 | BGA100 | Green | Industrial -40°C to +105°C |
| GD32F470VKT6 | 3072 | LQFP100 | Green | Industrial -40°C to +85°C |
| GD32F470VIT6 | 2048 | LQFP100 | Green | Industrial -40°C to +85°C |
| GD32F470VIT7 | 2048 | LQFP100 | Green | Industrial -40°C to +105°C |
| GD32F470VGT6 | 1024 | LQFP100 | Green | Industrial -40°C to +85°C |
| GD32F470VGT7 | 1024 | LQFP100 | Green | Industrial -40°C to +105°C |
| GD32F470VET6 | 512 | LQFP100 | Green | Industrial -40°C to +85°C |

121

GD32F470xx Datasheet

7.

Revision history

Table 7-1. Revision history

| Revision No. | Description |  |  |  |  |  | Date |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 1.0 | Initial Release |  |  |  |  |  | Feb.22, 2022 |
| 1.1 | 1. Add circuit 2. Add 3. Modify Package 4. Mofidy 5. Modify 6. Modify memory 7. Modify 4-11. | Figure 4-1. Recommended PDR_ON pin |  |  |  |  | May.24, 2022 |
|  |  | in chapter characteristics. description of characteristics. the BGA100 information the I2C characteristics. fsck maximum characteristics(1). the erase characteristics the value of ESD characteristics |  | 4.5. Power supply supervisor |  |  |  |
|  |  |  |  | EMI in chapter 4.4. EMC footprint parameters in . timing parameters in value in Table 4-37. cycles in Table 4-24. Flash . VESD(HBM) and VESD(CDM) (1). | chapter 5. 4.16. I2C |  |  |
|  |  |  |  |  | USART in Table |  |  |
| 1.2 | 1. Update 4-24. 2. Update 3. Update pin circuit 4. Update |  | Table 4-37 USART characteristics(1), Flash memory characteristics. Table 4-11. ESD characteristics(1). Figure 4-2. Recommended external |  |  | Table | Jul.12, 2022 |
|  |  |  |  |  |  | NRST |  |
|  |  |  | (1). Table 4-35. Standard SPI characteristics(1). |  |  |  |  |
| 1.3 | 1. Add GD32F470xxT7 related descriptions. |  |  |  |  |  | Aug.22, 2022 |
| 1.4 | 1. Add notes for Table 4-2. DC operating conditions and Table 4-7. Power consumption characteristics(2)(3)(4)(5)(6), and update Table 4-7. Power consumption characteristics(2)(3)(4)(5)(6). 2. Update Table 4-25. Flash memory characteristics(1). 3. Add description of EMI and Table 4-10. EMI characteristics(1). 4. Update Figure 4-7. I2C bus timing diagram. 5. Update Figure 4-26. I/O port DC characteristics(1)(3). |  |  |  |  |  | Jan.4, 2023 |
| 1.5 | 1. Add GD32F470VGH7. |  |  |  |  |  | Feb.21, 2023 |
| 1.6 | 1. Add the Table 4-44. Dynamics characteristics: Ethernet MAC signals for SMI(1) and Figure 4-12. |  |  |  |  |  | Mar.14, 2023 |

122

GD32F470xx Datasheet

|  | Revision No. | Description |  |  |  |  |  |  | Date |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Ethernet SMI timing diagram. |  |  |  |  |  |  |  |
|  | 1.7 | 1. Update Table definitions, definitions, definitions, definitions supply |  | 2-3. GD32F470Ix BGA176 pin Table 2-4. GD32F470Zx LQFP144 pin |  |  |  |  | Jul.8, 2023 |
|  |  |  |  | Table 2-5. GD32F470Vx BGA100 pin Table 2-6. GD32F470Vx LQFP100 pin |  |  |  |  |  |
|  |  |  |  | decoupling | and Figure 4-1. Recommended power |  |  |  |  |
|  |  |  |  |  | capacitors(1)(2). |  |  |  |  |
|  | 1.8 | 1. Modify the PECYC to 100 kcycles in Table 4-24. Flash memory characteristics(1). 2. Modify description note(3) of Table 4-26. I/O port DC characteristics(1)(3). 3. Add device GD32F470VIT7. |  |  |  |  |  |  | Dec.27, 2023 |
|  | 1.9 | 1. Delete EXMC pin in GD32F470Vx. |  |  |  |  |  |  | Apr.18, 2024 |
|  | 2.0 | 1. Modify Table 4-27. I/O port DC characteristics 2. Modify Figure 5-1. LQFP144 package outline Figure 5-5. LQFP100 package outline and 5-7. LQFP64 package outline. 3. Modify Table 4-5. Start-up timings of Operating conditions(1)(2)(3). |  |  |  |  |  | (1)(3). , Figure | Jul.15, 2024 |
|  | 2.1 | 1. 2. 3. 4. 5. 6. | Update the note on VREFP Table 4-28. ADC characteristic Modify DAC to DAC0 in chapter overview. Update the parameters fPLLOUT PLL characteristic, Table 4- characteristic and Table 4-22 characteristic. Update Figure 4-4. Recommended circuit and note. Update description in chapter interface and synchronous/asynchronous |  |  | power up . 2. Device and fVCO in 21. PLLI2S . PLLSAl 3.13. Serial | timing in - Table 4-20. |  | Jan. 23, 2025 |
|  |  |  |  |  |  |  | PDR_ON pin peripheral |  |  |
|  |  |  |  |  |  | 3.14 Universal |  |  |  |
|  |  |  |  |  |  | receiver transmitter |  |  |  |
|  |  |  | (USART/UART). Update the descripton of th(DATA) in Table 4-54. DCI characteristics(1). |  |  |  |  |  |  |

123

GD32F470xx Datasheet

Important Notice

This document is the property of GigaDevice Semiconductor Inc. and its subsidiaries (the "Company"). This document, including any product of the Company described in this document (the “Product”), is owned by the Company under the intellectual property laws and treaties of the People’s Republic of China and other jurisdictions worldwide. The Company reserves all rights under such laws and treaties and does not grant any license under its patents, copyrights, trademarks, or other intellectual property rights. The names and brands of third party referred thereto (if any) are the property of their respective owner and referred to for identification purposes only.

The Company makes no warranty of any kind, express or implied, with regard to this document or any Product, including, but not limited to, the implied warranties of merchantability and fitness for a particular purpose. The Company does not assume any liability arising out of the application or use of any Product described in this document. Any information provided in this document is provided only for reference purposes. It is the responsibility of the user of this document to properly design, program, and test the functionality and safety of any application made of this information and any resulting product. Except for customized products which has been expressly identified in the applicable agreement, the Products are designed, developed, and/or manufactured for ordinary business, industrial, personal, and/or household applications only. The Products are not designed, intended, or authorized for use as components in systems designed or intended for the operation of weapons, weapons systems, nuclear installations, atomic energy control instruments, combustion control instruments, airplane or spaceship instruments, transportation instruments, traffic signal instruments, life-support devices or systems, other medical devices or systems (including resuscitation equipment and surgical implants), pollution control or hazardous substances management, or other uses where the failure of the device or Product could cause personal injury, death, property or environmental damage ("Unintended Uses"). Customers shall take any and all actions to ensure using and selling the Products in accordance with the applicable laws and regulations. The Company is not liable, in whole or in part, and customers shall and hereby do release the Company as well as it’s suppliers and/or distributors from any claim, damage, or other liability arising from or related to all Unintended Uses of the Products. Customers shall indemnify and hold the Company as well as it’s suppliers and/or distributors harmless from and against all claims, costs, damages, and other liabilities, including claims for personal injury or death, arising from or related to any Unintended Uses of the Products.

Information in this document is provided solely in connection with the Products. The Company reserves the right to make changes, corrections, modifications or improvements to this document and Products and services described herein at any time, without notice.

© 2025 GigaDevice – All rights reserved

124